3GPP TSG-WG SA2 Meeting #170	S2-2507168

Goteborg, Sweden, Aug 25th – 29th, 2025

Source:	NTT DOCOMO, MediaTek Inc., Deutsche Telekom

Title:	Discussion on UE as a sensing service consumer

Document for:	Discussion

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Discussion on UE as a sensing service consumer.

# 1. Introduction

TR 23.700‑14 defines in its architectural requirements that “The Sensing Service needs to support exposure of the sensing results to a Sensing Service consumer”. Furthermore, In Key Issue #5 on “Sensing Result Exposure”, the same term “Sensing Service Consumer” is used without further qualification.

During the study, some companies have interpreted Sensing Service Consumer to include the UE. This discussion paper seeks to clarify that, for the current Rel‑20 5G-A study, Sensing Service Consumer should refer to a trusted Application Function (AF) interfacing via the NEF, and not to the UE.

The rationale is based on the interpretation of SA1 requirements, operational and security considerations, and lessons learned from previous work items in 5GS. The contribution also proposes options for the way forward, including sending LSs to SA1 and SA3 for clarification and alignment if necessary.

# 2. Discussion

Observation 1 – Alignment with SA1 Requirements: In the SA1 requirements for sensing in 5G (TS 22.137), it is stated that sensing results are to be reported to a “trusted third‑party”. TS 22.137, sub-clause 5.2.3 for Network exposure mentions the requirements as follows:

“Subject to operator’s policy, the 5G network shall be able to provide secure means to report sensing result to a trusted third-party requesting information about a target object when specific requested conditions are met.”

In the architecture context, this trusted third‑party is typically realized as an AF under a contractual and operational agreement with the operator. UEs, in contrast, are not explicitly identified by SA1 as recipients of sensing results.

Observation 2 – Trust and Control Model for UEs: Historically, UEs are not fully trusted entities and operate only partially under the control of the MNO. In contrast, AFs are bound by SLAs with the operator, providing clear accountability in case of misuse or security breaches. Exposing network-derived sensing results directly to UEs introduces uncertainties regarding the purpose and handling of this information, increasing the risk of uncontrolled dissemination or misuse.

Observation 3 – Security and Privacy Risks: Allowing UEs to receive sensing results raises unresolved questions about how to enforce data protection, prevent exposure to end‑users, secure against malicious exploitation, and other user-consent and privacy related concerns. Without clear requirements and enforcement mechanisms, the exposure of such data to UEs poses significant risks to network security and user privacy.

Observation 4 – Precedent from previous Releases: In the past releases, for example, in Rel-18, under work item Ranging_SL, clauses enabling UE-initiated ranging/SL positioning service exposure through UP/CP were ultimately removed due to lack of agreement in SA2 and SA3 on handling UE privacy checks (TS 23.586, void clauses in clause 5.6). Proceeding with UE as a Sensing Service Consumer in Rel-20 without resolving these privacy concerns is likely to encounter the same inter-WG deadlock. Furthermore, in Rel-18 under study item FS_AIMLsys, for Key Issue #2 on “5GC information exposure to UE”, same issue was noticed (S2-2203768). This suggests a proactive approach to avoid repeating past issues.

NOTE: 	Above observations are applicable in the context of Rel-20 5G-A study on sensing based on existing requirements for sensing in 5G as of Rel-20 but does not preclude the possibility to study such exposure mechanisms in the context of 6G subject to scoping and the outcome of SA1 study on 6G.

# 3. Conclusion and proposal(s)

Based on the above observations, we have the following proposals:

Proposal 1 (primary proposal): For Rel-20 5G-A ISAC study, define Sensing Service Consumer exclusively as a trusted AF interfacing via the NEF and remove any consideration of UE as a direct sensing service consumer from the current study scope in this release.

Proposal 2 (alternative proposal):

- If consensus cannot be reached on the primary proposal, send a Liaison Statement to SA1 requesting clarification on existing 5G requirements whether “trusted third‑party” be considered to include UE as well.

- Also, send a Liaison Statement to SA3 highlighting the security and privacy implications of exposing sensing results to UEs, requesting their input on whether adequate safeguards can be defined for such exposure in Rel-20 in 5GS.