SA WG2 Meeting #170	S2-2507179

25 – 29 Aug, 2025, Goteborg, SE

Source:	NEC

Title:	[WT#7], Key Issue on NTN in 6G – Study proposal for Service Continuity aspects

Document for:	Discussion / Approval

Agenda Item:	20.6.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the study considerations relating to Service Continuity aspects of NTN in 6G under WT7, as defined in S2-2506096.

1. Justifications

S2-2506096 indicates 7. Study on how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 8:  The detailed scope for WT#7 will be coordinated and aligned with RAN

In relation to the “Service Continuity aspects of NTN”, this paper discusses the following inputs:

Architectural Motivation: The satellite access technology has been specified in 4G/5G enabling a UE to receive communication services from satellites that are deployed at different altitudes by the ways of transparent or regenerative payload. And normal communication mode and store and forward communication mode has been supported to provide data transmission in connected and intermittent network environment respectively. But for the LEO satellite which can provide lower time delay and higher bandwidth, it only provides the network coverage in a certain area for approximate several minutes due to its faster moving speed. It is unacceptable for some application in the short communication availability time period.

To improve the service duration time for NTN, it is reasonable to deploy multiple satellites in the same orbit. The satellite in the same orbit can be interconnected with each other and constitutes a “constellation”. Generally speaking, the “constellation” is composed of the homogeneous satellites i.e. these satellites share the same communication capability and feature.

Gap Analysis: A “constellation” can provide a broader network coverage and more persistent communication service than single satellite including both the connected and intermittent communication service modes. But it is unknown how to define the functions of each satellite, UE and ground network and support the service continuity for a UE in a “constellation”. In a “constellation”, both UE and satellite are consistently moving. The handover of UE among satellites may trigger the signalling message between UE and the satellites, source and target satellite, satellite and ground network. Moreover, there are multiple ground network, the handover procedure of the satellite in a “constellation” between two ground networks also should be taken into account.

Technical Impact: How would this WT/KI influence:

- Improvements to ensure efficient mobility and service continuity for UEs in a “constellation”, which includes:

- the handover between satellites in a “constellation” for a UE including the signalling interaction between UE and the satellites, source and target satellite, satellite and ground network;

- the handover between ground network for a satellite in a “constellation” including the signalling interaction between UE and satellite, satellite and ground network.

**** First Change ****

Annex A.X. WT#X Scope

WT 7.X - Support of Service Continuity for NTN

Study on how 6G networks shall support service continuity when a UE moves among satellites in a “constellation” including

- the handover between satellites in a “constellation” for a UE including the signalling interaction between UE and the satellites, source and target satellite, satellite and ground network;

- the handover between ground network for a satellite in a “constellation” including the signalling interaction between UE and satellite, satellite and ground network.

NOTE x:  The “constellation” indicates a group of satellites with homogenous communication capability and interconnected with each other via inter-satellite link.

**** Second Change ****

5.X. Key Issue #X (7.X): Support of Service Continuity for NTN

This key issue (KI) will study to cover the following aspects:

Study on how 6G networks shall support service continuity when a UE moves among satellites in a “constellation” including

- the handover between satellites in a “constellation” for a UE including the signalling interaction between UE and the satellites, source and target satellite, satellite and ground network;

- the handover between ground network for a satellite in a “constellation” including the signalling interaction between UE and satellite, satellite and ground network.

**** End of Changes ****