SA WG2 Meeting #170	S2-2507184

25 – 29 Aug, 2025, Goteborg, SE

Source:	CATT

Title:	[WT#6, Computing] Support computing in 6G

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the justifications and potential Key Issues for WT#6.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

1.1 Architectural Motivation

1.1.1 Motivation on coordination between communication and computing

Currently, many new services such as immersive communication and AI have put forward higher requirements for both communication and computing. Unilateral performance optimization of either the communication or computing can no longer meet the application demands. Especially for computing intensive services, it is usually necessary to evaluate the E2E delay, which includes communication delay, computing delay (e.g. XR services need to perform rendering and processing), etc. However, in current design of 5G system, only the communication delay is considered, so the E2E delay cannot be guaranteed. In addition, the application data processing is commonly done on the device to ensure low latency; however, due to the limited computing capacity on UE, the extremely low latency requirements can’t be meet, and resulting in bad user experience. In this case, the data processing could be done on network side. Many use cases and requirements of supporting 6G computing have been studied in SA1 per TR 22.870.

The Figure 1 illustrates a scenario only the communication delay is considered for AS selection, the optimal AS (i.e. minimum RTT) can’t be selected, and the resource allocation may be not appropriated due to separated control of communication resources and computing resources.

Figure 1. Separated control of communication and computing (communication delay based AS selection)

The computing resource refers to hardware and software that provides the required processing power, storage capability etc. to perform computational function. Based on the understanding of computing resource, it can be various forms, e.g. GPU, memory, OS, application etc. How to integrate various forms of computing resource needs investigation.

For 6G, it is required to research on the 6G network architecture that supports computing, and coordinates the communication resources and computing resources.

1.1.2 Motivation on utilizing computing resource in 6GC

Many uses cases, requirements for utilizing computing resource in Core Network have been proposed in SA1. Also the 6G Computing Service is defined per TR 22.870, as following:

6G Computing Service: A service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party.

The definition of “Service Hosting Environment” included in TS 22.261 can be reused, as following:

Service Hosting Environment: the environment, located inside of 5G network and fully controlled by the operator, where Hosted Services are offered from.

Based on above definitions, the computing resources should be located in 6G network and fully controlled by the operator, which can be utilized to provide the computing service.

The Figure 2 illustrates Core Network can joint control of communication resource and computing resource, then an optimal computing node is selected based on both network and computing metrics, e.g. communication delay, computing delay, form of computing resource.

Figure 2. Joint control of communication and computing in CN

Some advantages of utilizing computing resource in 6GC can be foreseen:

- Improve the utilization of computing resource and bring new revenue: Currently, the computing resources of 5G networks have not been effectively utilized (e.g. in service idle state), so it is expected to utilize the idle computing resource in the core network, which can improve the utilization rate of computing resources, and enable exposure of computing service (e.g. offload computing tasks from UE to network) to bring new revenue to MNO.

- Joint control of communication and computing: The core network can monitor the real-time network status (e.g. UP latency), and if computing resources is deployed in the 6G network, the core network can also monitor the status of computing resources, thereby achieving joint control of communication resources and computing resources, reasonable resource allocation and flexible scheduling of resources.

- Delay controllability: From the perspective of delay controllability, when support of utilizing computing resource in 6GC, the computing delay does not need to rely on the third party and will be controllable. The data can be processed within the 6G network and does not need to be routed outside (via N6), and then communication delay can also be controllable. So the E2E delay (including computing delay and communication delay) can be guaranteed.

- Data security and data privacy: From the perspective of data security and data privacy, the raw data can be computed within the network instead of exposed outside. Based on the security mechanism provided by the operator, the E2E protection of user data security and privacy can be achieved.

- Support of 6G native service: The 6G native service (e.g. 6G native AI, ISAC) can be supported by invoking NF service and performing resource sharing between NFs (i.e. without going through the N6 and NEF). It is more efficient and ensures security and privacy.

1.2 Gap Analysis

To support computing in 6G, comparing to existing architecture (Rel-15-Rel-20), there are some gaps requiring be resolved, including the following aspects:

- Computing capability is not aware and utilized in network. Due to lack of the information of computing resources, 5G network can’t support coordination of communication resources and computing resources to meet the application’s requirements (e.g. both communication requirement and computing requirement).

- Based on the design of 5G Edge Computing, some gaps are existing which are difficult to resolve:

- The EAS discovery is based on DNS mechanism and only the application server can be discovered, however, the computing resources can also be hardware and software other than application, so the discovery for other forms of computing resource can’t be supported.

- Only the communication delay is considered for EAS selection, while the computing delay is not considered. The DNS system is difficult to support the management of computing resource information dynamically, and it lacks the information of network status, so it can’t select an optimal EAS that ensures E2E delay.

- The UE is agnostic whether the service is provided by edge computing, and it is difficult to represent the computing requirements (e.g. time, location, required computing capabilities, computing task) by sending DNS query with FQDN.

1.3 Technical Impact

- 6G network should support introducing computing resource in core network, e.g. A NF (new NF or enhance existing NF) serves as computing node to provide computing resource/computing capability.

- A new CP NF can be introduced to support coordination of communication and computing, e.g. control and scheduling of computing resources within the network.

- The interaction (e.g. new service operation, new parameter) with these new/enhanced NF should be designed.

- UE can be aware of computing service, including requesting computing service and receiving service results etc.

**** First Change ****

Annex A.X. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

**** Second Change ****

5.X. Key Issue #X: Architecture support for computing

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue will address the system architecture and function required to support integrated computing, and the following aspects to be studied:

-	Architecture and function to support computing, including whether needs to introduce any new NF(s) and/or enhance existing functions in the Core Network and their interactions.

-	Whether and how to define the metrics of computing resource (i.e. computing capability).

-	How to acquire and maintain the information of computing resource (located in core network), and monitor computing resource status.

-	How to coordinate with a UE to decide on computing offloading to network.

-	Which entity and how to parse computing requirements.

-	How to select/allocate computing resource (located in core network).

-	How to perform computing service authorization.

-	How to ensure proper policy control and QoS enforcement (e.g. ensure E2E latency).

-	How to support computing service continuity (e.g. computing node changes).

**** End of Changes ****