SA WG2 Meeting #S2-170	S2-2507190

25 - 29 August, 2025, Goteburg, Sweden

Title:	Draft LS on Average Window for Alternative QoS

Response to:	S2-2506109/ R3-252397

Release:	REL-18

Work Item:	XRM

Source:	SA2 (CATT)

To:	RAN3

Cc:	-

Contact person:	Chunshan Xiong (chunshan.xiong@cictmobile.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA2 thanks for the LS from the RAN3, SA2 has the following answer for the LS:

- Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

- Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

RAN3 could not reach consensus on introducing averaging window in the alternative QoS Profile, and respectfully requests SA2 to provide clarification on introducing averaging window in the alternative QoS Profile.

[SA2 Answer:] SA2 provides the following description on how to select and use the averaging window:

The NG-RAN only uses the averaging window configuration of the current activated QP to measure GFBR/MFBR and to determine whether the current QoS profile can be fulfilled and which alternative QoS profile can be used as a reference if the current QoS profile is not fulfilled. Only after a new QP is selected from the AQP list and configured to be used in the NG-RAN, the NG-RAN begins to use the new averaging window configuration of the new activated QP to measure GFBR/MFBR and to determine whether the new current QoS profile can be fulfilled and which alternative QoS profile can be used as a reference if the new current QoS profile is not fulfilled, and so on.

There are a lot of benefits to introducing averaging window with different values in different AQP. E.g., the different AQP can provide different GFBR and MFBR, and different GFBR and MFBR can be used by the different codecs in the AS media. e.g., for the same video stream, the high FPS (frames per Second)  (e.g., 120 FPS) normally needs more bitrate than the low FPS video (e.g., 30 FPS). For the low 30 FPS, a 2000ms default averaging window can provide 60 times of measurement of GFBR/MFBR in the NG-RAN, but for the high 120 FPS, the 60 times of measurement of GFBR/MFBR only need a 500ms  averaging window. In such cases, the different FPS videos can have different averaging windows with the same measurement times for the GFBR and MFMR in the NG-RAN.

The video stream with high FPS can change more quickly in codec, the short averaging window can provide QoS Notification message in shorter time than the low FPS with the same measure time of GFBR/MFBR to the AS. In such a short averaging window for high FPS can provide better adaptation to the bitrate changes in the NG-RAN.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to take the above information into account and update the related TS.

3. Dates of Next SA2 Meetings:

SA2#171		13th – 17th October 2025 		 Wuhan, CN

SA2#172		17th – 21st November 2025 		 Dallas, US