3GPP TSG-WG SA2 Meeting #170	S2-2507200

25th – 29th August, 2025, Gothenburg, Sweden	(revision of S2-250XXXX)

Source:	Lenovo

Title:	[WT#1.1, NAS] Key Issues on NAS and operator services in 6G

Document for:	Discussion and Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to split the WT#1.1 into two independent key issues – one for NAS framework and one for operator services over the user plane.

# 1. Justifications

## 1.1 Architecture motivation and gap analysis

Today there are operator services offered via the user plane, e.g. IMS, and other services offered via the control plane, e.g. short message service (SMS), location/positioning services, UE configuration like Steering-of-Roaming (SoR) or UE parameter update (UPU). In 5G a number of new services emerged and yet new services will continue to be introduces in 6G, e.g. Sensing, data collection for AI/ML, etc.

The start of 6G offers the opportunity to analyse and design the principles of using operator services in a new way. It is beneficial to deploy operator services over the user plane, especially operator services that require exchange of large amounts of data, frequent signalling exchanges or services that require differentiated QoS handling. The transmission of data over the user plane is supported in more efficient way than the transmission over the control plane.

Thus, a unified framework to deploy an operator service over the user-plane in 6G would offer:

-	the opportunity to re-use the same framework in future generations;

-	deploy a new operator service without (or minimal) impact to the control plane;

-	deployment of a new service with minimal impact to the 3GPP specifications and thus, faster and easier deployment.

On the other side, the signalling exchange between the UE and the network for basic functionalities like registration procedure and data connectivity establishment and management (e.g. PDU Session management) is realized via the control plane. The NAS protocol used for the control plane signalling can be analysed and new design can be achieved to make the NAS protocol modular. One challenge with the NAS protocol in 5G is that all NAS messages are still routed via a single AMF. It is worth to study whether the NAS messages can be routed in decentralized way in 6G.

The introduction of new 6G services may required the increased data exchange between UE and CN. New way of communication may need to be explored.

## Technical impact

The following aspects may be considered impacted:

-	The UE and the network support independent UE to CN exchange via the control plane and via the user plane.

-	unified framework to allow operator services deployment via the user plane.

-	new design of the NAS protocol to allow:

-	improved routing of NAS message between the UE and the target control plane NF;

-	direct security procedure (e.g. authentication, authorisation) between the UE and the target control plane NF.

# 2. Proposal

It is proposed to include the following text in the TR 23.801-01.

**** First Change ****

# Annex A.X. WT#1.1 Scope

The technical scope for WT#1.1 include the following aspects:

1.1	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to design the non-access stratum (NAS) to enable the introduction of a new NAS functionality (over control plane) without impacting other NAS functionalities. This includes how to efficiently route NAS message between UE and core network NFs and how to enable direct security procedure (e.g. authentication, authorization) between the UE and the target NF.

a.1.	Whether and how to identify a minimal set of NAS functionalities that does not get impacted by additional NAS functionalities.

b.		How to develop a generic mechanism for UE-Core Network interaction to support operator services beyond connectivity, e.g. to support positioning, Sensing, AI/ML, data collection, etc.

**** Second Change ****

# 5.X.	Key Issue for WT1.1a: Modular NAS framework for connectivity services

This key issue proposes to study how to support non-access stratum (NAS) signalling for connectivity between UE and 6G control plane NFs.

The key issue will investigate the following aspects:

-	Whether and how to identify a minimal set of NAS functionalities that does not get impacted by additional NAS functionalities.

-	How to efficiently route NAS message between UE and control plane NFs;

-	how to enable direct security procedure (e.g. authentication, authorization) between the UE and the target core network NF.

**** Third Change ****

# 5.Y.	Key Issue for WT1.1b: Common framework to support operator services (i.e. beyond connectivity services) over user plane

This key issue proposes to study how to design a generic mechanism for UE to Core Network interaction to support operator services, i.e. beyond connectivity services, over the user plane.

The key issue will investigate the following aspects:

-	How to establish the signalling exchange for operator services (i.e. beyond connectivity services) between the UE and the target core network NFs.

-	How to route the data for operator services between the UE and the target core network NFs.

-	How to support target NF selection.

**** End of Changes ****