3GPP TSG SA WG2#170	S2-2507201

Goteborg, Sweden, August 25 – 29, 2025	(was S2-241xxxx)

Source:	Lenovo

Title:	Motivation and proposed solution for TEI20 on Distributed Inference for VFL

Document for:	Discussion

Agenda Item:	30.1

Work Item / Release:	Release 20

Abstract of the contribution:

# 1	Discussion

## 1.1	Status of VFL in Rel. 19 and motivation for TEI20 WID

In Release 19, due to time constraints and to ensure a feature for VFL in Release 19 it was assumed that for any VFL scenario the VFL Server and VFL clients must support both Model Training and Inference functionality (i.e. must support both MTLF and AnLF functionality).

Due to this assumption for any scenario where any VFL server receives a request for analytics and/or model training, if such VFL server does not have a model and VFL training is needed then the VFL server must start a new VFL training process which results in duplication of computing resources.

In order to support a more flexible design and better deployment options it is proposed to include in Release 20, VFL for the case where VFL server/clients support only training (i.e. is an NWDAF containing MTLF) or only inference (i.e. an NWDAF containing AnLF). Cases for AF (either as VFL server or VFL client) are out of scope.

For such scenarios changes are needed in VFL procedure described in clause 6.2H.2 of 3GPP TR 23.288 while ensuring backwards compatibility with Release 19 procedure. The changes compared to Release 19 procedure are:

-	At VFL training (applicable for NWDAF containing MTLF)

-	For the scenario where VFL server is an NWDAF containing AnLF that supports inference using VFL.

-	The VFL Server (NWDAF containing AnLF) requests an ML model from the VFL server supporting VFL training (NWDAF containing MTLF) and receives a VFL correlation ID, a local ML model used by the VFL server used for training (if applicable), a list of VFL client ID(s) and associated Feature ID(s) used during VFL training and contribution weights for each VFL participant.

-	If the VFL server (NWDAF containing AnLF) does not receive a model then the VFL server reverts back to the applicable procedures defined in Case A-D.

-	At VFL inference

-	VFL server for inference (NWDAF containing AnLF) discovers for each VFL client used during training the VFL clients for inference (NWDAF containing AnLF) that support the list of Feature ID(s) that each VFL client for training used during VFL training (no additional changes needed in the procedure)

-	VFL server for inference (NWDAF containing AnLF) includes in the VFL inference request to each VFL client for inference the VFL client ID used for training and associated Feature ID(s).

-	VFL clients for inference retrieve the local model from the VFL client for training using the VFL correlation ID and initiates inference using the local model.

-	VFL server (for inference) aggregates the inference output from each VFL client taking into account accuracy of model and contribution weights.

## 1.2	Solution

The following flow diagram describes the VFL inference procedure for the case where the VFL server/clients for inference are different from the VFL server/clients for training.

Figure 1.2-1: Proposal for support of distributed inference

- Based on consumer request the VFL server supporting only VFL inference (i.e. an NWDAF containing AnLF) request an ML model

2.	VFL server supporting only training using VFL (i.e. NWDAF containing MTLF) may start training procedure as per step 1-7 of Figure 6.2H.2.3.1-1.

3.	When training is complete the NWDAF containing MTLF supporting VFL server provides to the VFL server supporting inference: VFL correlation id, the local model of the VFL server used at training and associated Feature ID(s) (if applicable), a list of participating VFL clients during VFL training and their associated Feature ID(s) and  contribution weights.

The VFL server (AnLF) performs inference by first selecting the VFL clients that can provide intermediate inference output. For each VFL client participating during VFL training thee VFL server discovers VFL clients for inference from the NRF .

4.	The VFL server (AnLF) sends an Nnrf_NFDiscovery request including Analytic ID, VFL client indication, Feature ID(s), NF type, VFL interoperability ID.

5.	The NRF responds with a list of candidates

6.	The AnLF VFL Server selects VFL clients supporting the Features received in step 3.

The VFL server sends to each VFL client an inference request including the VFL correlation ID and to each VFL client for inference the VFL client ID and the Feature ID(s) used for training.

7. The VFL server (AnLF) sends an Nwdaf_VFLInference request including to each VFL client: The Analytics ID, VFL correlation ID, VFL client ID for training and list of Feature ID(s).

8.	Each VFL client (AnLF) requests a local model from the VFL client used for training including the VFL correlation ID, list of Feature ID(s), ML model interoperability indicator.

9. The VFL client (MTLF) sends the local model available

10.	The VFL server (AnLF) starts the VFL inference procedure as described in steps 2-7 Figure 6.2H.2.4.2-1.