3GPP TSG-SA WG2 Meeting #170	S2-2507203

Goteborg, SE, 25th August – 29th August, 2025

Source:	Samsung

Title:	Key issue for WT#3: Support of routing of SMS to EC based on UE location

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#3.

1. Discussion

The SID on Short Message Service to Emergency Response Centre (SP-250837) includes the following WTs:

-	WT-1: How to identify a Short Message to Emergency Response Center based on the use of emergency numbers. Address aspects when serving PLMN does not support this feature, but UE supports it.

-	WT-2: Whether and how to identify the type of emergency service (police, ambulance, fire brigade, etc.) using the same identification mechanism as for emergency calls.

-	WT-3: How to route a Short Message to Emergency Response Center (i.e. PSAP) serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location).

NOTE 1:	Depending on the type of emergency service, Short Messages can be routed to different emergency response centres.

NOTE 2:	This study assumes that Short Messages initiated by the Emergency Response Center in response to the Short Message to the Emergency Response Center are treated as regular Short Messages.

For EPS and 5GS, 3GPP access (including TN and NTN) is in scope. UE in limited service state will not be in scope of this study item.

The architectures for SMS over NAS (see TS 23.272, TS 23.401, TS 23.501, TS 23.502) and SMS over IP (see TS 23.204), and the architectural principles of Emergency Services (see TS 23.167) will be used as a basis.

SMS over IP can be applied for both 3GPP access and non-3GPP access (either over EPS or 5GS).

For emergency calls, the network may retrieve the UE’s location using the LRF, according to local regulations, as described in TS 23.167[x]. This information can be used by the E-CSCF to route the UE to a PSAP serving the UE’s location. A similar solution should be implemented for SMS to EC.

Additionally, the network should also be able to retrieve the UE’s location in the case of roaming, according to local regulations.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-65.

* * * Start of Change * * * *

## 5.x	Key Issue #X: Support of routing of SMS to EC based on UE location

### 5.x.1	Description

To support routing of SMS to EC based on UE location, it is proposed to study the following issue:

- How to retrieve the UE’s location.

- How to select a PSAP serving the UE’s location.

- How to route a Short Message to a PSAP serving the UE’s location.

NOTE :	Retrieveing a UE’s location is subject to local regulations. In roaming case, the regulations applicable are that of the UE’s location.

* * * End of Change * * * *