3GPP TSG-WG SA2 Meeting #170	S2-2507204

25th – 29th August, 2025, Gothenburg, Sweden	(revision of S2-250XXXX)

Source:	Lenovo

Title:	[WT#1.2, Slicing] Proposal to study Network Slicing in 6G

Document for:	Discussions and Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to study Network Slicing for 6G.

# 1. Justifications

## 1.1 Architectural Motivation

Network Slicing is one of the features introduced in 5GS. The basic Network Slicing specified in Rel-15 introduced requirements on the UE to support various network slicing configurations, e.g. Configured NSSAI; Allowed NSSAI, Rejected NSSAI, and also to support the URSP rules to allow the routing of traffic on network slices. Further impacts to the UE were introduced by specifying the support of Target NSSAI, Pending NSSAI, Partially Allowed NSSAI, partially Rejected NSSAI, alternative S-NSSAI, etc.  The implementation of the UE, and the associated test cases, becomes complex. It is worth to study whether a re-design of the Network Slicing can achieve less UE configuration requirements which may increase the deployment for real business use cases.

Furthermore, the basic Network Slicing specified in Rel-15 was enhanced with many optional features during the evolution of the 5GS. Those enhancements originated in other SDOs (e.g. GSMA) or from individual companies or network operators. It is beneficial to evaluate the market needs and assess whether some of the Network Slicing features may be obsolete in 6G and instead new market-driven features need to be introduced in order to ease the monetization and applicability of the Network Slicing deployment.

## 1.2 Gap Analysis

Taking the URSP concept of the 5GS as an example, the implementations in the UE and the testing in the network appeared to be problematic. The original URSP concept specified in Rel-15 was continuously studied in Rel-17 and 18 in order to investigate whether:

- the UE enforces URSP rule;

- actions can be performed by the 5G network if the UE does not follow the URSP rules,

- to provision the UE with consistent URSP across 5GC and EPC,

- to support standardized and operator-specific traffic categories in the traffic descriptor of URSP.

It may be observed that configuring the UE with policies, maintaining and updating those policies, leads to increased complexity. It may be more flexible to configure the UE with basic (or simple) traffic routing policy and perform dynamically configuration to the UE on demand.

Furthermore, the Rel-15 Network Slicing concept relies on the Subscribed Network Slices for each UE which is a static list of S-NSSAI. Later in Rel-18 and Rel-19 it was identified that UE’s traffic can use alternative Network Slices (e.g. subscribed or not subscribed network slices). Instead of specifying Subscribed Network Slices, it is more flexible to have subscribed services for the UE and the MNO can determine (e.g. depending on current UE location or network load) which service is mapped on which Network Slice. This would avoid to perform Network Slice replacement in 6G.

In summary, it is proposed to re-evaluate the 5GS Network Slicing concept with the goal to design more flexible mechanism how “allowed Network Slices” are assigned to the UE and how application traffic is mapped to a Network Slice.

## Technical impact

The following aspects may be impacted:

- The UE configuration to support Network Slicing may be changed (e.g. avoid Configured NSSAI and Default Configured NSSAI) to allowed to be used in the current area without the.

- How to configure the UE with basic traffic routing policy and perform dynamically configuration to the UE on demand for specific application traffic (i.e. how to re-design the URSP concept and avoid Network Slice replacement).

- Identifying the Network Slicing features from 5GS which do not need to be supported in 6G (at least in the first release) and identifying new the Network Slicing to support flexible selection of connectivity (i.e. Network Slice) for a service traffic. Studying the interworking and migration between 5G and 6G for network slicing.

# 2. Proposal

It is proposed to include the following text in the TR 23.801-01.

**** First Change ****

# Annex A.X. WT#1.2 on Network Slicing

The technical scope for WT#6 should include:

-	Study the overall design of the 6G network slicing feature in order to improve:

-	how to minimize the network slice related configuration of the UE.

-	Study how to simplify the UE Route Selection Policy (URSP) for traffic routing to avoid i) dependency on UE implementation and ii) monitoring by the network.

-	Identifying the network slicing features from 5GS which do not need to be supported in 6G (at least in the first release) and identifying new the network slicing features to support flexible selection of connectivity (i.e. network slice and PDU Session) for a service traffic.

**** Second Change ****

# 5.X. Key Issue for WT#1.2 on Network Slicing

This key issue proposes to study how to design the network slicing feature in 6G with the goal to ease the network slicing deployment.

The key issue will investigate the following aspects:

-	Study the overall design of the 6G network slicing feature in order to improve:

-	How to minimize the network slice related configuration of the UE.

-	Study how to simplify the UE Route Selection Policy (URSP) for traffic routing to avoid i) dependency on UE implementation and ii) monitoring by the network.

-	Identifying the network slicing features from 5GS which do not need to be supported in 6G (at least in the first release) and identifying new the network slicing features to support flexible selection of connectivity (i.e. network slice and PDU Session) for a service traffic.

**** End of Changes ****