3GPP TSG-WG SA2 Meeting #170	S2-2507222

25th – 29th August, 2025, Gothenburg, Sweden	(revision of S2-250XXXX)

Source:	Lenovo

Title:	[WT#1.3, non3GPP access] WT and KI on support of non-3GPP access in 6G

Document for:	Discussion and Agreement

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to an updated WT and key issue on non-3GPP access.

# 1. Justifications

## 1.1 Architecture motivation and gap analysis

Non-3GPP technologies can provide backup connectivity in case of failures, can offload the 3GPP access, and can empower multiaccess connectivity. The 6G system should be able to integrate the wireless and wireline non-3GPP accesses so that the Core Network can continue to serve as access technology independent Core Network.

The 5GS was designed to integrate non-3GPP accesses and align the RAN-CN interfaces between 3GPP and non-3GPP accesses. The 5G non-3GPP access framework was considerable different from the integration of the non-3GPP accesses in the 4G/EPS. One of the main characteristics of non-3GPP access support in 5GS was that the UE registration (including authentication/authorisation) is performed via the NAS protocol and the same level of NAS security is provided for 3GPP access and non-3GPP access anchored at the AMF. Such design principle may be carried on for the 6G system.

However, not all enhancements specified in 5GS for the support on non-3GPP accesses (e.g. support of trusted access, support of ATSSS or support of MASS) have seen commercial market interest. In 6G, it should be evaluated which essential features should be supported in the first release.

It can be further studied whether a simplified integration of the non-3GPP access (e.g. WiFi) can be supported in 6G.

Depending on the outcome of WT#1.1 on the NAS design over the 3GPP access, it may be expected that impacts to the N3IWF (and possibly to TNGF or W-AGF) are possible. Therefore, the design of the UE to Core Network exchange in general, i.e. including connectivity service and services beyond connectivity, is expected to also impact the integration of the non-3GPP accesses and such impact should be reflected in the first 6G release.

## Technical impact

The following aspects may be considered impacted:

-	No major difference to the principles of using NAS to perform registration via non-3GPP access is expected.

-	The design of the UE to Core Network signalling exchange for WT#1.1 is expected to impact the integration of the non-3GPP accesses in the 6G system.

# 2. Proposal

It is proposed to include the following text in the TR 23.801-01.

**** First Change ****

# Annex A.X. WT#1.3 Scope

The technical scope for WT#1.3 includes the following aspects:

Study how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access including:

-	Study whether and how to enhance non-3GPP access in 6G System architecture based on the 5G principles.

-	Whether untrusted access type and wireline access only are considered in 6G.

-	Study whether and how the design of the UE to Core Network signalling exchange for WT#1.1 is expected to impact the integration of the non-3GPP accesses in the 6G system.

-	Study whether and how to support the multi-access data connectivity (e.g. MA PDU Session) between 3GPP access and non-3GPP access in 6G.

**** Second Change ****

# 5.X.	Key Issue for WT1.3: Support of non-3GPP access in 6G

This key issue proposes to study how to support the non-3GPP access in 6G.

The key issue will investigate the following aspects:

-	Study whether and how to enhance non-3GPP access in 6G System architecture based on the 5G principles.

-	Whether untrusted access type and wireline access only are considered in 6G.

-	Study whether and how the design of the UE to Core Network signalling exchange for WT#1.1 is expected to impact the integration of the non-3GPP accesses in the 6G system.

-	Study whether and how to support the multi-access data connectivity (e.g. MA PDU Session) between 3GPP access and non-3GPP access in 6G.

**** End of Changes ****