SA WG2 Meeting #170	S2-2507223

25 – 29 Aug, 2025, Goteborg, SE

Source:	Huawei, HiSilicon

Title:	[WT#3, Network AI Agent] Agentic core network for 6G

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document explores the role of AI Agents in solving fundamental issues and challenges for 6GC design and real business deployment. It assesses the current technical and ecosystem maturity of AI Agents and proposes a focused investigation into their potential to revolutionize 6G networks for all stakeholders in the mobile communications industry.

# 1. Introduction

As we transition from the era of 5G towards 6G, the mobile communications industry stands at a pivotal inflection point. Although 5G offers an ample spectrum of features and architecture innovations such as cloud-native architecture, network slicing, SBI, we are still facing several fundamental issues and challenges for 6GC design and real business deployment in the timeframe 2030+:

-	New Business Opportunities/new monetization capability based on customization and efficiency: 6G has the vision and target to support both connectivity and beyond connectivity services, e.g. AI, Data, Sensing, Computing. With the fact that in the future, there are more diverse network capabilities (more than just a connection pipe) and more customized services request from the consumers, the network architecture design should be natively intelligent to understand the customized service requirements and invoke different network capabilities and service flows dynamically to provide better service experience.

-	Total cost of operation (TCO) reduction with more intelligent network design: utilize the AI capability to support the self-learning, real-time self-optimization, efficiently make the decision and adjust the network behaviour to simplify the control complexity—leaving the complexity to AI.

-	Forward-compatible network design: the mobile communication system could cope with future service demand via flexible service and capability extension without significant requirements of relevant standardization updates. In this way, the time to market (TTM) could be reduced.

In this document, we propose a structured study of Network AI Agents, which are inherently capable of handling highly complex and dynamic consumer service requirements as defined in SA1 TR 22.870: "an automated intelligent entity capable of e.g., interacting with its environment, acquiring information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.". Their autonomous and self-learning capabilities can significantly enhance the operator’s monetization potential. Furthermore, by adopting the AI agent design paradigm, operator networks can seamlessly adapt to the broader AI ecosystem—unlocking their full potential and positioning themselves as foundational players in the global AI landscape.

In this document, several use cases are first provided to demonstrate how AI Agent could assist the network for autonomous control, agile new service deployment, adaption to the broader AI ecosystem, and therefore help operator monetize the network, which overcomes the constraints and limitations of current network. Secondly, an overview of the technical readiness and ecosystem maturity of AI Agents are described. Finally, it is proposed to launch a dedicated investigation into how AI agents can revolutionize 6G network and provide solid monetization capabilities to all stakeholders of mobile communication industry.

# 2. Gap Analysis

In this section we present a series of use cases that exemplify the fundamental challenges for 6GC design and explore the transformative role of AI Agents in addressing these challenges.

## 2.1 Gap category 1: Predefined network workflows/procedures/templates cannot satisfy customized consumer requirements and adapt to ever-changing network conditions.

Use case 1a: Consumers may have various experience assurance requirements for the network. For example, an AR game is running on UE, and the UE requests for network assistance to guarantee the game experience. There are numerous factors that can have influence on game experience, and the usage of appropriate network capabilities need to be dynamically determined based on different conditions.

For example, when the UE moves rapidly, the network needs to dynamically determine how to maintain the necessary quality of experience, for example, it could consider whether to establish a dedicated QoS bearer, enable the redundant transmission, or perform local diversion based on fast changes in network conditions such as UE location, access quality, and network load, or several other options available in that deployment, making it difficult to meet the service requirement through predefined network rules/procedures/templates.

By introducing the AI Agent into the core network, which has a rich network knowledge of the specific network deployment, the most suitable network capabilities combinations can be dynamically determined based on actual set of network conditions, fulfilling the consumer requirement and experience through the whole life time of the applications.

In addition, the current network operates in a way where the network provides services to the UE always upon request from the UE, which may limit the ability for the network to optimally provide its capabilities to its subscribers.

With AI Agent in the network, the AI Agent is able to proactively provide tailored services to specific UEs in certain use cases based on the UE status and the network conditions. For instance, the Agent can accurately determines that certain UEs may have potential requirement to improve the service experience, taking multiple factors into account related to both the network and the UE, such as user subscription tier (e.g. if the user is VIP user), the services currently in use, the current service experience, whether the UE has entered a specific location area (high-speed trains, shopping malls, popular venues, etc.), the network AI Agent can trigger changes or make these target UEs aware of some specific opportunities, such as temporary turbo packages for service experience promotion, allowing the operator to better promote and utilise its network resources for new revenue.

Use case 1b:

To support the needs of the growing market of intelligent autonomous terminals such as robots, the 6G network will not only provide connectivity services, but also offer a variety of new services including sensing, computing, and AI inference. Considering that robots need to handle diverse task scenarios, they may utilize various network capabilities including the new ones based on task intentions, and it also need dynamically adapt to the ever-changing network environment for the success of task execution. In 6G, the network capabilities have more combinations and even dependency among themselves, making the traditional predefined workflows/procedures/templates no longer applicable.

For example, the request from robots to network can include remote control, navigation, obstacle avoidance, hazard alerts, and multi-robot collaboration. Robots' task requests may imply the network to involve the dynamic combination of different network capabilities, including URLLC, positioning, sensing, trajectory prediction, computing offloading, AI model inference and 6GLAN. These capabilities need to be dynamically assembled to adapt to various scenarios, which is difficult to assess outside the network. During task execution, real-time adjustments based on different factors will be also necessary, which are specific to each network deployment requiring close loop collaboration between the robot and the network.

By introducing AI Agent into the network, this kind of customized service can be provided to autonomous terminals like Robots. AI Agents have the ability to understand these terminals’ diverse requirements, assemble various network capabilities to complete complex tasks, perceive the network environment including robots and tasks status, and control the task execution.

In the case where the robot has a task to carry an item to a place, it would elaborate a request to the network AI Agent to support its needs to e.g. safely deliver an item to a certain destination. The network AI Agent will understand the request, then obtain the robot's location and build a local 3D map based on network sensing, predicting the robot's movement trajectory and navigation routes. When other objects, possibly UEs, appear nearby, the network AI Agent predicts their paths to identify collision risks in advance and interacts with these UEs promptly. When the robots need to closely collaborate with other nearby robots, the network AI Agent can also organise shared connectivity, e.g. establish a 6GLAN or whatever is necessary for them to precise movement synchronisation.

Proposal 1:

- 6G network should support dynamic assembling, adjustment and invocation of network capabilities based on customized service requirements and different network conditions.

## 2.2 Gap category 2:  The interaction between 5GC and third-party/UE lacks flexibility, making it difficult to dynamically exchange customized information for complex network tasks.

Use case 2: Network exposure API is for dedicated network services, which means complex task requires the consumer itself to decompose the task into several required network exposure APIs, resulting in high interpretation complexity of network exposure APIs provided by MNOs and complicated combination of them for the consumer, especially with the diversity of operator deployments.

For example, the ASP (live steaming platform) would like to request the service from the operator to identify the proper area with high quality of data traffic experience and crowded people for live steaming during 18:00-20:00. However, this service requirement cannot be fulfilled by specific network exposure API, which means the ASP itself needs to determine the required APIs (e.g., Service Experience Analytics, Movement Behaviour Analytics), collect and process the data (e.g., filtering, associating, aggregating) for the above purpose. In general, not all the ASPs have the capability or willingness to understand the complex network exposure APIs and the required format of input/output.

If an AI Agent is applied for interaction between 3GPP core network and 3rd parties, then it brings intelligence in the network to translate the service request with collecting the corresponding data and generate the outputs. For example, the alternative way for the same requirement by using the AI Agent is that the ASP can forward its intent to the AI Agent, e.g., to recommend proper area in a given location with high quality of data traffic experience and crowded people for live steaming during 18:00-20:00. Then the AI Agent will decompose the task into several procedures for data collection and processing, and generate the final result to the ASP.

On the other hand, it is difficult for the core network to comprehend external non-standardized information. For example, the network has defined some IEs to present external non-standardized information, such as OSid/OSappid, which is not designed or controlled by 3GPP. It is hard to set the mapping between a specific OSid/OSappid and the application in practice reflecting the specific design of the OSes. Even for a single OS, there can be many sources for the software (e.g. Android supports many APK stores, Linux supports many software distributions, each having a variety of software repositories, Windows itself can have many sources for software beside the "Microsoft Store", etc). Moreover, new applications become available every day, possibly with more demanding needs that cannot be anticipated.

If the UE sends intent to the network, the operator does not need to pre-configure complex and endless OSAppids in the network and the UE to define rules for each and every scenario. The AI Agent in this case will ease the work from the customer side and enable the monetization of network capabilities. AI Agent can also support the provision of customized services based on consumer intent, which simplifies the network service invocation interface, eliminates the need for consumer to generate complex network API invocation parameters, and makes it more flexible and forward-compatible as well.

Proposal 2:

-	6G network should support having AI agent in the network interact with the UE or 3rd party to understand the service requests (e.g., based on intent), generate intelligently the required output and provide the necessary network information.

## 2.3 Gap category 3：Each new capability or features added to the network requires redefinition or extension of interfaces and processes, delaying time-to-market.

Use case 3: During 5G standardisation era, when a new functionality or feature has been specified, all involved NFs have had to be upgraded for this new functionality or feature to be deployed. For example, in Rel-17, EADSF is defined for DNS handling and discovery of edge services, and in order to support the EASDF functionality, the SMF needed to be upgraded to support discovery and selection of EASDF and send the DNS message handling rules and ECS option to the EASDF, to establish the user plane path to the EAS returned by the EASDF.

If AI Agents were available in the system, it could have been applied to support session management scenarios, and when it was needed to do an EAS selection, the AI Agent could take advantage of its capabilities to understand how to leverage EASDF as a tool to perform an EAS selection. For example, the system could have been designed so that the EASDF can register itself as a tool to be invoked, the network AI agent would then be able to find the EASDF and comprehend the functional description of EASDF, including the input parameters for DNS handling rules and the output of EAS address(es) by the EASDF, and then perform the EAS selection.

NSACF for slice can be another example, which was defined in Rel-18, for monitoring and controlling the number of registered UEs per network slice, monitoring and controlling the number of established PDU Sessions per network slice, and so on. To support NSACF, the AMF and the SMF needed to be upgraded to update the number of the registered UEs per slice or PDU Sessions established per slice and so on to the NSACF during the corresponding procedure and decide to accept or reject the request from the UE according to the NSAC result performed by the NSACF.

If an AI Agent had been available to support mobility management or session management, it could have found the NSACF as a tool or other appropriate data storage tool and comprehend the functional description of the tool, and then sends the request to update the number of UEs registered or the number of PDU Sessions established with the network slice.

By have Network AI agents available as part of the system, we can make use of their intrinsic capabilities to simplify the design of new features. This can be achieved based on the agent and tools design paradigm to define the network with the target to identify atomic network functionalities so that they can be invoked as tools. When introducing new services or new features, new tools and functions can be introduced without rewriting or extending existing tools or APIs. Agents can learn how to use them automatically, improving the rate of standardised features that can easily be deployed in networks, and allow 3GPP to develop more ambitious functionality

Proposal 3:

-	6G network should support the agility for new service innovation, to reduce the TTM of new service and features and minimize the impact on the existing network functionalities.

## 2.4 Summary of gap analysis

-	Inherent limitations of predefined procedures: procedures in 3GPP are defined for individual features, e.g., URLLC, MBS, XRM, 5GVN, EC, etc. As a result, when a complex interaction with the network is needed to provide a service, requiring to put in motion several of these features in different ways, it is difficult to map the required interaction with the network to combine procedures across different features in predefined patterns, as shown in Category 1.

-	Difficult to take advantage of the vast amount of information available to dynamically exchange customized information for complex network tasks: due to heavy burden on applications or operators side and inflexible information sharing, as shown in Category 2.

-	Long TTM for standardizing new network features, even basic: it usually takes years to specify a simple network feature or event, thus delay the time to market, resulting in the current network being unable to agile rollout services, and limiting the ability of 3GPP to build complex features, as shown in Category 3.

# 3. Why Network AI agent

Given the gap analysis above, it becomes apparent that continuing to use the existing approach to design features per use case or via static APIs will increasingly become unsustainable, as it will face three key limitations:

-	Explosion of permutations: It’s infeasible to pre-define all combinations of network conditions and service scenarios to guarantee optimal performance.

-	Burden on developers: The developers for applications must deeply understand API semantics, network KPIs, and system-level events to make effective decisions. While the network operator operational teams need to pre-configure complex and ever-changing information in the core network to make effective decisions for increasingly specific "niche" scenarios. This complexity is not sustainable or scalable for developers in other domains.

-	Slow adaptation: Each new capability or feature added to the network requires redefinition or extension of interfaces and processes, delaying time-to-market, and limiting the ability for 3GPP to address the ever-faster needs of the market.

Those limitations can be inherently unlocked by the use of AI agents. An AI agent is an intelligent, autonomous software entity capable of understanding its environment, reasoning over complex inputs, and making decisions or taking actions to achieve specific goals. AI agents is especially suitable to solve complex and dynamic problems where the solution needs intricate interaction with system components and take various conditions into account. Specifically,

-	Instead of opening API of single function, Network AI Agents can consolidate various network capabilities to execute complex tasks;

-	Third-party applications no longer need to track network metrics or maintain logic for complex network interactions, specifically for each operator. The AI agent abstracts away the complexity by interpreting service intents and mapping them to appropriate configurations, tools, and functions, making the available features of the network more valuable.

-	Unlike fixed rules or service profiles, the Network AI agent evolves its behaviour through reinforcement learning, model updates, or feedback loops, continuous improvement of its understanding of the network and its users, enabling, e.g., continuous performance tuning, context-aware service delivery, failure recovery and self-healing

-	Network AI agent can autonomously learn to leverage a variety of tools to meet diverse service requirements, thereby enabling greater agility in supporting new service innovations.

-	Agents can make run-time adjustment for providing the best service experience anytime, anywhere by perceiving the network environment and service conditions through self-optimization, self-reflection;

Recently, AI agents powered by LLM (Large Language Model) have achieved significant successes across many industries. For example, Google’s Med-PalM 2, a medical large language model, achieved over 85% accuracy on US medical Licensing Examination-style questions, demonstrating expert-level performance in medical reasoning [3]; In fashion industry, AI agent is being used to scans social media and shopping data to predict fashion trends, helping designers stay ahead of consumer demand [2]; In logistics, UPS’s ORION Agent is used to optimize delivery routes dynamically, saving 100 million miles annually and cut $300 million in costs [1]; In IT industry, plenty of Web Agents are developed which can take natural language instructions from users, reasoning about user intent or goals, planning and execute sub tasks, e.g., open relevant web applications, compose emails [6]. Every day, new applications building on AI capabilities are being published, new scenarios are being addressed.

Leveraging AI agent to resolve 6G challenges is not aspirational. It is technically feasible and operationally scalable. This readiness is driven by a combination of infrastructure evolution, ecosystem maturity and robust development frameworks.

Thanks to the successful adoption of cloud-native architectures in 3GPP systems, AI agents can be deployed, scaled, and dynamically updated at the precise locations where they are most needed.

Key capabilities essential to the development of effective AI agents—such as reasoning, planning, and tool integration—have been extensively studied across both academic and industrial domains [4]. In response, various architectural paradigms have emerged to support agentic system design, including ReAct, Reflexion, DyLAN, and MetaGPT, particularly for multi-agent coordination. In parallel, significant progress has been made in designing communication protocols that enable seamless interaction between agents and external tools, as well as among agents themselves. Protocols such as MCP, A2A, and ANP [5] have been developed to meet varying goals, and are engineered with properties such as efficiency, scalability, reliability, and extensibility. Moreover, numerous off-the-shelf frameworks are now available to streamline the development and deployment of autonomous agents. These include LangChain, LangGraph, AutoGen, and MARTI—each supporting agent collaboration, workflow generation, and reinforcement learning.

While we do not advocate for the adoption of any specific protocols, frameworks or tools, we acknowledge that these existing solutions offer valuable insights and practical approaches that can help inspire innovation and address potential challenges throughout the 6G study process.

Given the increasing complexity of mobile network environments, the demonstrated effectiveness of AI agents in other domains, and the maturity of the enabling technologies, now is the time to rigorously explore how AI agents can transform 6G network. What’s needed next is a structured study to identify appropriate agentic architectures for 6G applications, determine how agents will interact with dynamic network environments, and establish governance and trust frameworks that ensure safe and reliable deployment.

# 4. Technical Impact

Technical Impact:

-	The Network AI agents are introduced as network entities for complete network tasks based on consumer intent.

-	Network tools are introduced to support Network AI agents to complete tasks, e.g, configure user traffic path, get network events, manipulate various network data.

-  Interaction needs to be defined among AI agents.

-	NFs may support tool-based interface to be called by the Network AI agent.

-	Intent-based service interfaces should be introduced that allow AI agents to interpret, fulfil, and optimize consumer demands.

-	Cross impact with features both for connectivity (NAS, UP, SBA, Exposure, etc) and beyond connectivity (data, sensing, computing).

# 5. References

[1] UPS and Agentic AI: A Case Study in Logistics Innovation, https://cdotimes.com/2025/01/06/ups-and-agentic-ai-a-case-study-in-logistics-innovation/#google_vignette, 2025

[2] AI-Powered Fashion: How Tech is Reshaping the Future of Zara’s Fashion Empire, AI-Powered Fashion: How Tech is Reshaping the Future of Zara’s Fashion Empire – Michigan Journal of Economics, 2025

[3] Med-PaLM 2: Will This Google Research Help You Increase Your Healthspan? Med-PaLM 2: Will This Google Research Help You Increase Your Healthspan? – Be on the Right Side of Change, 2023

[4] Masterman T, Besen S, Sawtell M, Chao A. The landscape of emerging ai agent architectures for reasoning, planning, and tool calling: A survey. arXiv preprint arXiv:2404.11584. 2024 Apr 17.

[5] Yang Y, Chai H, Song Y, Qi S, Wen M, Li N, Liao J, Hu H, Lin J, Chang G, Liu W. A survey of ai agent protocols. arXiv preprint arXiv:2504.16736. 2025 Apr 23.

[6] Ning L, Liang Z, Jiang Z, Qu H, Ding Y, Fan W, Wei XY, Lin S, Liu H, Yu PS, Li Q. A survey of webagents: Towards next-generation ai agents for web automation with large foundation models. arXiv preprint arXiv:2503.23350. 2025 Mar 30.

**** First Change ****

# Annex A.3. WT#3 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	Define, support and enable Network AI Agents in 6G architecture.

# Annex A.1. WT#1 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	NAS: Support interaction between Network AI agent and UE.

-	Mobility, Access control, Session management, Policy framework, etc: support exposure and control of services by Network AI agent.

-	Exposure: Support consolidation of exposure framework.

-  New Interface: Support interaction among Network AI agents and other NFs

# Annex A.4. WT#4 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

As part of the study of integration of sensing and communication:

-	Introduce the support of integration of Sensing and communication for use by Network AI agents.

# Annex A.5. WT#5 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

As part of the study of a network-wide data framework:

-	Introduce the functionality in the data framework to support the Network AI agents interaction with Data framework regarding data access, processing and other operations to fulfil data requirements.

# Annex A.6. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

As part of the study of a network-wide generic computing capability in the network:

-	Introduce the support for Network AI agents to access computing resources to fulfil the computation requirements (e.g., offloading UE computation tasks).

**** Second Change ****

# 5.X. Key Issue #X: Support of AI Agent in core network

This key issue will address the Network AI Agent aspects within the 6G system architecture. This key issue will attempt to develop a comprehensive network AI Agent framework for the 6G system:

-	Functionalities of network AI agent to support connectivity and beyond connectivity service:

-  How to define network AI Agents and their functionalities,

-  How to allow network AI Agents to access and support connectivity and beyond connectivity services,

-  How to support the registration and discovery of network AI Agents,

-	How to allow network AI Agents to collaborate to fulfil complex tasks,

-	How to allow network AI Agents interact with Data framework regarding data access, processing and other operations to fulfil the data requirements,

-	How to allow network AI Agents to access computing resources to fulfil the computation requirements,

-	How to allow network AI Agents to invoke sensing processing.

NOTE: the granularity of network AI agent is to be further clarified as part of the solutions to the key issue.

-	Interactions between network AI agents and other entities:

-	other network AI agents,

-	How Network AI agent interact with UE or 3rd party entities, e.g, based on intent.

-	How Network AI agent interact with other network functions.

**** End of Changes ****