3GPP SA WG2#170	S2-2507228

Goteborg, Sweden, 25-29 August, 2025	(revison of S2-250xxxx)

Source:	OPPO

Title:	New solution to KI#6 on Sensing Task aggregation

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution to KI #6 on sensing task aggregation for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

This paper proposes a new solution to KI#6 on sensing task aggregation when configuring Sensing Entities for saving the sensing resources.

Sensing data could be collected and processed for multiple sensing services if the sensing requirements can be met simultaneously. For example, a Sensing Entity might satisfy multiple separate sensing requests for mobility tracking, interference mapping, and QoS validation with a single dataset. So it is proposed to allow Sensing Function collect sensing data from the same Sensing Entity with a single sensing task for multiple services in order to reuse the sensing data more efficiently.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.2.0.

* * * * Start of Changes (All text new)* * * *

6.X	Solution #X: Sensing Task aggregation

6.X.0	High-level solution Principles

The solution is based on the following general principles to support sensing service:

1)	The Gateway Function can aggregate different sensing requests to the same sensing session according to the service requirements.

2)	The Sensing Function configures the Sensing Entity with the same sensing task for different sensing sessions according to the sensing requirements.

3)	The Sensing Function would priorly select and configure the Sensing Entity with similar on-going sensing task for a new sensing session.

6.X.1	Description

This solution resolves KI#6 for sensing task aggregation.

6.X.2	Procedures

Figure 6.x.2-1: Procedure for sensing task aggregation

- Sensing Consumer sends sensing service request indicating target area and related sensing requirements (e.g. sensing mode, sensing QoS) to the Gateway Function.

- Gateway Function may aggregate serveral simmilar sensing service requests accoring to the target area and sensing requirements and allocates the same sensing session ID.

3.	Gateway Function sends sensing service request to the selected Sensing Function(s) with the allocated sensing session ID and aggregated sensing service request.

4.	Sensing Function selects Sensing Entities based on the target area and sensing requirements. The Sensing Function may priorly select the Sensing Entity with similar on-going sensing task for the sensing session.

5.	Sensing Function Configures the Sensing Entity with a single sensing task for different sensing sessions according to the target area and sensing requirements. For instance, the Sensing Function can configure the Sensing Entity that covers the target area of different sensing sessions according to the strictest sensing requirements (e.g. highest accuracy, lowest latency, etc.).

6.	Sensing Entity performs Sensing measurement and sends sensing data report to Sensing Function.

7.	Sensing Function calculates the sensing results for different sensing sessions according to the sensing data of the same sensing task.

8.	Sensing Function notifies Gateway Function about the sensing results for sensing sessions.

9.	Gateway Function exposes sensing results to Sensing Consumer for each sensing request according to the sensing results of aggregated sensing session.

6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

Gateway Function:

-	Support to aggregate different sensing requests to the same sensing session according to the service requirements.

Sensing Function:

-	Support to select the Sensing Entities according to the on-going sensing task for a new sensing session.

-	Support to update the configuration of an on-goning sensing task to the Sensing Entities for new sensing session.

-	Support to configure the Sensing Entity with the same sensing task for different sensing sessions according to the sensing requirements.

* * * * End of Changes * * * *