3GPP TSG-SA WG2 Meeting #170	S2-2507238

Goteborg, Sweden, Aug 25 – 29, 2025

Source:	China Mobile

Title:	Key issue for WT#1: Study the support of AIoT services in Topology 2

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_ARC_Ph2/ Rel-20

Abstract of the contribution: The contribution proposes the key issue for WT#1 on how to support AIoT services in Topology 2.

- Discussion

FS_AmbientIoT_ARC_Ph2 SID was approved with the following WT#1 for supporting AIoT services in Topology 2:

WT#1: Study the support of AIoT services in Topology 2, including: - Study and investigate how to support AIoT services under the RRC-based option for UE Reader connectivity. NOTE 3: Rel-19 TR solution and interim conclusion should be used as basis for rel-20 work. Solution submitted to rel-20 need to align with rel-19 TR interim conclusion. No rel-19 AIoT Device impact from this WT is expected.

This paper proposes a key issue to address WT#1 on how to support AIoT services in Topology 2.

The AIOTF may select one or more targeted AIoT T2 UE readers based on their identification (such as GPSI) or target area information, as well as their serving AF ID.

In some AIoT application scenarios with T2 readers’ networking, such as AF-initiated inventory service operation with target area information, it is necessary for the core network to ensure that the selected targeted T2 UE reader(s) can cover the entire service area (such as the whole 7-11 store’s service area). Otherwise, the result of the inventory service operation is untrustworthy. However, due to the UE Reader’s mobility feature, the core network finds it challenging to select targeted UE readers to meet the above requirement. The core network needs to be enhanced to ensure the selected targeted T2 UE reader(s) can cover the entire service area.

To treat T2 UE reader(s) as T1 mini–Base Station Reader(s) in situations with smaller service area needs, such as convenience stores, 7-Eleven, and similar venues, the UE reader’s registration may be improved by including a fixed or mobile UE reader type indication. This allows the core network to treat the fixed UE Reader(s) as mini–Base Station Reader(s) during AIoT UE Reader Selection, ensuring that the chosen T2 targeted UE reader(s) can fully cover the target service area in certain scenarios, thus providing a cost-effective and quickly deployable solution.

It is proposed to agree on the following changes to 3GPP TR 23.700-30 v0.0.0.

- Proposal

* * * Start of Change (all new)* * * *

5.x	Key Issue #x: Study the support of AIoT services in Topology 2

5.x.1	Description

The key issue is to address the support of AIoT services in Topology 2.  The Key issue aims to study the following aspects:

- Study the architecture under the RRC-based option to support AIoT services (e.g., inventory, command, and disable).

- Study the enhancement of the AIoT UE reader’s registration procedure, including authentication, authorization, and management of the UE reader’s profile.

- Study how to assist in allocating radio resources between RAN and AIoT UE reader(s) during AIoT services.

- Determine which NF is responsible for AIoT UE Reader Selection.

- How does the Core Network determine the selection of the AIoT UE Reader based on UE Reader identification (e.g., GPSI) or Target Area Information (e.g., TAC list)?

- Study how to ensure that selected AIoT UE reader(s) can cover the entire service area required by AF to guarantee AIoT services?

* * * End of Change * * * *