3GPP TSG-WG SA2 Meeting #170	S2-2507240

Goteburg, Sweden, Aug 25 – 29, 2025

Source:	China Mobile

Title:	KI#4 New solution: Sensing data and associated information transport via user plane or control plane connection

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution to KI#4 on sensing data and the associated information transport for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

In some sensing application scenarios, such as intelligent transportation and intelligent manufacturing, the sensing entity(s) may generate large amounts of sensing data. Using the user plane to transfer sensing data from sensing entity(s) to the core network is more efficient, although it requires establishing an extra user plane.

However, in some other sensing application scenarios, such as intruder detection at home or UAV presence detection, the sensing entity(s) may produce very little sensing data to report a binary sensing result (yes or no) to the core network. It is more efficient to use the control plane to transfer sensing result data from sensing entity(s) to the core network without the extra overhead of establishing the user plane.

Taking these factors into consideration, the core network can determine the sensing data transfer method (UP or CP) based on AF ID, sensing service type, sensing target area, sensing quality of service requirements, and SLA. It is very similar to the LCS service; the LMF can determine whether to use the control plane or user plane to perform the LCS service. Then, the core network includes the sensing data transfer method indication in the sensing service request message sent to the sensing entity(s). The sensing entity(s) can determine how to transfer the sensing data to the core network based on the sensing data transfer method indication.

This paper introduces a new solution to address KI#4 on sensing data and the associated information transport. This solution refers to clause 6.18 of TS 23.273 on the LCS-UP connection establishment procedure and clause 6 of TS 23.273 on the LCS-CP connection establishment procedure.

Based on the assumptions above, the following procedures are provided.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.2.0.

* * * * Start of Changes (All text new) * * * *

6.X	Solution #X: Sensing data and associated information transport via user plane or control plane connection

6.X.0	High-level solution Principles

The solution is based on the following general principles to support the sensing service:

1)	The sensing data and associated information are transmitted between the Sensing Entity and the Sensing Function via the user plane or control plane connection.

2)	The data connection establishment procedure is carried out on the control plane signalling interaction.

3)	The Sensing Function can determine the user plane or control plane based on AF ID, Sensing service type, SLA, Sensing service quality of service, Sensing entity capability, Sensing target area, and related factors.

4)	The Sensing Function includes the sensing data transfer method indication in the sensing service request message sent to the sensing entity(s). The sensing entity(s) can determine how to transfer the sensing data to the core network based on the sensing data transfer method indication

6.X.1	Description

This solution addresses KI#4, focusing on sensing data and associated information transport.

the core network can determine the sensing data transfer method (UP or CP) based on AF ID, sensing service type, sensing target area, sensing quality of service requirements, and SLA. It is very similar to the LCS service; the LMF can determine whether to use the control plane or the user plane to perform the LCS service. Then, the core network includes the sensing data transfer method indication in the sensing service request message sent to the sensing entity(s). The sensing entity(s) can determine how to transfer the sensing data to the core network based on the sensing data transfer method indication.

6.X.2	Procedures

Figure 6.x.2-1: Procedure to deliver the sensing data between the Sensing Function and the Sensing Entity via UP or CP

1.	The sync-up procedure between sensing entity(s) and the core network regarding the capability to support sensing data delivery from sensing entity(s) to the core network (CP, UP, or both CP and UP).

1a.	If the sensing entity is the UE sensing entity, it uses the registration procedure with sensing data delivery capability information to inform the core network about the supported delivery methods. The sensing data delivery method details will be stored in the UE sensing entity’s profile in UDM/UDR.

1b.	If the sensing entity is the RAN sensing entity, it uses the following options to synchronize the data delivery method information between the RAN sensing entity and the core network.

Option 1: If the RAN sensing entity connects indirectly with the SF via AMF, the RAN sensing entity will inform the AMF of its sensing data delivery method during the NGAP setup. Then, the SF can obtain the RAN sensing entity’s data delivery method information from the AMF by subscription/notification mechanism.

Option 2: If the RAN sensing entity connects directly with the SF, the RAN sensing entity will inform the SF of its sensing data delivery method during the NGAP setup.

Option 3: The SF obtains the RAN Sensing entity’s data delivery method through local configuration.

2.	AF sends the sensing service request either directly to the SF or through the NEF to the SF. The request message may include the following parameters:

AF ID;

Sensing Target Area List;

Sensing service type (e.g., monitor, tracking, etc.);

Sensing service quality of service requirements.

3. The SF responded to the request from AF.

4. The SF selects the targeted sensing entity as described in KI#3.

5. The SF decides whether to use the user plane or control plane to deliver sensing data from target sensing entity(s) to the SF based on the following information:

AF ID;

Sensing Service quality of service requirement;

Sensing Service Type;

Target Sensing Entity(s), including their sensing data delivery method capability.

NOTE:  The SF may obtain information related to the UE sensing entity from UDM/UDR.

6. The SF sends the sensing service request to the target sensing entity(s) with the sensing data delivery indication (CP or UP) parameter. If the sensing data delivery indication is set to CP, then CP-related parameters (e.g., SF ID) are included in the request message. If the sensing data delivery indication is set to UP, the UP-related parameters (e.g., SF User plane IP address or FQDN, port) are included in the request message.

7. Sensing Entity(s) determines the sensing data delivery method based on the sensing data delivery indication. If the sensing data delivery indication is set to the UP method, it will use the user plane method to deliver the sensing data to the SF. If the sensing data delivery indication is set to the CP method, it will use the control plane method to deliver the sensing data.

8. SF sends the sensing service notification message to the AF with the sensing result.

6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

Sensing Function:

-	The Sensing Function can determine whether to use the user plane or control plane to deliver sensing data from the sensing entity(s) to the Sensing Function.

-	The Sensing Function supports including the sensing data delivery indication parameter in the sensing service request towards sensing entity(s).

Sensing Entity:

-	Support for transferring sensing data and the associated information with the Sensing Function via the user plane or the control plane based on the sensing data delivery indication.

* * * * End of Changes * * * *