3GPP TSG-WG SA2 Meeting #170	S2-2507251

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	WT1 New Sol: Topology 2 RRC-based option baseline architecture

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Rel-19 provided interim conclusions for topology 2 support for RRC option for T2.

# 1. Introduction/Discussion

This solution provides the conclusion from the Rel-19 TR 23.700-13 for topology 2 based on the RRC option into the Rel-20 TR.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new) * * * *

## 6.X	Solution #X: Topology 2 RRC-based option baseline architecture

### 6.X.0	High-level solution Principles

The Rel-19 Ambient IoT study (see TR 23.700-13 [x]) provided conclusions for 2 Topology 2 architectures, while the KI for topology 2 focuses only on the RRC-based option. This is the Rel-19 RRC-option baseline solution based on the Rel-19 conclusions.

The Rel-19 conclusion provided subscription aspects, radio resource management aspects and how requests/responses are transferred between the AIOTF and UE Reader.

### 6.X.1	Description

Subscription Aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

-	UE Reader subscription information is available to AMF and AIOTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then the AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g., taking into account assistance information received from the AIOTF) from the gNB.

NOTE 1:	Assistance information is specified in TS 23.369 [x].

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

In the indirect connectivity architecture messages between the UE Reader and the AIOTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 6.x.1-1.

Figure 6.x.1-1: Protocol Stack for the indirect connectivity RRC option

NOTE x:	The UE requires a serving AMF, and the AMF used by the AIOTF does not need to be the same AMF.

In the direct connectivity architecture messages between the UE Reader and the AIOTF are delivered using RRC between UE and RRC and NGAP between gNB and AIOTF. The related protocol stack is shown in Figure 6.x.1-2.

Figure 6.x.1-2: Protocol Stack for the direct connectivity RRC option

NOTE x:	The UE requires a serving AMF which is not shown in the direct connectivity protocol stack.

### 6.X.2	Procedures

#### 6.x.2.1	Subscription Aspects

##### 6.x.2.1.1	General

In order to support AIOT radio resource control in NG-RAN, AIoT UE Reader Authorisation information needs to be made available in NG-RAN for the UE.

##### 6.x.2.1.2	Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the AIoT UE Reader Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the AIoT UE Reader Capability operation.

-	The AIoT UE Reader Capability indicates whether the UE supports performing Ambient IoT service operations.

-	The AMF obtains the UE subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to provide AIoT UE Reader operations based on UE's AIoT UE Reader Capability and the AIoT Service Authorisation included in the subscription data received from UDM

-	If the UE is authorised to provide AIoT UE Reader operations, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"AIoT UE Reader Authorisation" information

-	If the UE is authorised to provide AIoT UE Reader operations, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

##### 6.x.2.1.3	Service Request procedure

The Service Request procedures for UE in CM-IDLE state are performed as defined in TS 23.502 [5] clause 4.2.3.2 and clause 4.2.3.3 with the following additions:

-	If the UE is authorised to provide AIoT UE Reader operations, then the AMF shall include "AIoT UE Reader Authorisation" information in the NGAP message, indicating the UE is authorised to provide AIoT UE Reader operations.

##### 6.x.2.1.4	N2 Handover procedure

The N2 based handover procedures for UE are performed as defined in TS 23.502 [5] clause 4.9.1.3 with the following additions:

-	If the UE is authorised to provide AIoT UE Reader operations, then the target AMF shall send the " AIoT UE Reader Authorisation" information, to the target NG-RAN in the NGAP Handover Request message.

##### 6.x.2.1.5	Xn Handover procedure

The Xn based handover procedures for UE are performed as defined in TS 23.502 [5] clause 4.9.1.2 with the following additions:

-	If the " AIoT UE Reader Authorisation" information is included in the UE context, then the source NG-RAN shall include a "AIoT UE Reader operations authorised" information, in the XnAP Handover Request message to the target NG-RAN.

-	If the " AIoT UE Reader Authorisation" information is included in the UE context, then the AMF shall send the " AIoT UE Reader Authorisation" information to the target NG-RAN in the Path Switch Request Acknowledge message.

#### 6.x.2.2	Procedures between AIOTF and UE using indirect connectivity architecture

An AIOTF and UE use an indirect interface via an AMF and NG-RAN as described in clause 6.x.1. The additional steps used between AIOTF and UE are shown in Figure 6.x.2.2-1.

Figure 6.2.4-1: Procedure for AIOTF and UE connectivity via NG-RAN Indirect Connectivity

1.	The AIOTF sends Namf_AIoT_MessageDelivery message (NGAP AIoT information, NG-RAN ID, AIOTF ID, Message Type for NGAP AIoT information) to the AMF. The NGAP AIoT information may be Inventory Request Transfer or Command Request Transfer.

NOTE x:	How the AIOTF, AMF and/or NG-RAN determine which UE to use and how to ensure that the UE is in CM_CONNECTED will be defined by other solutions.

2.	The AMF sends an NGAP message (AIOTF ID, NGAP AIoT information) to the target NG-RAN.

3.	NG-RAN verifies that that the UE has "AIoT UE Reader operations authorised".

The NG-RAN sends messages to the UE and receives messages from the UE using RRC, including information from NGAP AIoT information which provides the operation and its associated parameters for the AIoT operation for the UE to perform and the results from the UE.

NOTE y:	The details of the RRC messages, sequences, radio resource allocation for AIoT will be defined by RAN WG2.

4.	NG-RAN sends an NGAP message (AIOTF ID, NGAP AIoT information) to an AMF. NGAP AIoT information may be Inventory Response Transfer, Inventory Report Transfer, Inventory Failure Transfer, Command Response Transfer or Command Failure Transfer.

4.	AMF sends the Namf_AIoT_Notify message (NGAP AIoT information) to the AIOTF.

#### 6.x.2.2	Procedures between AIOTF and UE using direct connectivity architecture

An AIOTF uses a direct interface towards NG-RAN as described in clause 6.x.1. The additional steps used between AIOTF and UE are shown in Figure 6.x.2.2-1.

Figure 6.2.4-1: Procedure for AIOTF and UE connectivity via NG-RAN Direct Connectivity

1.	The AIOTF determines the NG-RAN node and sends an NGAP message (AIOTF ID, NGAP AIoT information) to the target NG-RAN. The NGAP AIoT information may be Inventory Request Transfer or Command Request Transfer.

NOTE x:	How the AIOTF, AMF and/or NG-RAN determine which UE to use and how to ensure that the UE is in CM_CONNECTED will be defined by other solutions.

2.	NG-RAN verifies that that the UE has "AIoT UE Reader operations authorised".

The NG-RAN sends messages to the UE and receives messages from the UE using RRC, including information from NGAP AIoT information which provides the operation and its associated parameters for the AIoT operation for the UE to perform and the results from the UE.

NOTE y:	The details of the RRC messages, sequences, radio resource allocation for AIoT will be defined by RAN WG2.

3.	NG-RAN sends an NGAP message (AIOTF ID, NGAP AIoT information) to the AIOTF. NGAP AIoT information may be Inventory Response Transfer, Inventory Report Transfer, Inventory Failure Transfer, Command Response Transfer or Command Failure Transfer.

### 6.X.3	Impacts on Services, Entities and Interfaces

UE:

-	Needs to support AIOT UE Reader operations.

-	Exchanges capability to operate as a UE reader with the AMF during Registration procedures

NG-RAN:

-	Receive whether the UE is authorized for AIoT UE Reader operations from the AMF, or via Nx and N2 handover procedures.

-	Receive AIoT Information and instruct the UE over RRC to perform AIoT operations and obtain results from the UE.

AMF:

-	Support capability exchange for the UE supporting AIoT UE Reader operations

-	Determine whether the UE is authorized to be a UE Reader based on subscription data.

-	Send whether the UE is authorized for AIoT UE Reader operations to the NG-RAN.

AIOTF:

- Updated Reader selection, taking into account UE Readers.

NOTE: The updated procedures for reader selection will be defined by other solutions.

UDM:

-	Updated UE subscription data for whether the UE is authorized for AIoT UE Reader operations.

# * * * * End of changes * * * *