3GPP TSG-WG SA2 Meeting #170	S2-2507252

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	WT1 Interim Agreement: Topology 2 Interim Agreements based in Rel-19 Study

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Rel-19 provided interim conclusions for topology 2 support for RRC option for T2, these are converted to interim agreements in Rel-20.

# 1. Introduction/Discussion

These Rel-20 interim agreements are copies of the interim conclusion from Rel-19 TR 23.700-13 for topology 2 for the RRC option.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new) * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI1

The following principles apply for Topology 2:

-	The architecture and procedures for Ambient IoT defined in TS 23.369 [x] are used as a baseline for topology 2 support, with the difference that a UE Reader performs AIoT Service Operation requests from an AIOTF instead of a RAN Reader.

-	Subscription Aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

-	UE Reader subscription information is available to AMF and AIOTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then the AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g., taking into account assistance information received from the AIOTF) from the gNB.

NOTE 1:	Assistance information is specified in TS 23.369 [x].

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

In the indirect connectivity architecture messages between the UE Reader and the AIOTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.y-1.

Figure 7.1.y-1: Protocol Stack for the RRC option

NOTE x:	The UE requires a serving AMF, and the AMF used by the AIOTF does not need to be the same AMF.

In the direct connectivity architecture messages between the UE Reader and the AIOTF are delivered using RRC between UE and RRC and NGAP between gNB and AIOTF. The related protocol stack is shown in Figure 7.1.y-2.

Figure 7.1.y-2: Protocol Stack for the direct connectivity RRC option

NOTE x:	The UE requires a serving AMF which is not shown in the direct connectivity protocol stack.

# * * * * Second change (all new) * * * *

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI1

The following topics require further consideration:

-	How the AIOTF determines which AMF/NG-RAN node and the path to the NG-RAN node to use to send NGAP AIOT Information for NG-RAN to instruct a UE Reader.

-	How the AIOTF, AMF and/or NG-RAN determine which UE to use and how to ensure that the UE is in CM_CONNECTED.

-	How to revoke authorisation of a UE Reader.

# * * * * End of changes * * * *