3GPP TSG-WG SA2 Meeting #170	S2-2507253

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[KI#2, New Solution] AIoT Device Registration Procedure

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Solution for Registration Procedure for the DO-A Capable AIoT device.

# 1. Introduction/Discussion

The following is the solution for registration procedure for DO-A capable device relating to the following WT in Study on Architecture support of Ambient power-enabled Internet of Things - Phase 2:

WT#2: Study the support of DO-A Capable AIoT Devices, including: - Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure). - Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF. - Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF. NOTE 4: topology 2 aspect of WT#2 has dependency on WT#1.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new) * * * *

## 6.X	Solution #X: AIoT Device Registration Procedure

### 6.X.0	High-level solution Principles

DO-A capable AIoT Devices have the ability to send MO originating signalling and data and the AIoT Device actively registers to the network to inform the network of its presence autonomously.

### 6.X.1	Description

This solution addresses KI#X, and the aspect about how to enable the device to inform the network of its presence autonomously for Topology 1.

In this solution, it is assumed that the AIoT Device actively initiates the registration process with the network. The registration type can be initial registration or a mobility registration update when it to moves outside of an AIoT Registration Area. When an AIoT Device registers a context is created in the network for the AIoT Device and the AIoT Device is provided with an AIoT Registration Area.

Editor’s Note: It is FFS whether other registration type need to be supported.

According to TS 23.369 [x] and TS 33.369 [y], AIOTF supports termination of AIoT NAS protocol with an AIoT Device and supports the authentication of the AIoT Device. In this solution, it is also assumed that AIOTF performs the NAS termination and device authentication for DO-A capable AIoT Devices and the AIOTF also performs registration management and mobility management for AIoT Devices.

### 6.X.2	Procedures

Figure 6.X.2.1: Registration Procedure

1.	The AIoT Device sends a D2R message to NG-RAN, including D2R parameters and a NAS Registration Request (Registration Type, device information, security parameters).

The D2R parameters contains, e.g., a Temporary ID, the selected PLMN ID and allows NG-RAN to route the request to the appropriate AIOTF.

Editor’s Note: The details of the D2R parameters need to align with RAN WG.

The Registration Type indicates if the AIoT Device is perform an Initial Registration or a Mobility Registration Update. The AIoT Device performs an Initial Registration if it is not registered with the network and performs a Mobility Registration if it has moved outside of its AIoT Registration Area.

2.	The NG-RAN selects AIOTF based on the D2R parameters, the use AIoT from the AIoT Device or its local configurations. If there is AMF, NG-RAN sends the selected AIOTF ID to AMF which routes the messages to the AIOTF from NG-RAN and vice-versa.

3.	NG-RAN sends the Registration Request to the selected AIOTF. If the temporary ID indicates another AIOTF, the selected AIOTF fetches the AIoT Device context from that AIOTF. The NG-RAN also sends the selected PLMN ID and Location Information to the AIOTF.

4.	[Conditional] The new AIOTF determines the old AIOTF using the 5G-GUTII, and retrieves the AIoT Device context from the old AIOTF.

5.	The AIOTF performs Authentication/Security procedure, and stores the security context.

Editor’s Note: Details of authentication/security procedures will be determined by SA3 WG.

6.	The AIOTF determines the ADM for the AIoT Device based on the AIoT Device’s permanent ID.

7.	The AIOTF may register itself with ADM as serving AIOTF for the AIoT Device and may retrieve the subscription data for the AIoT Device.

Editor’s Note: The subscription data of DO-A capable device is FFS.

8.	[Conditional] The ADM informs the old AIOTF using Nadm_SDM_unsubscribe that it is no longer the service AIOTF for the AIoT Device.

9.	The AIOTF sends the Registration Accept to the AIoT Device, including a new temporary ID and AIoT Registration Area.

### 6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Support Registration and Mobility Management.

NG-RAN:

-	Supports AIOTF selection and transferring NAS messages to and from the AIoT Device.

AIoT Device:

-	Support performing registration procedure.

ADM:

-	Support new interface with AIOTF.

# * * * * End of changes * * * *