3GPP TSG-WG SA2 Meeting #170	S2-2507254

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[KI#2 New Solution] AIoT Data Transmission Procedure

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Solution for DO-A data transmission procedure for the DO-A Capable AIoT Device.

# 1. Introduction/Discussion

The following is the solution for DO-A data transmission procedure for DO-A capable device relating to the following WT in Study on Architecture support of Ambient power-enabled Internet of Things - Phase 2:

WT#2: Study the support of DO-A Capable AIoT Devices, including: - Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure). - Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF. - Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF. NOTE 4: topology 2 aspect of WT#2 has dependency on WT#1.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new) * * * *

## 6.X	Solution #X: AIoT Data Transmission Procedure

### 6.X.0	High-level solution Principles

DO-A capable AIoT Devices have the ability to send MO originating signalling and data. The AIoT Device actively sends data to the network autonomously without triggers from the network. The transferred data is included in AIoT NAS messages. The network is configured with a route for uplink data from the AIOT Device and the route is used when traffic is sent.

### 6.X.1	Description

This solution addresses KI#X, and the aspect about how to enable the device to perform DO-A data transmission in Topology 1.

In this solution, it is assumed that the AIoT Device can actively initiates the data transmission procedure without any trigger from the network.

According to TS 23.369[x] and TS 33.369 [y], the AIOTF supports termination of AIoT NAS protocol with an AIoT Device and supports the service operation towards AIoT Devices. In this solution, it is also assumed that AIOTF is the NAS termination point, and AIOTF supports the service operation from the AIoT Device. The AIOTF is able to route the DO-A data to the corresponding AF.

### 6.X.2	Procedures

#### 6.x.2.1	Configuration of MO data routing

The procedure to configure the routing of uplink traffic from an AIoT Device is show in figure 6.x.2.1-1.

Figure 6.X.2.1-1: DO-A Target AF information configuration Procedure

1.	The AF sends an Nnef_DOAconfiguration_Create Request message (AIoT Device information, AF Identifier, valid time) to the NEF. The valid time is used to indicate the how long network maintain the DO-A configuration information.

2.	The NEF selects ADM based on the AIoT Device information received in step 1.

3.	The NEF sends a Nadm_DOAConfiguration_Create Request message (AIoT Device information, AF Identifier, valid time) to the ADM.

4.	The ADM validates the request from the NEF and stores the AF information for devices’ DO-A data transmission, i.e.,, AF Identifier and valid time.

5.	The ADM sends the Nadm_DOAConfiguration_Create Response message to NEF to confirm the route has been stored.

6.	The NEF sends Nnef_DOAconfiguration_Create Response to the AF to acknowledge acceptance of the Nnef_DOAconfiguration_Create Request.

#### 6.x.2.2	DO-A traffic routing

The procedure to use the routing information stored in the ADM to route DO-A traffic from an AIoT Device is shown in figure 6.x.2.2-1.

Figure 6.X.2.2-1: DO-A Data Transmission Procedure

1.	The AIoT Device sends D2R message to NG-RAN, including Temporary ID and AIoT UL NAS message (Temporary ID, DO-A Data).

2.	Based on the Temporary ID, NG-RAN selects an AIOTF, and forward the AIoT UL NAS message and the RAN AIoT device NGAP ID to the selected AIOTF. If there is AMF, NG-RAN sends the selected AIOTF ID to AMF to route the messages.

3.	The AIOTF obtains the target AF information for the AIoT Device through subscribing to the ADM.

4.	The AIOTF forwards the AIoT Device ID, DO-A Data and the target AF information an NEF. The NEF does not have to be the same NEF that that AF used to provide the routing information to the network.

5.	The NEF forwards the AIoT Device ID, DO-A Data to the AF based on the target AF information.

### 6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Retrieving target AF ID information from ADM.

NG-RAN:

-	Support AIOTF selection.

NEF:

-	Service operations to allow an AF to register a route and sending DO-A to an AF.

ADM:

-	Storing the routing information (AF identifier) per AioT Device and providing it to the AIOTF.

AIoT Device:

-	Sending DO-A data:

# * * * * End of changes * * * *