3GPP TSG-WG SA2 Meeting #170	S2-2507256

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[KI#2, New Solution] Command Procedure for Registered DO-A capable AIoT Devices

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Solution for Command procedure for the DO-A Capable AIoT device.

# 1. Introduction/Discussion

The following is the solution for Command procedure for DO-A capable device relating to the following WT in Study on Architecture support of Ambient power-enabled Internet of Things - Phase 2:

WT#2: Study the support of DO-A Capable AIoT Devices, including: - Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure). - Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF. - Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF. NOTE 4: topology 2 aspect of WT#2 has dependency on WT#1.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new) * * * *

## 6.X	Solution #X: Command Procedure for Registered DO-A capable AIoT Devices

### 6.X.0	High-level solution Principles

When an AIoT Device registers with the network a context is created in an AIOTF. When an MT data delivery to an AIoT Device is performed, the AIOTF that performs the MT data transmission needs to be the one that holds the context for an AIoT Device. The procedure allows delivery of MT data to registered AIoT Devices by determining the serving AIOTF for the AIoT Devices and selecting that one.

### 6.X.1	Description

This solution addresses KI#X, and the aspect about how to enable the device to perform MT traffic delivery to DO-A capable AIoT Devices in Topology 1 that have registered with the network.

In this solution, it is assumed that the AIoT Devices have already registered to the network. According to TS 23.369[x], the AIOTF supports termination of AIoT NAS protocol from AIoT Devices and supports the service operation towards AIoT Devices. In this solution, it is also assumed that AIOTF performs the NAS termination.

The Command Request from AF may be performed for some specific AIoT Devices, or a group of AIoT Devices in a certain area. For the former one, the NEF can select the serving AIOTFs based on the serving AIOTF in ADM. However, for the latter one, it is hard to guarantee the AIOTF selected by the Target area information is the serving AIOTF for all the AIoT Devices located in that area. Hence, for the uplink Inventory result, the selected AIOTF may needs to fetch the AIoT Device context from the serving AIOTF by using the temporary ID received in the Inventory Report to be able to send MT traffic to the AIoT Device.

### 6.X.2	Procedures

The Command Procedure as specified in TS 23.369 [xx], clause 6.2.3 is performed with following differences:

-	In step 2, if the information about the target AIoT Device(s) has been provided, the NEF determines the corresponding serving AIOTFs by querying the ADM for the serving AIOTF based on the AIoT Device ID, and uses that AIOTF to perform inventory procedure.

-	In step 7, the AIOTF may find the serving AIOTF based on routing information, e.g., from within the temporary ID, and fetch the AIoT Devices context from the serving AIOTF. It also registers/updates itself to ADM as new serving AIOTF.

If the serving AIOTF has changed, a routing information, e.g., a new temporary ID, needs be generated and provided to the AIoT Device.

-	In step 8, the new routing information, e.g., a new temporary ID, is provided to the AIoT Device along with the NAS Command Request message.

### 6.X.3	Impacts on Services, Entities and Interfaces

AIOTF:

-	Support fetching AIoT Device contexts from serving AIOTF.

-	Support generating new routing information, e.g., a new temporary ID for the AIoT Device.

NEF:

-	Support selecting serving AIOTF to perform command procedure.

# * * * * End of changes * * * *