3GPP TSG-WG SA2 Meeting #170	S2-2507257

# Goteborg, SE, 25th Aug – 29th Aug, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0047 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on AIoT Data, AIoT Command services and AIoT NAS messages |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The AIoT Command service clause should clearly describe the interactions with the AIoT Device via AIoT NAS messages and the service operations used within a network. At present this separation is not clear. It is also not clear how the Command type and Command type specific parameters within the AIoT command service operations are used to determine how to interact with AIoT Device(s). The term AIoT Data is not the term used for data that is transferred between the an AIoT Device and AF, as well as possibly being easily confused with the ADM “AIoT Data Management” NF. The command procedure for read/write transfers application data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Provide better separation between the network aspects and the AIoT Device interaction aspects for the AIoT Command service, and provide clarity on how Command type and Command type specific parameters are used. Alignment of the term AIoT Data to application data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect implementation due to lack of clarity between application data, AIoT Data, AIoT Device, AIoT NAS messages and network service operations, leading to incorrect results. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.2, 4.6.2.3, 5.2.2.1, 5.2.2.2, 5.2.2.3, 6.2.3, 7.3.2, 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.6.2.2	Protocol Stack between AF and AIoT Device for NG-RAN Direct Connectivity

Legend:

-	Application Data: It is the application data exchanged between the AIoT Device and the AF.

-	AIoT NAS: The NAS protocol between AIoT Device and the AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN as specified in TS 38.300 [5].

-	NGAP: Application Layer Protocol between the AIOTF and the NG-RAN as defined in TS 38.413 [10].

-	NGAP AIoT Information: It is the subset of NGAP information and is included in the NGAP messages over AIOT2 reference point.

Figure 4.6.2.2-1: Protocol Stack between AF and AIoT Device for Direct Connectivity option

The AIoT NAS messages between the AIoT Device and the AIOTF are transferred via the NG-RAN transparently.

# * * * * Second change * * * *

4.6.2.3	Protocol Stack between AF and AIoT Device for NG-RAN Indirect Connectivity

Legend:

-	Application Data: It is the application data exchanged between the AIoT Device and the AF.

-	AIoT NAS: The NAS protocol between AIoT Device and the AIOTF.

-	AIoT AS: It is between the AIoT Device and the NG-RAN as specified in TS 38.300 [5].

-	NGAP: Application Layer Protocol between the AMF and the NG-RAN as defined in TS 38.413 [10].

-	NGAP AIoT Information: It is the subset of NGAP information that the AMF transparently relays between the NG-RAN and the AIOTF, and is included in the NGAP messages and the AIOT3 messages.

Figure 4.6.2.3-1: Protocol Stack between AF and AIoT Device for Indirect Connectivity via AMF option

NOTE:	From the NG-RAN perspective, there is a single termination of N2 i.e. the AMF.

In the Indirect Connectivity option, the AMF is involved and its functionality is as defined in clause 4.5.7. The AIoT NAS messages between the AIoT Device and the AIOTF are transferred via the NG-RAN and AMF transparently.

# * * * * Third change * * * *

5.2.2	AIoT Command service

5.2.2.1	Overview

The Naiotf_AIoT_Command service operation supports 3 Command types: Read, Write and Disable.

The Naiotf_AIoT_Command with the Read Command type is used to request the AIOTF read application data from AIoT Device(s), and the AIOTF uses the AIoT NAS messages described in clause 5.2.2.2 towards the AIoT Device for the operation.

The Naiotf_AIoT_Command with the Write Command type is used to request the AIOTF to write application into AIoT Device(s), and the AIOTF uses the AIoT NAS messages described in clause 5.2.2.2 towards the AIoT Device for the operation.

The Naiotf_AIoT_Command with the Permanent Disable Command type is used to request the AIOTF permanently disables the capability of AIoT Device(s) to transmit RF signals, and the AIOTF uses the AIoT NAS messages described in clause 5.2.2.3 towards the AIoT Device for the operation.

5.2.2.2	Read and Write Commands

It is optional for an AIoT Device to support of Read and Write AIOT NAS messages. If supported, an AIoT Device contains User Memory that can be used to store application specific data and is accessed using AIoT NAS messages from an AIOTF. An AIoT Device implementation may have other storage outside of the User Memory that is used to store e.g. AIoT Device Permanent Identifier, etc.

The User Memory is accessed by the following AIoT NAS messages between the AIOTF and AIoT Device:

-	Read Request: used to read application data from User Memory.

-	Read Response: Response from the AIoT Device to a Read Request, providing a status and the application data read from User Memory.

-	Write Request: used to write application data to User Memory.

-	Write Response: Response from the AIoT Device to a Write Request, providing a status for the Write Request.

The Read Request AIoT NAS message is used by an AIOTF to read application data from the User Memory of AIoT Devices(s)when an authorized AF uses the Command type Read, as described in clause 5.2.2.1. The Write Request AIoT NAS message is used by an AIOTF to write application data into the User Memory of AIoT Device(s) when an authorized AF uses the Command type Write, as described in clause 5.2.2.1. The Read Request and Write Request NAS messages are sent in the Command Request step and the Read Response or Write Response AIoT NAS messages are sent in the Command Response step of the Command procedure described in clause 6.2.3.

The physical memory map of an AIoT Device and where the User Memory is within it is implementation specific.

The Commands type Read and Write operations and the AIoT NAS messages to access User Memory of AIoT Device(s) include an offset from the start of the User Memory to indicate where to read application data from or where to write application data to. The offsets and the application data transferred have no special meaning to the network, and the AIOTF or other NFs do not attempt to interpret them.

Figure 5.2.2.2-1: Logical AIoT Device User Memory

The Write Request AIoT NAS message is used to write application data into the User Memory of an AIoT Device and includes:

-	the application data to write and its length; and

-	the offset within User Memory to write the application data to.

The AIoT Device responds with a Write Response AIoT NAS message indicating whether the Write Request was successful.

The Read Request AIoT NAS message is used to read application data from the User Memory of an AIoT Device and includes:

-	the offset within User Memory to read the application data from; and

-	the length of the application data to read.

The AIoT Device responds with a Read Response AIoT NAS message including whether the read was successful and the application data read from the User Memory.

If an AIoT Device does not support Read Request or Write Request AIoT NAS messages, or the parameters in an AIoT NAS message are invalid the AIoT Device shall reject the AIoT NAS message.

5.2.2.3	Permanent Disable Command

An AIoT Device may be permanently disabled. A permanently disabled AIoT Device does not respond to the Inventory Procedure, as described in clause 6.2.2.

An AIoT Device is permanently disabled by the Permanent Disable AIoT NAS message sent by an AIOTF to the AIoT Device.

The Permanent Disable AIoT NAS message is sent to an AIoT Device when an authorized AF uses the Command type Permanent Disable as described in clause 5.2.2.1, or if the network determines to disable an AIoT Device. The Permanent Disable AIoT NAS message is sent in the Command Request step and a response is sent in the Command Response step of the Command procedure described in clause 6.2.3. The AIoT Device responds indicating whether the Permanent Disable AIoT NAS message was successful.

NOTE:	The trigger conditions when a network determines to disable the AIoT Device depends on operator and implementation policy.

When the AIoT Device has received and verified a Permanent Disable AIoT NAS message, it shall no longer respond to the inventory procedure.

# * * * * Forth change * * * *

6.2.3	Command Procedure

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, information about the target AIoT Device(s), [External Target Area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters]) message to NEF.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The External Target Area information is specified in clause 5.3.

The approximate number of AIoT Devices (see clause 5.4), if provided, is used to indicate the number of AIoT Devices expected to respond to this command service operation, which is sent by AIOTF to the NG-RAN in the assistance information as specified in clause 5.4.

Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, information about the target AIoT Device(s), [Target area information], [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters]) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF uses the Command type and Command type specific parameters to determine the AIoT Command service to perform, AIoT NAS messages and related parameters as described in clause 5.2.2.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step12 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 7, the AIOTF also includes follow on command indication in the Inventory Request message to inform the NG-RAN command delivery occurs after the inventory.

-	In step 10, the NG-RAN also includes the RAN AIoT Device NGAP ID for each AIoT Device in the Inventory Report as specified in TS 38.413 [10].

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information. The AIOTF updates the corresponding AIoT device context in the AIOTF to include the RAN AIoT Device NGAP ID.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size], RAN AIoT Device NGAP ID for each AIoT Device) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the application data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID for each AIoT Device is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10].

The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2.

NOTE:	Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device as defined in TS 38.391 [11].

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN as defined in TS 38.391 [11]. The NAS Command Response message may include the application data.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response, RAN AIoT Device NGAP ID) to the AIOTF directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The AIOTF determines the AIoT device context by the RAN AIoT Device NGAP ID received.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), application data), AF ID, [Last Report Indication]).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (a list of AIoT Device(s) response information (AIoT Device ID(s), application data), AF ID, [Last Report Indication]).

# * * * * Fifth change * * * *

7.3.2	Namf_AIoT_MessageDelivery service operation

Service operation name: Namf_AIoT_MessageDelivery

Description: The NF consumer requests to send information and request information from NG-RAN or AIoT Devices.

Inputs, Required:

1)	NGAP AIoT Information to deliver to NG-RAN.

2)	NG-RAN ID.

3)	AIoT NGAP Message Type ("Inventory" or "Command").

4)	AIOTF Identifier and Correlation Identifier, this is to allow identifying the association between NG-RAN and AMF.

Outputs, Required: Result indication (Success or Failure), Failure Cause in case of Failure.

# * * * * Sixth change * * * *

7.3.3	Namf_AIoT_Notify service operation

Service operation name: Namf_AIoT_Notify

Description: The NF consumer requests to receive information from NG-RAN or AIoT Devices. If the NF consumer invokes the Namf_AIoT_MessageDelivery, the NF consumer implicitly subscribes to receive the information from NG-RAN or AIoT devices.

Inputs, Required:

1)	NGAP Information received from NG-RAN.

Input, Optional: None.

Outputs, Required: Operation execution result indication.

Output, Optional: None.

# * * * * End of changes * * * *