3GPP TSG-WG SA2 Meeting #170	S2-2507269

# Goteborg, SE, 25th Aug – 29th Aug, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6409 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update UDR description for Ambient IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Release 19 introduced new feature Ambient IoT and specified the features in TS 23.369. Clause 5.5 in TS 23.369 specifies “AIoT Device Profile Management” and defines AIoT Device Profile which is stored in UDR. CT4 agreed the below principles and informed the SA2 in LS C4-252411: - Nudr_DR service is proposed to be used for accessing AIoT device profile data - a new TS will be created for the data modelling of AIoT device profile data In the LS, CT4 asks SA2 to confirm using Nudr_DR service for accessing AIoT device profile data, and update TS 23.501 and TS 23.502 accordingly if needed. It is proposed to update TS 23.501 to add the new UDR functionality due to the introduction of AmbientIoT data, according to CT4’s request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - update clause 4.2.5 “Data Storage architectures”, to add description for Ambient IoT data access by ADM - update clasue 6.2.11 “UDR”, to add new UDR functionality “storage and retrieval of Ambient IoT data by the ADM” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | CT4 request on UDR service for AIoT data profile is not considered by SA2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.5, 6.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.11	UDR

The Unified Data Repository (UDR) supports the following functionality:

-	Storage and retrieval of subscription data by the UDM.

-	Storage and retrieval of Ambient IoT data by the ADM.

-	Storage and retrieval of policy data by the PCF.

-	Storage and retrieval of structured data for exposure.

-	Application data (including Packet Flow Descriptions (PFDs) for application detection, AF request information for multiple UEs, 5G-VN group information for 5G-VN management, Non-3GPP Device Identifier Information).

-	Storage and retrieval of NF Group ID corresponding to subscriber identifier (e.g. IMPI, IMPU, SUPI).

The Unified Data Repository is located in the same PLMN as the NF service consumers storing in and retrieving data from it using Nudr. Nudr is an intra-PLMN interface.

NOTE 1:	Deployments can choose to collocate UDR with UDSF.

# * * * * Second change * * * *

4.2.5	Data Storage architectures

As depicted in Figure 4.2.5-1, the 5G System architecture allows any NF to create/read/update/delete its unstructured data in a UDSF (e.g. UE contexts). If such an NF is using UDSF is part of an NF set, then any of the NF instance within this NF set may read/update/delete the unstructured data that was created by this NF. The UDSF belongs to the same PLMN where the network function is located. CP NFs/NF Sets may share a UDSF for storing their respective unstructured data or may each have their own UDSF (e.g. a UDSF may be located close to the respective NF).

NOTE 1:	Structured data in this specification refers to data for which the structure is defined in 3GPP specifications. Unstructured data refers to data for which the structure is not defined in 3GPP specifications.

NOTE 2:	If a NF Set has its own UDSF, it is up to UDSF implementation and deployment that only the NF instance within the set can access the data created by another NF instance within the NF set. If a UDSF is shared between several NFs not part of the same set or is shared between several NF sets, it is up to UDSF implementation and deployment to make sure that only NFs that are authorized can access the data. For further information about Guidelines and Principles for Compute-Storage Separation see Annex C.

Figure 4.2.5-1: Data Storage Architecture for unstructured data from any NF

NOTE 3:	3GPP will specify (possibly by referencing) the N18/Nudsf interface.

As depicted in Figure 4.2.5-2, the 5G System architecture allows the UDM, PCF, NEF and ADM to store data in the UDR, including subscription data and policy data by UDM and PCF, structured data for exposure and application data (including Packet Flow Descriptions (PFDs) for application detection, AF request information for multiple UEs) by the NEF, Ambient IoT data by ADM. UDR can be deployed in each PLMN and it can serve different functions as follows:

-	UDR accessed by the NEF belongs to the same PLMN where the NEF is located.

-	UDR accessed by the UDM belongs to the same PLMN where the UDM is located if UDM supports a split architecture.

-	UDR accessed by the PCF belongs to the same PLMN where the PCF is located.

-	UDR accessed by the ADM belongs to the same PLMN where the ADM is located.

NOTE 4:	The UDR deployed in each PLMN can store application data for roaming subscribers.

Figure 4.2.5-2: Data Storage Architecture

NOTE 5:	There can be multiple UDRs deployed in the network, each of which can accommodate different data sets or subsets, (e.g. subscription data, subscription policy data, data for exposure, application data) and/or serve different sets of NFs. Deployments where a UDR serves a single NF and stores its data and, thus, can be integrated with this NF, can be possible.

NOTE 6: The internal structure of the UDR in figure 4.2.5-2 is shown for information only.

NOTE X:	Reference point AIOT7 is defined in clause 4.3, TS 23.369 [220].

The Nudr interface is defined for the network functions (i.e. NF Service Consumers), such as UDM, PCF, NEF and ADM, to access a particular set of the data stored and to read, update (including add, modify), delete and subscribe to notification of relevant data changes in the UDR.

Each NF Service Consumer accessing the UDR, via Nudr, shall be able to add, modify, update or delete only the data it is authorised to change. This authorisation shall be performed by the UDR on a per data set and NF service consumer basis and potentially on a per UE, subscription granularity.

The following data in the UDR sets exposed via Nudr to the respective NF service consumer and stored shall be standardized:

-	Subscription Data,

-	Policy Data,

-	Structured Data for exposure,

-	Application data: Packet Flow Descriptions (PFDs) for application detection and AF request information for multiple UEs, as defined in clause 5.6.7.

-	Ambient IoT data, as defined in clause 5.4.8 in TS 23.369 [220].

The service based Nudr interface defines the content and format/encoding of the 3GPP defined information elements exposed by the data sets.

In addition, it shall be possible to access operator specific data sets by the NF Service Consumers from the UDR as well as operator specific data for each data set.

NOTE 7:	The content and format/encoding of operator specific data and operator specific data sets are not subject to standardization.

NOTE 8:	The organization of the different data stored in the UDR is not to be standardized.

# * * * * End of changes * * * *