3GPP TSG-WG SA2 Meeting #170	S2-2507277

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Title:	[DRAFT] Reply LS on USS changeover procedure

Response to:	LS (S2-2506153/S3-252263) on USS changeover procedure

Release:	Release 19

Work Item:	UAS_Ph3 (SA2) / UAS_Ph3_Sec (SA3)

Source:	[Huawei to be] SA2

To:	SA3

Cc:	CT3

Contact Person:

Name:	Runze Zhou

Tel. Number:

E-mail Address:	zhourunze  AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	CR 0205 of TS 23.256

1. Overall Description:

Guidance – Include the document reference if this responds to an incoming LS.

Guidance – Inform clearly what is the oldest release for which this LS needs to be considered by the destination group(s)

Guidance – Inform clearly if the LS is only for information or ACTION [see 2. below]

SA2 thanks SA3 for sending the LS on USS changeover procedure.

Regarding the question in the LS:

“SA3 would like to ask clarification for USS changeover procedure during a UAV flight. TS 23.256 clause 5.13.2 step 20 states the following: 20. The serving USS (USS 1) informs the UAV via application layer (not defined in this specification) about the changeover to the new USS (USS 2). If required USS (USS 1) may instruct the UAV to execute the UUAA procedure with the new USS (USS 2) when initiated by the USS 2.”

[SA2 answer]

SA2 considers two scenarios when defining step 20 in clause 5.13.2, TS 23.256.

In both scenarios, the pre-condition is the USS1 informs the UAV about the changeover to the new USS (USS 2), including the USS2 address and the indication of USS changeover.

Scenario 1: After receiving the USS2 address and the indication of USS changeover, UAV may initiate the UUAA-SM procedure to the USS2. The use plane connectivity to communicate with the USS1 is assumed can be reused by the UAV to communicate with the USS2, so the UUAA-SM is executed by PDU session modification procedure. SA2 has clarified and updated the related procedure in the attached CR. This scenario is helpful for the UAV to communicate with the USS2 immediately, regardless whether the USS2 side initiate any operation (e.g. authentication) to the 5G network.

Scenario 2: (UAV does not initiate UUAA procedure autonomously). USS2 may initiate the re-authentication procedure as defined in clause 5.2.4.1 or clause 5.2.4.2, TS 23.256. The agreed CR S3-252314 from SA3 reflects this scenario. SA2 has clarified and updated the related procedure accordingly in SA2 TS 23.256 and attached in the LS reply.

From network point of view, if receiving the UUAA request from the UAV and the USS2 concurrently, the network should be able to combine the two requests into a single request.

2. Actions:

To SA3 group.

ACTION: 	SA2 asks SA3 to take into account the reply from SA2 for your future work.

Guidance – Provide clear information on what the other groups must do, and by when

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13th Oct – 17th Oct, 2025		Wuhan, CN

TSG-SA2 Meeting #172		17th Nov – 21st Nov, 2025		Dallas, US