3GPP TSG-SA WG2#170	S2-2507280

Gothenburg, Sweden, 25-29 August 2025	(was S2-250xxxx)

Source:	Qualcomm

Title:	[WT#6] Scope for work task on compute

Document for:	Discussion and approval

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses the scope of the potential work task on compute.

# 1.	Background

SA2’s 6G SID [1] contains the following description for WT#6:

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.). NOTE 7: Application layer mechanism and exposure framework may require coordination with SA6.

The remaining sections discuss the different aspects of compute support and the potential relevance of studying system support for compute in the context of 6G.

# 2.	Discussion

Compute support can be broadly broken down into the following aspects:

1)	Discovery and selection of compute instances with certain characteristics (e.g. details of available CPUs and GPUs, memory, storage and location of the compute instance).

2)	Provisioning of compute workloads.

3)	Optionally, configuration of network resources to enable efficient connectivity to the selected compute instance.

The last aspect (efficient connectivity to the selected compute instance) is access specific and has already been studied and addressed extensively in SA2 by features such as SSC modes, UL classifier, branching point functionality, Application Function influence on traffic routing, edge computing functionality etc. It is important to ensure that such functionality is also available for 6G and to study whether the existing functionality is sufficient for efficient connectivity to compute instances.

(1) and (2) are instead generally considered access agnostic problems for which various solutions are already available.

Public cloud providers offer APIs for discovery and selection of available compute instances, e.g. AWS Resource Explorer [1], Azure Resource Graph [2] or Google Cloud Compute Engine API [3].

For private clouds, various frameworks are available that enable discovery and selection of available compute instances, e.g. OpenStack [5].

To avoid platform lock-in, meta frameworks like Terraform [6] allow application developers to abstract away public and private cloud-specific APIs and follow an infrastructure-as-code (IaC) approach.

In the context of cellular networks, 3GPP SA6 has already standardized solutions for discovery and selection of edge application servers (EAS) with specific EAS Service KPIs like available compute, memory, storage, etc. (see Table 1).

Table1: EAS Service KPIs (source: TS 23.588 [7] clause 8.2.5)

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the AC supported by the server. |
| Maximum Response time (NOTE) | O | The maximum response time advertised for the AC's service requests. |
| Availability | O | Advertised percentage of time the server is available for the AC's use. |
| Available Compute | O | The maximum compute resource available for the AC. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the AC. |
| Available Memory | O | The maximum memory resource available for the AC. |
| Available Storage | O | The maximum storage resource available for the AC. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the AC's use. |
| NOTE: The maximum response time includes the round-trip time of the request and response packet for communication between the AC and EAS, the processing time at the server and the time required by the server to consume 3GPP Core Network capabilities, if any. |  |  |

In summary, discovery and selection of compute instances with certain characteristics are access agnostic challenges that can be considered solved by mechanisms outside the scope of SA2 specification work.

Observation 1: Discovery and selection of compute instances with certain characteristics are access agnostic challenges that can be considered solved by mechanisms outside the scope of SA2 specification work.

These existing solutions can be used for compute needs in the core network or the application server domain.

Observation 2: Compute needs in the core network or the application server domain can be addressed using existing solutions.

Also for potential compute needs for applications running on the UE it is not obvious why SA2 should study solutions for compute instance discovery and selection on 6G architecture level, i.e. below IP.

On the one hand solutions exist that can be reused for this purpose as illustrated above. On the other hand, it is not clear why any cellular-specific solution for compute instance discovery and selection would be attractive to application developers.

Application developers can be assumed to prefer their apps to be useable regardless of the access technology (e.g., WiFi or other cellular access). This also applies to the use of external compute resources that an application relies on. If compute resources can only be discovered and used by an application when the UE is using 6G cellular access only, then this would severely limit the addressable market for that application. It is not obvious why application developers would prefer to limit the addressable market for their apps.

Observation 3: Application developers can be assumed to prefer to maximize the addressable market for their apps by relying on access technology agnostic solutions for compute resource discovery and selection.

In summary, there is no obvious gap that needs to be addressed for compute discovery and selection by SA2. Also, there is no benefit of a cellular specific solution given that it would prevent app developers from maximizing their addressable market. In conclusion, it is not clear why SA2 should study architecture support for discovery and selection of compute resources and hence this paper suggests to not do so.

It is important to highlight that even though there is no need for standardization work in 3GPP SA2 on discovery and selection of compute resources, MNOs can offer compute services using mechanisms defined outside of 3GPP.

Conclusion: SA2 should not study support for discovery and selection of compute resources in the context of 6G given that there is no clear gap that needs to be addressed and since a cellular specific solution is not attractive to application developers who aim at maximizing their addressable market. SA2 should study whether the existing functionality for efficient traffic routing (SSC modes, UL classifier, branching point functionality, Application Function influence on traffic routing, related edge computing functionality, etc.) is sufficient for efficient connectivity to compute instances.

# 3.	References

[1]	TSG SA: Study on Architecture for 6G System. SP-250806.

[2]	AWS Resource Explorer. https://docs.aws.amazon.com/resource-explorer/latest/apireference/Welcome.html.

[3]	Azure Resource Graph. https://learn.microsoft.com/en-us/rest/api/azure-resourcegraph/

[4]	Google Cloud Compute Engine API. https://cloud.google.com/compute/docs/apis

[5]	OpenStack. https://www.openstack.org/

[6]	TerraForm. https://developer.hashicorp.com/terraform

[7]	TS 23.558: Architecture for enabling Edge Applications.

# 4.	Proposal

This paper proposes to apply the following changes to TR 23.801-01.

**** First Change ****

# Annex A.X. WT#6 Scope

WT#6 (Compute related): Study whether the existing functionality for efficient traffic routing (SSC modes (see TS 23.501 clause 5.6.9), UL classifier (see TS 23.501 clause 5.6.4.2), branching point functionality (see TS 23.501 clause 5.6.4.3), Application Function influence on traffic routing (TS 23.501 clause 5.6.7), related edge computing functionality (see TS 23.548), etc.) is sufficient for efficient connectivity to compute instances or identify required enhancements.

**** End of Changes ****