3GPP TSG-WG SA2 Meeting #170	S2-2507286

# Goteborg, SE, 25th Aug – 29th Aug, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3935 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Avoid transfer of user plane PDN connection to S&F Mode network with split MME |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1) Data transfer over UP is not supported in split MME case as stated in 23.401: NOTE 3: Some deployments cannot support all system features, for example user plane establishment and user plane data transfer cannot be supported, when a satellite is operating in S&F Mode with the split MME deployment described in Annex O. The UE may be on a terrestrial network (TN) and then moves to a network which is operating in S&F mode. However, the UE does not know if the S&F is deployed with split MME functionality. A PDN connection with UP data transfer from the TN cannot be moved to the NTN S&F network. Such scenario is not addressed in the specification. 2) Incorrect term “S&F mode” exists in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) When the target MME in the S&F network fetches the context from the source MME, the target MME indicates whether or not is supports S1-U data transfer. Based on this, the source MME does not include any PDN connection with S1-U data transfer in the transferred context. 2) Change “S&F mode” to “S&F Mode”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) A PDN connection which cannot be supported in the S&F network with split functionality is included in the UE context for which there is currently no recovery mechanism. 2) Terminology misalignment across specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 4.13.9.2	Transition between S&F Mode and non S&F Mode

If a UE is attached to a PLMN using satellite operating in S&F Mode, the UE follows the existing AS and NAS procedures to access a satellite that is not operating in S&F Mode (either TN or NTN eNodeB) and vice versa.

In order to ensure smooth idle mode transition from S&F Mode to non S&F Mode, if the old (S&F capable) MME in inter MME mobility has the knowledge that the UE context is not up to date (e.g. based on implementation) then the old (S&F capable) MME rejects the MM context transfer.

After the UE transitions to a network in S&F Mode and the target MME attempts to fetch the UE context from the source MME, the target MME should indicate to the source MME whether or not it supports user plane data transfer. If the target MME does not support user plane data transfer and there exists any PDN connection with user plane data transfer in the UE context, the source MME removes the PDN connection with user plane data transfer from the UE context before forwarding the context to the target MME.

If a satellite has no feeder link connectivity, it can only operate in S&F Mode. When a satellite supporting S&F Mode obtains a feeder link, then, the satellite may cease to operate in S&F Mode depending on operator policy and regulatory requirement.

# * * * * End of changes * * * *