3GPP TSG-WG SA2 Meeting #170	S2-2507287

Göteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[KI2, New Sol] SIP timer adjustment based on UE’s RAT type

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: A solution to support the UE and IMS element to adjust the SIP timers based on UE’s RAT type. During an IMS voice call, if either the RAT type of the MO or MT UE is NB-IoT (GEO), the UEs and IMS elements may apply the SIP timers dedicated to GEO access.

# 1. Introduction/Discussion

TS 24.229 clause 7.7 specifies a set of timers applied to transfer SIP messages. For example, the estimated RTT of a SIP message (indicated by T1) at the UE is set to 2s by default and this timer is used to initialise the timeout of an INVITE (Timer B) and of non-invite transactions (Timer F).

Table 7.7.1: SIP timers

| SIP Timer | Value to be applied between IM CN subsystem elements | Value to be applied at the UE | Value to be applied at the P-CSCF toward a UE | Meaning |
| --- | --- | --- | --- | --- |
| T1 | 500ms default (see NOTE) | 2s default | 2s default | RTT estimate |
| T2 | 4s (see NOTE) | 16s | 16s | The maximum retransmit interval for non-INVITE requests and INVITE responses |
| T4 | 5s (see NOTE) | 17s | 17s | Maximum duration a message will remain in the network |
| Timer A | initially T1 | initially T1 | initially T1 | INVITE request retransmit interval, for UDP only |
| Timer B | 64*T1 | 64*T1 | 64*T1 | INVITE transaction timeout timer |
| Timer C | > 3min | > 3 min | > 3 min | proxy INVITE transaction timeout |
| Timer D | > 32s for UDP | >128s | >128s | Wait time for response retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer E | initially T1 | initially T1 | initially T1 | non-INVITE request retransmit interval, UDP only |
| Timer F | 64*T1 | 64*T1 | 64*T1 | non-INVITE transaction timeout timer |
| Timer G | initially T1 | initially T1 | initially T1 | INVITE response retransmit interval |
| Timer H | 64*T1 | 64*T1 | 64*T1 | Wait time for ACK receipt. |
| Timer I | T4 for UDP | T4 for UDP | T4 for UDP | Wait time for ACK retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer J | 64*T1 for UDP | 64*T1 for UDP | 64*T1 for UDP | Wait time for non-INVITE request retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer K | T4 for UDP | T4 for UDP | T4 for UDP | Wait time for response retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer L | 64*T1 | 64*T1 | 64*T1 | Wait time for accepted INVITE request retransmits |
| Timer M | 64*T1 | 64*T1 | 64*T1 | Wait time for retransmission of 2xx to INVITE or additional 2xx from other branches of a forked INVITE |
| Timer N | 64*T1 | 64*T1 | 64*T1 | Wait time for receipt of a NOTIFY request upon sending SUBSCRIBE |
| NOTE: As a network option, SIP T1 Timer's value can be extended, along with the necessary modifications of T2 and T4 Timers' values, to take into account the specificities of the supported services when the MRFC and the controlling AS are under the control of the same operator and the controlling AS knows, based on local configuration, that the MRFC implements a longer value of SIP T1 Timer. |  |  |  |  |

In typical terrestrial scenarios (e.g., LTE, NR), these default values are sufficiently long given the low latency and stable connectivity of terrestrial networks. However, considering a NB-IoT accesses via GEO satellite, the RTT may be much longer than the default T1 due to the propagation delay and low transmission rate. Therefore, the SIP timer default values for a SIP transaction involving a GEO leg are to be set appropriately to avoid undesired timeouts. Besides, using appropriate timers in satellite scenarios helps avoid frequent and unnecessary signalling retransmissions.

TS24.229 allows the UE and P-CSCF to modify the SIP timer values to accommodate air interface delays. However, recommended SIP timer values dedicated to GEO access shall be specified.

It is proposed that SIP timers are configured taking into account the RAT type used by the UE.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

# * * * * Second change (All new text) * * * *

## 6.X	Solution X: Optimization for NB-IoT GEO IMS call set up

### 6.X.0	High level solution Principles

The high level principles of the solution are:

-	Introduce a dedicated set of SIP timers when the UE is accessing the network via GEO satellite.

-	During an IMS voice call, any UE that is connected via GEO satellite as well as the IMS elements serving that UE apply the set of SIP timers dedicated for GEO access for the session-related SIP messages. The SIP timers for the SIP messages not related to the session remain unchanged.

-	As an option, if during an IMS voice call only one of the UEs is connected via GEO satellite, the UE not connected via GEO and the IMS elements serving that UE may also apply the set of SIP timers dedicated for GEO access to the session-related SIP messages.

NOTE 1: Session-related SIP messages refer to the SIP messages transmitted between UEs during a IMS session, such as INVITE, 183 Session Progress, UPDATE, etc.

NOTE 2: Session-unrelated SIP messages refer to the SIP messages transmitted between the UE and its IMS core elements, such as Registration Request, OPTIONS, SUSCRIBE, etc.

### 6.X.1	Description

This solution addresses KI#2 concerning IMS enhancement for GEO NB-IoT NTN access.

In addition to the SIP timers defined in TS 24.229 [16], a set of SIP timers dedicated for GEO access shall be introduced to accommodate the long RTT.

When a UE registers to the IMS network, it includes the P-Access-Network-Info header field in the SIP Registration Request, which contains the UE RAT type information. A third-party registration procedure is also necessary to inform the AS of the UE RAT type. The UE and the IMS shall configure the appropriate set of SIP timers based on the UE RAT type.

To minimize the risk of a session timer timeout, any UE accessing via GEO and the IMS elements serving the UE apply a set of SIP timers optimised for GEO access, i.e. that take into account the larger propagation delay and the possibility of messages being segmented.

The dedicated SIP timers are only used for the session-related SIP messages while default timer values may be used for session-unrelated SIP messages.

As an option, if during an IMS voice call only one of the UEs is connected via GEO satellite, the UE not connected via GEO and the IMS elements serving that UE may also apply the set of SIP timers dedicated for GEO access to the session-related SIP messages. This will avoid risks of frequent re-transmissions of messages. To activate this functionality the MO and MT UEs expose the RAT type in use to each other.

### 6.X.2	Procedures

.

### 6.X.2.1 Procedures for SIP timer adjustment based on UE’s RAT Type

Figure 6.X.2-1 provides an example procedure for the UE and IMS elements to adjust their SIP timers based on the UE’s RAT type.

UE-A

P-CSCF-A

AS-A

1. Registration Request

I/S-CSCF-A

4. 200 OK

3. Registration Request

2. 200 OK

Start SIP timers based on UE-A's RAT type

Figure 6.X.2-1: Session setup procedures for the MO and MT UE negotiating SIP timers based on RAT type

1-2. The UE sends a SIP Registration Request to IMS network (P-CSCF, I-CSCF and S-CSCF), including a P-Access-Network-Info header field which consists of the UE’s RAT type. The UE and the IMS core shall start the SIP timers based on the UE’s RAT type. Then the IMS core responds with a 200 OK.

3-4. The S-CSCF initiates the third-party registration to AS (typically MMTel AS and SCC AS), to inform the AS of the UE’s RAT type. After receiving the Registration Request, the AS starts the SIP timers based on UE’s RAT type. Then the AS responds with a 200 OK.

### 6.X.2.2 Procedures for the MO and MT UE Negotiating SIP Timers based on RAT Type

Figure 6.X.2-2 provides an example call setup procedure when the caller and/or called UE accesses through satellite. If either the UE-A or UE-B accesses via GEO satellite, the UEs and the IMS elements may both initiate the set of SIP timers dedicated for GEO access.

UE-A

P-CSCF-A

AS-A

UE-B

1. Registration Request

1. Registration Request

I/S-CSCF-A

4. 200 OK

3. Registration Request

2. 200 OK

AS-B

I/S-CSCF-B

P-CSCF-A

2. 200 OK

3. Registration Request

4. 200 OK

5. INVITE

6. 183 Session Progress

Start SIP timers based on UE-A's RAT type

Start SIP timers based on UE-B's RAT type

Start SIP timers based on UE-A and UE-B's RAT type

7. Remaining session setup procedures

5. INVITE

5. INVITE

5. INVITE

6. 183 Session Progress

6. 183 Session Progress

6. 183 Session Progress

6. 183 Session Progress

5. INVITE

Figure 6.X.2-2: procedures for the MO and MT UE negotiating SIP timers based on RAT type

1-2. The UE sends a SIP Registration Request to IMS network (P-CSCF, I-CSCF and S-CSCF), including a P-Access-Network-Info header field which consists of the UE’s RAT type. The UE and the IMS core shall start the SIP timers based on the UE’s RAT type. Then the IMS core responds with a 200 OK.

3-4. The S-CSCF initiates the third-party registration to AS (typically MMTel AS and SCC AS), to inform the AS of the UE’s RAT type. After receiving the Registration Request, the AS starts the SIP timers based on UE’s RAT type. Then the AS responds with a 200 OK.

5.	The UE-A initiates the IMS voice call with a SIP INVITE message, including a P-Access-Network-Info header field which consists of the UE-A’s RAT type.

6.	The UE-B response with a SIP 183 Session Progress message to UE-A, including a P-Access-Network-Info header field which consists of the UE-B’s RAT type. If either the UE-A or UE-B accesses via GEO, the UEs and IMS elements may apply the set of timers dedicated for GEO access.

7.	UE-A and UE-B continue the remaining procedures for IMS call setup, based on the SIP timers determined in step 6.

NOTE 1:	The SIP timers unrelated to the session are not affected. For example, the SIP timers for Registration Request, OPTIONS, SUBSCRIBE remain unchanged.

NOTE 2: The value for the set of SIP timers dedicated for GEO access is FFS.

### 6.X.3	Impacts on Services, Entities and Interfaces

UE:

-	Support the set of SIP timers dedicated for GEO access.

-	If either the MO or MT UE accesses via GEO, may apply the set of SIP timers dedicated for GEO access for session related SIP messages.

IMS:

-	Support the set of SIP timers dedicated for GEO access.

-	If either the MO or MT UE accesses via GEO, may apply the set of SIP timers dedicated for GEO access for session related SIP messages.

-	Can send the RAT type used by the UE to the other UE in the session

# * * * * End of changes * * * *