3GPP TSG-WG SA2 Meeting #170	S2-2507291

Göteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[KI2, New Sol] Alert caller user the call setup time will be longer when called UE accessing via GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract: A solution to support IMS system to play a local media to the caller user before ringing when the called UE is accessed via GEO.

# 1. Introduction/Discussion

In Rel-20 FS_5GSAT_Ph4-ARC study item, it is proposed to study IMS enhancement for GEO NB-IoT NTN access as shown below:

Key Issue #2: IMS enhancement for GEO NB-IoT NTN access

To meet the requirements on experienced data rate and call setup time documented in TS 22.261 [8], this key issue studies:

-	Whether and how to enhance IMS to utilize NB-IoT as IP-CAN;

-	Potential IMS optimization.

Packets transferred via GEO NB-IoT NTN access experience a long transmit delay, which result in an extremely long call setup time for IMS voice call (about 30s). This will bring high risk that caller user may hang up the call before call setup procedure complete due to no ringing response.

Hence, it is necessary for caller UE to get an indication that the call setup time will be longer to let caller user be patient for the unusual user experience.

A new mechanism needs to be introduced to solve the above issue and enhance user experience.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-19.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  | X |  |  |

# * * * * Second change (All new text) * * * *

## 6.X	Solution X: Optimization for NB-IoT GEO IMS call set up

### 6.X.0	High level solution Principles

The high level principles of the solution are:

-	When called IMS system knows the RAT type of the called UE is NB-IoT (GEO), it includes an indication for call setup time alert in the 183 message, and the caller UE informs caller user about the longer call setup time with a local media.

### 6.X.1	Description

This solution address KI#2 about IMS enhancement for GEO NB-IoT NTN access.

This solution enhances the IMS serving called UE accessed via GEO NB-IoT NTN to indicate caller UE starts a local media before ringing.

It is proposed to support following mechanisms:

1.	During call setup procedure, the IMS system includes an indication for call setup time alert if called UE is accessed via GEO NB-IoT NTN access.

2.	The caller UE starts a local media if the indication for call setup time alert is included in the 183 message.

### 6.X.2	Procedures

Figure 6.X.2-1 provides an example call setup procedure when the called UE is access through satellite. If the RAT type of the called UE is NB-IoT(GEO), an indication for call setup time alert is included in 183 message and the caller UE alerts caller user about the longer call setup time with a local media.

5a. start resource reservation for UE-B

1. INVITE (Initial SDP offer)

PCRF-A

3. AAR message

PCRF-B

UE-B

CSCF-A

CSCF-B

2. 183 (PANI)

4. AAR response

6. AAR message

8. resource reservation for UE-A

7. AAA message

5b. 183 (indication for call setup time alert)

UE-A

9. 183 (indication for call setup time alert)

11. PRACK

12. 200 OK (PRACK)

10. start alert user with longer call setup time

14. resource reservation complete for UE-B

18. user answer

19. 200 OK (INVITE)

21. ACK

22. session media transfer

16. 180 RINGING

17. stop local media and start ringing

15. alert user

20b. start voice media

20a. stop local media/ringing and start voice media

13. UPDATE

Figure 6.X.2-1: high-level procedure for alert caller UE with longer call setup time when called UE accessed via GEO

1.	The caller UE A sends SIP INVITE message to the called UE B.

2-4.	The called UE B sends SIP 183 message with P-Access-Network-Info header field. The P-CSCF sends AAR message to PCRF for resource reservation.

5.	If the RAT type of called UE-B is NB-IoT (GEO), it includes an indication for call setup time alert in the 183 message. The CSCF-B forwards 183 message to CSCF-A and the PCRF-B starts resource reservation.

6-8.	When receiving the 183 message, the CSCF-A of caller UE A indicates PCRF-A to start resource reservation for caller UE A.

9-10.	If the caller UE A receives an indication for call setup time alert in the 183 message, it alerts caller user with a local configured media which informs the user of the longer call setup time.

NOTE X: The caller UE A could local configured/stored with a media indicating user the longer call setup time. For example, “The call setup may take longer time than usual, please wait patiently.” This media could be configured to the UE during IMS registration procedure, via application layer or operating system, etc.

11-15.	Existing PRACK and UPDATE interaction between caller UE A and called UE B. After resource reservation for called UE-B is complete, it alerts the called user.

16-17.	Optional, the called UE-B can also send 180 RINGING message to caller UE-A, and caller UE-A stops local media and starts ringing (e.g., let caller user knows the called user starts alerting).

18-21.	When the called user answers the call, the called UE starts voice media and sends 200 OK(INVITE) to caller UE A. When receiving the 200 OK(INVITE) message, caller UE A stops local media or ringing, responds with ACK, and starts voice media.

22.	Media transfer between the caller UE and called UE.

NOTE Y: This procedure can be combined with other solutions for Key Issue #2. For example, if UE-B sends 180 message directly after receiving SIP INVITE, the indication for call setup time alert and/or RAT type of UE-B are included in 180 message and the 180 message triggers local media of caller UE.

### 6.X.4	Impacts on Services, Entities and Interfaces

IMS:

-	For IMS of called UE, if the RAT type of called UE is NB-IoT(GEO), it includes an indication for call setup time alert in the 183 message.

UE:

-	For caller UE, if an indication for call setup time alert is included in the 183 message, it alerts caller user with a local configured media indicating the longer call setup time.

# * * * * End of changes * * * *