3GPP TSG SA WG2 Meeting #170	S2-2507302

25 – 29 Aug, 2025, Goteborg , SE

# Source:	Huawei, HiSilicon

# Title:	New WID on In-band Network Information Exposure via Payload Header

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	In-band Network Information Exposure via Payload Header

Acronym:	TEI20_IBEX

Unique identifier:	TBD

Potential target Release:	Rel-20

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  | x |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:  None

3          Justification

Since Rel-18, in order to assist the application layer for better awareness of network status, lots of mechanisms have been introduced, such as the exposure of packet latency, available bit rate, congestion information, etc. Especially for the cloud XR, cloud gaming services, the real-time awareness of the network status is essential for corresponding application layer adjustment, e.g., media rate adaptation. The APIs supported by the UPF/NEF/PCF can expose the above network status information to the Application Function.

However, it is also beneficial to have a user plane based network information exposure to the applications running on the UE side or the Application Server side to achieve an even faster real-time awareness of the network status.

In Rel-19, the Handling of Payload Headers feature (UPEAS) enables the exchange of information in-band in the user plane packets between the application (in the UE or in the application server) and the UPF in the 5GS by supporting detection, insertion, replacement, or removal of payload headers. The Handling of Payload Headers feature is requested by the AF.

Therefore, the Handling of Payload Headers feature (UPEAS) needs to be enhanced to expose the network status information (i.e. existing QoS monitoring) via an in-band and user plane way. Then the application running on the UE side or the Application Server can get aware of the most accurate network status in real-time and behave accordingly.

4	Objective

The following aspects will be specified:

WT 1: Add support of the in-band network information exposure (i.e. related to existing QoS monitoring) via payload headers.

This work item will require 0.5 TU(s) to discuss and agree on the corresponding CR(s).

TU estimates and dependencies:

| Work Tas ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT 1 |  | 0.5 | No | None |

5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Introduce In-band network information exposure via the payload header handling. | SA#110 (Dec 2025) |  |
| TS 23.503 | Introduce In-band network information exposure via the payload header handling. | SA#110 (Dec 2025) |  |

This TEI20_WID requires 0.5 TU.

6	Work item Rapporteur(s)

mirko.schramm@huawei.com

7	Work item leadership

SA WG2

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

Huawei

HiSilicon

CMCC

Xiaomi

Tencent