3GPP TSG-WG SA2 Meeting #170	S2-2507308

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#7, NTN] 6G support of NTN

Document for:	Approval

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract: This contribution proposes some scenarios and key issues for NTN in 6G.

# 1.	Introduction

For most procedures, the NTN-capable RAT behaves the same as a terrestrial RAT: the UE and network performs access, mobility and session management related procedures, and aspects like UE location reporting, discontinuous coverage, RAN node mobility can be coordinated and aligned with RAN as normal, e.g., based on fundamental procedures designed for both 6G NTN and TN network.

However, the NTN scenario has some other specific aspects relying on architecture/system/framework level study, like user performance degradation issues caused by unstable/dynamic satellite (constellation) backhaul.

# 2.	Justifications

## 2.1	General

The 6G SID in SP-2506096 contains the following WT for NTN:

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects. NOTE 8: The detailed scope for WT#7 will be coordinated and aligned with RAN

This discussion on the justification on WT#7 looks at architectural motivations for included aspects under this WT, along with gaps there are with other WTs and 5G and the technical impacts from that.

## 2.2	Architectural Motivation

The existing work in 5GS to support satellite operation has fallen into 4 broad categories for work:

1.	General support for UE access, including RAT type determination, UE location verification, etc, as described in clause 5.4.10 and 5.4.11.

2. 	Understanding the backhaul/ISL links between RAN and 5GC, as described in clause 5.43 and 5.4.14 of TS 23.501, and mitigations and understanding of that, for example locating UPF/computing services/IMS services on a satellite or measuring QoS of the backhaul.

3. 	Supporting a UE accessing in discontinuous coverage from MEO/LEO satellites, as described in clause 5.4.13 for TS 23.501.

4. 	gNB mobility, i.e., supporting gNBs disconnecting and reconnecting, as described in clause 5.4.11.9 of TS 23.501.

Several of these categories should be taken into account by the work generally in 6G, for example, understanding the RAT type and location awareness of where a UE is when it is accessing.

## 2.3	Gap Analysis

### 2.3.1	Support for UE Access via NTN

How to support 6G RAT for NTN still relies on RAN decision, but support for UE access via NTN is a basic concept in terms of overall architecture. It is believed nothing specific for NTN RAT needs to be studied in SA2, and it can be supported in general with an overall consideration on both TN and NTN, as 6G is expected to embrace NTN access from the start.

Proposal 1: For 6G NTN, support of UE access to NTN following the same principle with TN, and there is no need to study NTN RAT at the architecture level.

### 2.3.2	Understanding backhaul/ISL

During the satellite studies in 5G, the satellite backhaul connecting RAN and CN is via feeder links and inter-satellite links in two cases: (1) with RAN and CN located on the ground (also known as the reflective architecture), and (2) with RAN on board and CN on the ground or possibly on other satellites (also known as the regenerative architecture).

With RAN on board communicating with CN nodes on the ground then the backhaul from RAN towards ground will use feeder links, and with RAN on board communicating and CN nodes on other satellites then inter-satellite links will be used.

Satellite backhaul via feeder links and/or inter-satellite links has the following characteristics:

-	The Feeder links can switch quickly due to fast movement of the satellites.

-	The inter-satellite links are significantly less stable when compared with e.g., fibre links, especially for cross-orbit inter-satellite links, due to harsh environment and relative movement of satellites.

The above characteristics may cause problems including:

-	High jitter, high packet loss rate, and packet disordering, which in turn causes the drop of TCP throughput. According to “LEO Satellite vs. Cellular Networks: Exploring the Potential for Synergistic Integration”, TCP severely suffers from such a high packet loss of Starlink, leading to only 1/5 of the throughput achieved by UDP over Starlink.

-	Additionally, it leads to long packet delay due to retransmission and slow start of TCP.

The assumption of high reliability in terrestrial transport networks does not apply in the NTN scenario. NTNs need to address challenges such as higher packet loss, higher jitter, and higher packet reordering to prevent degradation of user experience and maintain system performance.

Proposal 2: Study and design mechanism of the user connection between UE and CN to avoid degradation of user experience due to unstable feeder link and/or ISL and maintain system performance.

### 2.3.3	Supporting Discontinuous Access

In NTN, discontinuous access is caused by the satellite deployment. In the early stage of satellite constellation deployment, the number of satellites can be relatively small. TNs also have scenarios of discontinuous access, e.g., for IoT devices, such as smart meters, sensors, which don’t require frequent communication. The only difference between NTN and TN for discontinuous access is the underlaying reasons for the discontinuous access – i.e., the result is same, only the cause is different.

For 6G NTN, there is nothing special or additional to be considered when compared with TNs at the system and architecture level.

Proposal 3: Discontinuous access can be considered and follow general principles designed both for TN and NTN. There is no need to study NTN RAT per architecture level now.

### 2.3.4	RAN Node Mobility

Both VMR and NTN cases involve RAN node mobility. For regenerative satellite payload with RAN on board, the RAN moves frequently. The moving speed of a satellite is much faster than VMR. This may lead to specific considerations for NTN access design, especially to support both NTN and TN RAT types in 6G. This also relies on RAN. From the SA2 perspective, the system level impacts introduced by RAN mobility can be considered along with the progress of TN study, e.g., access, mobility, session management and NAS related study.

Proposal 4: For 6G NTN, study RAN node mobility for regenerative payload along with the progress of TN study.

## 2.4	Technical Impact

The following proposals capture the impacts that this KI has on the overall system architecture and interactions between network functions/services:

Proposal 1: For 6G NTN, support of UE access to NTN following the same principle with TN, and there is no need to study NTN RAT at the architecture level.

Proposal 2: Study and design mechanism of the user connection between UE and CN to avoid degradation of user experience due to unstable feeder link and/or ISL and maintain system performance.

Proposal 3: Discontinuous access can be considered and follow general principles designed both for TN and NTN. There is no need to study NTN RAT per architecture level now.

Proposal 4: For 6G NTN, study RAN node mobility for regenerative payload along with the progress of TN study.

# 3. Text Proposal

It is proposed to capture the following changes.

# * * * * First change (all new) * * * *

# Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

-	Mitigate the effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

# * * * * Second change (all new) * * * *

# 5.X. Key Issue #X: Support of NTN in 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

For the support of NTN, the following aspects are studied:

-	Study how to mitigate the effects from the use of dynamic satellite backhaul and/or feeder links on the user plane traffic between a UE and the CN and maintain system performance.

# * * * * End of changes * * * *