3GPP TSG-WG SA2 Meeting #170	S2-2507312

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	KI#1, New Sol: Support of Energy Consumption calculation for redundant transmission

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2/ Rel-20

Abstract: Aims at enhancing calculation of energy consumption for a PDU Session and/or Service Data Flow when redundant transmission is applied.

1. Introduction/Discussion

1.1 Introduction

This document addresses KI#1: Whether and how to enhance the collection, determination and exposure of energy related information in the 5GS.

The solution is to enhance Energy Consumption calculation for a PDU Session and/or Service Data Flow with redundant transmission.

1.2 Discussion

In Release 19, a new network function EIF is introduced for Energy Consumption (EC) calculation for UE at different granularities (i.e., UE, PDU Session and/or service data flow). Take UPF as an example, EIF generates the energy consumed at UPF(s) for a PDU session by multiplying the data volume consumed at the UPF by the PDU session (i.e., DVSession,UPF) by the average EC per bit at the UPF (i.e., EUPF/DVUPF) as per the formula below, whereby the data volume consumed at the UPF by the PDU session is retrieved from the UPF via SMF using N4 reporting procedure:

ESession, UPF=EUPFDVSession,UPFDVUPF

When it comes to PDU sessions with redundant transmission, the EC evaluation is not correct since the UPF will not count the redundant packets on the N3/N9 interfaces in Usage Reports (e.g. Volume Measurement) in N4 reporting for such PDU sessions as per clause 5.2.2.3.3 in TS 29.244 reported below:

“The following requirements shall apply when using Redundant Transmission on N3/N9 interfaces or Redundant Transmission at transport layer:

-	the UPF shall not count redundant packets in Usage Reports (e.g. Volume Measurement), i.e. it shall count the traffic only once in Usage Reports;

-	the SMF shall provide the quota for the non-redundant transmission (i.e. not counting redundant traffic).”

Figure 1 illustrates an example of redundant transmission on N9 interface, assume PSA UPF receives 5Mbits data from DN for PDU session 1 and the traffic is duplicated and transmitted over the two N9 tunnels separately i.e., 10Mbits data volume is consumed at the PSA UPF. When the UPF reports data volume to EIF via SMF using N4 reporting procedure, only 5Mbits are counted, since these are the “valid” packets for accounting purposes. Therefore for evaluating the Energy Consumption the “redundant” traffic impact the Energy Consumption significantly, that if not considered in DV will lead to major deviation to the result the EIF generates and so as to account for inappropriate policy decision based on it.

Figure 1. Illustration of Redundant transmission on N9 interface

The document provides a solution addressing the calculation error of Energy Consumption for PDU Session and/or Service Data Flow with Redundant Transmission.

The concept is the SMF includes the duplicated data volume for PSA UPF in case of redundant transmission and provides it to EIF. The EIF calculates UE related Energy Consumption information by taking the redundant traffic into account.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-67.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| X | X |  |  |

# * * * * Second change * * * *

Solution #X: Energy Consumption calculation enhancement considering redundant transmission

6.X.0 High-level solution principles

-  SMF includes the redundant traffic for PSA UPF in the provided data volume to EIF when redundant transmission is deployed for a PDU session or a Service Data Flow.

-  EIF generates data volume consumed by the required granularities based on reported data volume from SMF when enforcing the Energy Consumption calculation.

6.X.1 Description

This solution is for KI#1: Whether and how to enhance the collection, determination and exposure of energy related information in the 5GS.

The document provides a solution addressing the calculation error of Energy Consumption for Redundant Transmission when the traffic on N3/N9 interfaces is duplicated.

The concept is EIF calculates UE related Energy Consumption information considering the Redundant related data volume provided by SMF.

6.X.2 Procedure

Existing procedure for Energy Consumption information collection as defined in clause 4.29.2 in TS 23.502 is reused. The solution is proposed with the following principles:

If redundant transmission is configured by the SMF at the PSA UPF for a PDU session and/or Service Data Flow (e.g., on N3/N9 interface or at transport layer) the SMF shall also include the redundant traffic in the data volume of the PSA UPF to be reported to the EIF based on operator configured logic e.g., the SMF may double the collected data volume from the PSA UPF. Specially, the SMF may use an NF specific Data Volume to represent the PSA UPF data volume along with the PSA UPF ID.

Upon receiving NF specific Data Volume from the SMF, the EIF shall use the NF specific Data Volume for the indicated PSA UPF while use the data volume reported from PSA UPF for the other Network functions for Energy Consumption calculation.

Based on the data volume provided by the SMF and other input parameters from OAM defined in clause 5.51.2.2.3 of TS 23.501, the EIF performs calculation and gets results of the Energy Consumption information as described in clause 5.51.2 of TS 23.501.

6.X.3 Impacts on existing services, entities and interfaces

The solution has the following impacts:

SMF

- Generate NF specific data volume for PSA UPF which includes the duplicated data volume in case of redundant transmission.

- Provide the NF specific data volume to EIF.

EIF

- Calculate Energy Consumption information for PDU Session and/or Service Data Flow based on reported NF specific data volume from SMF.

# * * * * End of changes * * * *