3GPP TSG-WG SA2 Meeting #170	S2-2507316

25th – 29th August, 2025, Gothenburg, Sweden	(revision of S2-250XXXX)

Source:	Lenovo

Title:	[WT#1.2, SBA] WT and KI on enhancements of the SBA

Document for:	Discussion and Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to introduce a key issue on enhancements of the SBA.

# 1. Justifications

## 1.1 Architecture motivation and gap analysis

The SBA was introduced in 5G to replace the point-to-point protocols (Diameter, Radius, GTP-C) in the core network between the network functions (NFs) in the control plane. After the initial Rel-15 specifications, the SBI was also introduced to UPF.

The 5G RAN (but also N3IWF, TNGF or W-AGF) use NGAP protocol to connect with the AMF. With the advancements of the RAN deployment using cloud-native technologies, it may be beneficial to introduce the SBI on the N2 interface. The following benefits can be considered:

-	Usually the N2 association between the RAN and AMF is on per UE basis. It would be advantageous to allow a NF in the CN control plane and RAN to exchange non-UE related data/signalling.

-	The UE related signalling can be exchanged between the RAN and CP NFs directly without involving the AMF.

Furthermore, the shift to 6G would allow to re-consider the design or some options of the 5G SBA while the basic SBA functionalities can be re-used. It can be studied, whether simplification of the NF discovery procedure via NRF can be introduced.

The potential use of new SBI protocols like QUIC/HTTP3 instead of HTTP2 can be studied by the CT WGs (e.g. CT4) and therefore the protocol aspects are left out in the SA2 study.

## Technical impact

The following aspects may be considered impacted:

-	The introduction of SBI-based framework on the N2 interface would impact the RAN, but also N3IWF/TNGF/W-AGF. The RAN-specific service operations and related profile needs to be studied.

-	Simplification of the NF discovery procedure (e.g. minimize the set of profile parameters stored in the NRF) can be studied.

# 2. Proposal

It is proposed to include the following text in the TR 23.801-01.

**** First Change ****

# Annex A.X. WT#1.2 on enhancements of SBA

The technical scope for WT#1.3 on SBA include the following aspects:

-	Study whether and how to introduce SBI-based framework on the N2 interface.

-	Study which RAN-specific services can be exposed to the CN and which operations can be supported.

-	Study whether and now to support UE based signalling between the RAN and the NFs in the CN control plane.

-	Study whether and how to simplify the NF discovery procedure via NRF (e.g. minimize the set of profile parameters stored in the NRF).

**** Second Change ****

# 5.X.	Key Issue for WT1.2 on enhancements of SBA

This key issue proposes to study how to support the SBI framework over the N2 interface in 6G.

The key issue will investigate the following aspects:

-	Study whether and how to introduce SBI-based framework on the N2 interface.

-	Study which RAN-specific services can be exposed to the CN and which operations can be supported.

-	Study whether and now to support UE based signalling between the RAN and the NFs in the CN control plane.

-	Study whether and how to simplify the NF discovery procedure via NRF (e.g. minimize the set of profile parameters stored in the NRF).

**** End of Changes ****