SA WG2 Meeting #170	S2-2507317

# August 25 - 29, 2025, Goteburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Non-5tuple SIP message transfer

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for KI#1 on IMS voice service over GEO satellite, in particular addressing SIP message size optimization with regards to 5tuple and security overhead.

This solution assumes to transfer SIP messages in NAS by using a CP bearer.

With this solution, SIP messages can be transferred over GEO satellite without 5tuple, and IPsec ESP header, trailer, and ICV (=Integrity Check Value).

2	Proposal

It is proposed to add the following text to TR 23.700-19 v020.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| #X | X |  |  |  |

********** Next Change (All new text) **********

6.x	Solution #x: Non-5tuple SIP message transfer

6.x.1	High level principles

This solution addresses KI#1, in particular SIP message size optimization with regards to 5tuple and security overhead.

In this solution, it is assumed to transfer SIP messages in NAS by using a CP bearer. In this solution, it is proposed not to use the security mechanism between UE and P-CSCF, considering the NAS security is practically sufficient.

For UL, UE sends non-5tuple packets and P-GW adds 5tuple. For DL, P-GW receives IP packets, removes 5tuple, and sends non-5tuple packets to S-GW.

6.x.2	Description

The assumptions of this solution are as follows:

- It is assumed to transfer SIP messages in NAS by using a CP bearer of a Control-plane PDN connection.

- Usually, the transport mode IPsec ESP is used between UE and P-CSCF (See TS 33.203). It is proposed not to use it for GEO voice.

- Usually, protected ports of UDP/TCP are assigned during the IMS Registration procedure and used between UE and P-CSCF (See TS 33.203). It is proposed to keep using it.

The principles of this solution are as follows:

- For UL, UE sends non-5tuple packets and P-GW adds 5tuple. For DL, P-GW receives IP packets, removes 5tuple, and sends non-5tuple packets to S-GW.

- P-GW uses default ports when adding 5tuple in the first SIP Register and when finding a bearer for its response. P-GW uses protected ports when adding 5tuple for the second SIP Register and when processing any other messages after that.

- P-CSCF informs protected ports via PCRF to P-GW during IMS Registration procedure.

- To cope with the change of ports, P-GW updates contexts of the bearer for the SIP message, so that the P-GW can consider the default ports at first and the protected ports later.

The advantages of this solution are as follows:

- SIP messages can be transferred over GEO satellite without 5tuple, and IPsec ESP header, trailer, and ICV(=Integrity Check Value).

6.x.3	Procedures

Figure 6.x.3-1: IMS Registration

0.	During the Attach procedure, the following are performed:

-	NAS security is established.

-	A PDN connection with a C-plane bearer is established. An IP address is allocated to UE. UE and P-GW recognize this IP address and an IP address of P-CSCF.

-	UE determines to use packets without 5tuple in this bearer. UE sends Extended protocol configuration options that have "CN header handling request" (a new indication) to P-GW. P-GW sends Extended protocol configuration options that have “CN header handling accepted" (a new indication) to UE. P-GW recognizes that it needs to add 5tuple for UL packets and removes 5tuple for DL packets for this bearer.

1.	UE determines not to use IPsec ESP between itself and P-CSCF. UE allocates port_uc and port_us (i.e., protected port numbers in UE). UE sends to MME a NAS message containing the initial SIP Register without 5tuple. This initial SIP Register contains port_uc and port_us and "P-GW context update indication" (a new indication), but does not contain information for IPsec ESP.

2.	MME sends the initial SIP Register without 5tuple via S-GW to P-GW.

3.	P-GW adds 5tuple to the initial SIP Register without 5tuple. P-GW uses the UE IP address as the source address and the P-CSCF IP address as the destination address. P-GW is pre-configured to use UDP. P-GW uses a port number that it has allocated in itself as the source port and the SIP port number as the destination port.

4.	P-GW sends the initial SIP Register to P-CSCF.

5.	P-CSCF does not treat the received initial SIP Register, which does not contain information for IPsec ESP, as an error and instead proceeds. P-CSCF sends the initial SIP Register to S-CSCF. This initial SIP Register does not contain port_uc and port_us and "P-GW context update indication". Some steps between step 5 and step 6 are omitted for the brevity of the description.

6.	S-CSCF sends 401 Unauthorized to P-CSCF.

7.	P-CSCF allocates port_pc and port_ps (i.e., protected port numbers in P-CSCF). P-CSCF sends to P-GW 401 Unauthorized containing port_pc and port_ps.

8.	P-GW removes 5tuple of 401 Unauthorized.

9.	P-GW sends 401 Unauthorized without 5tuple via S-GW to MME.

10.	MME sends to UE a NAS message containing 401 Unauthorized without 5tuple.

11.	P-CSCF recognizes the need to perform P-GW context update with regards to 5tuple based on "P-GW context update indication" that the P-CSCF has received at step 4. P-CSCF sends to PCRF an AA-Request containing Flow-Description that contains port_pc, port_ps, port_uc, and port_us.

12.	PCRF sends AA-Answer to P-CSCF.

13.	PCRF sends to P-GW an RA-Request containing Flow-Description that contains port_pc, port_ps, port_uc, and port_us.

14.	P-GW updates the context for the bearer with regard to 5tuple. P-GW sends RA-Answer to PCRF.

15.	UE sends to MME a NAS message containing SIP Register without 5tuple.

16.	MME sends the SIP Register without 5tuple via S-GW to P-GW.

17.	P-GW adds 5tuple to the SIP Register without 5tuple. P-GW uses the UE IP address as the source address and the P-CSCF IP address as the destination address. P-GW is pre-configured to use UDP. P-GW uses port_uc as the source port and port_ps as the destination port.

18.	P-GW sends the SIP Register to P-CSCF.

19.	P-CSCF sends the SIP Register to S-CSCF. Some steps between step 19 and step 20 are omitted for the brevity of the description.

20.	S-CSCF sends 200 OK to P-CSCF.

21.	P-CSCF sends 200 OK to P-GW.

22.	P-GW removes 5tuple of 200 OK.

23.	P-GW sends 200 OK without 5tuple via S-GW to MME.

24.	MME sends to UE a NAS message containing 200 OK without 5tuple.

25.	Subsequent SIP messages (e.g., SIP INVITE) are treated in the same way as steps 15 to 24 for 5tuple handling.

6.x.4	Impacts on Services, Entities and Interfaces

P-CSCF:

-	Receiving the initial SIP Register without IPsec related information and (not judging as an error, instead) proceeding without setting up IPsec components (i.e., SA).

-	Receiving a new indication in the initial SIP Register requesting to send a trigger for bearer context update. And sending the trigger containing protected ports of UE and of the P-CSCF to PCRF.

UE:

-	Handling of new indicators "CN header handling request" and "CN header handling accepted".

-	Sending the initial SIP Register as explained in the above.

NOTE:	Not using security between UE and P-CSCF may prohibit UE from sending some information (e.g., PANI). These side effects and their remedies need to be considered.

P-GW:

-	Handling of new indicators "CN header handling request" and "CN header handling accepted".

-	Adding 5tuple for UL packets based on the bearer context and finding a bearer for DL packets based on the bearer context and removing 5tuple from them.

-	Receiving a trigger from PCRF and updating the context of the bearer used for SIP message transfer in terms of 5tuple.

********** End of Changes **********