SA WG2 Meeting #170	S2-2507318

# August 25 - 29, 2025, Goteburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#2: New Sol: Use of SigComp and a GEO-voice-specific static dictionary

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#2.

1	Introduction

This contribution proposes a solution for KI#2 on IMS enhancement for GEO voice, in particular addressing SIP message size optimization with regards to SIP and SDP.

With this solution, the length of SIP and SDP content becomes shorter, in particular at the start of message exchange, by using SigComp and a specific static dictionary.

2	Proposal

It is proposed to add the following text to TR 23.700-19 v020.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| #X |  | X |  |  |

********** Next Change (All new text) **********

6.x	Solution #x: Non-5tuple SIP message transfer

6.x.1	High level principles

This solution addresses KI#2, in particular SIP message size optimization with regards to SIP and SDP.

It is proposed to use SigComp for GEO voice and a newly introduced GEO-voice-specific static dictionary.

6.x.2	Description

The principles of this solution are as follows:

- It is proposed to define a GEO-voice-specific static dictionary for terminologies frequently used for GEO voice communication.

- UE sends SIP Register containing an indication indicating use of SigComp and the GEO-voice-specific static dictionary, when judging GEO is used.

NOTE:	According to TS 24.229, SigComp is not used under E-UTRA and NR. In addition, the use of SigComp is not re-negotiated between initial registration and deregistration. These aspects need to be considered.

The advantages of this solution are as follows:

-	The length of SIP and SDP content becomes shorter, in particular at the start of message exchange.

6.x.3	Procedures

Figure 6.x.3-1: IMS Registration

1.	UE sends to P-CSCF a SIP Register containing PANI and an indication indicating use of SigComp and the GEO-voice-specific static dictionary. This SIP Register is compressed by using the GEO-voice-specific static dictionary and SigComp.

NOTE:	When UE uses a C-plane bearer, the UE can judge that SIP messages are protected and can send an initial SIP Register containing PANI and the above indication, so that the initial SIP Register is compressed.

2.	P-CSCF sends a response to UE.

6.x.4	Impacts on Services, Entities and Interfaces

P-CSCF:

-	Support of a GEO-voice-specific static dictionary.

UE:

-	Support of a GEO-voice-specific static dictionary.

-	Sending SIP Register containing an indication indicating use of SigComp and the GEO-voice-specific static dictionary, when judging GEO is used.

********** End of Changes **********