SA WG2 Meeting #170	S2-2507321

# August 25 - 29, 2025, Goteburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Simultaneous dedicated bearer setup during the Attach procedure

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for KI#1 on IMS voice service over GEO satellite, in particular addressing call setup time reduction by preparing before the call is initiated.

With this solution, call setup time becomes shorter, since a dedicated bearer is already set up during the Attach procedure. A default bearer and a dedicated bearer are simultaneously and therefore efficiently established.

2	Proposal

It is proposed to add the following text to TR 23.700-19 v020.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #1 | X | X |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X | X | X | X |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X | X |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 |  | X |  |  |
| #X | X |  |  |  |

********** Next Change (All new text) **********

6.x	Solution #x: Simultaneous dedicated bearer setup during the Attach procedure

6.x.1	High level principles

This solution addresses KI#1, in particular call setup time reduction by preparing before the call is initiated.

In this solution, MME decides to set up a default bearer and a dedicated bearer simultaneously and sends to S-GW the Create Session Request containing two Bearer Contexts to be created. MME sends to UE the Attach accept containing a request to activate a default bearer and a request to activate a dedicated bearer.

6.x.2	Description

The assumptions of this solution are as follows:

-	It is assumed that 1 PDN connection is used. Two bearers can be UP+UP, CP+CP, or CP+UP.

The principles of this solution are as follows:

-	MME, S-GW, and P-GW treat two bearers simultaneously.

-	Create Session Request contains two Bearer Contexts to be created and a Linked EPS Bearer ID. The latter is added to show which bearer is the default bearer.

-	MME and UE treat two bearers simultaneously.

-	Attach accept contains ESM message container and a newly introduced ESM message container2. The former contains Activate Default EPS Bearer Context Request and the latter contains Activate Dedicated EPS Bearer Context Request. Attach complete has the same structure.

The advantages of this solution are as follows:

-	Call setup time becomes shorter, since a dedicated bearer is already set up during the Attach procedure.

-	Setting up two bearers simultaneously is more efficient than setting up one after another.

6.x.3	Procedures

Figure 6.x.3-1: Attach

1.	The Attach procedure is performed to the point of MME accessing S-GW.

2.	MME sends Create Session Request to S-GW. This Create Session Request contains two Bearer Contexts to be created, where one is for the default bearer and the other is for a dedicated bearer, and Linked EPS Bearer ID, the value of which is set as EPS Bearer ID of the default bearer.

3.	S-GW creates two bearer contexts, first for the default bearer and next for the dedicated bearer. S-GW sends Create Session Request to P-GW. This Create Session Request contains two Bearer Contexts to be created and Linked EPS Bearer ID.

4.	P-GW creates two bearer contexts, first for the default bearer and next for the dedicated bearer. P-GW sends Create Session Response to S-GW. This Create Session Response contains two Bearer Contexts created.

5.	S-GW updates two bearer contexts, first for the default bearer and next for the dedicated bearer. S-GW sends Create Session Response to MME. This Create Session Response contains two Bearer Contexts created.

6.	MME updates two bearer contexts, first for the default bearer and next for the dedicated bearer. MME sets Activate Default EPS Bearer Context Request into ESM message container and Activate Dedicated EPS Bearer Context Request into ESM message container2. (ESM message container2 is a new IE) MME sends to UE Attach accept containing ESM message container and ESM message container2.

7.	UE processes at first ESM message container and next ESM message container2. For Attach complete, UE sets Activate Default EPS Bearer Context Accept into ESM message container and Activate Dedicated EPS Bearer Context Accept into ESM message container2. UE sends to MME Attach complete containing the ESM message container and the ESM message container2.

8.	MME processes at first ESM message container and next ESM message container2. When UP bearer(s) is used, MME sends Modify Bearer Request to S-GW as per the existing specifications.

9.	S-GW sends Modify Bearer Response to MME as per the existing specifications.

6.x.4	Impacts on Services, Entities and Interfaces

MME:

-	Capability of handling two Bearer Contexts to be created and a Linked EPS Bearer ID in Create Session Request.

-	Capability of handling the second ESM message container in the Attach procedure.

S-GW:

-	Capability of handling two Bearer Contexts to be created and a Linked EPS Bearer ID in Create Session Request.

P-GW:

-	Capability of handling two Bearer Contexts to be created and a Linked EPS Bearer ID in Create Session Request.

UE:

-	Capability of handling the second ESM message container in the Attach procedure.

********** End of Changes **********