SA WG2 Meeting #170	S2-2507322

# August 25 - 29, 2025, Goteburg, Sweden

Source: 	NTT DOCOMO

Title: 	KI#1: Update Sol#10 Editor's note resolution

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes to resolve Editor's notes in solution 10.

1	Introduction

This contribution proposes to resolve Editor's notes in solution 10.

Editor's note: FFS whether there is still no impact on QoS when the use of GBR is required.

Editor's note: Whether to use GBR for the default bearer or not and what impact is there are FFS.

No technical limitation is found against the use of GBR for the default bearer other than the text in TS 23.401 saying "A default bearer shall be a Non-GBR bearer". Still, specification-wise, it is considered there are impacts on the system as a whole if GBR is used for the default bearer. This aspect is captured, and these Editor's notes are removed.

Editor's note: FFS whether and how to perform authorization check.

This Editor's note is not addressed for now expecting a solution for this aspect will be proposed.

Editor's note: Potential impacts to IMS nodes (e.g. P-CSCF) due to the use of two PDN connections (i.e. two UE IP addresses) are FFS.

Editor's note: The impacts on P-CSCF is FFS.

An IP address is used only in SIP messages and only for the IP packet transfer between UE and P-CSCF, while the other IP address is used only in SDP messages and only for the IP packet transfer between UE and IMS AGW. Therefore, basically there is no problem for using two IP addresses, but there are cases where the IMS system does not work for this "two UE IP addresses" scenario.

-	IMS Local Breakout roaming does not work: TS 23.228, clause 5.1.0, has the following text, "For a UE using the IMS Local Breakout procedure as shown in Annex M, the IP address of the UE obtained from the local Gateway (i.e. single IP address) is used for both IM Subsystem related signalling and media."

While no impacts are expected in IMS nodes, there will be features (e.g., IMS Local Breakout roaming) that will not be supported. This aspect is captured, and these Editor's notes are removed.

2	Proposal

It is proposed to add the following changes to TR 23.700-19 v020.

********** First Change **********

## 6.10	Solution #10: Support of CP/UP combination and QoS for IMS voice service over GEO satellite with a pair of PDN connections

### 6.10.0	High-level solution Principles

This solution addresses KI#1, in particular QoS aspects.

This solution utilizes a pair of PDN connections.

NOTE:	This solution does not address any optimization aspect e.g. on how to achieve a shorter call setup time or how to transfer SIP message/voice media efficiently in CP or UP. This solution is intended to be used together with other solutions to achieve such optimization.

### 6.10.1	Description

Principles of this solution are as follows:

-	No impacts on the QoS mechanism in UE using NB IoT in the sense that a dedicated bearer does not need to be supported.

NOTE:	Using GBR for the default bearer has a system-wide impact.

-	Two PDN connections are used for IMS voice, where one without DRB is used for SIP message exchange and the other with DRB is used for voice media. These two PDN connections are linked together by using (newly introduced) PDN connectivity pair ID.

-	Each of the two PDN connections, when established with DRB, uses a default bearer only as per the existing specification. Different QCI is used for the default bearer depending on APN.

-	UE may explicitly request to establish PDN connection without DRB by using (newly introduced) "Control Plane Only Indicator requested" indicator.

-	UE internally associates contexts of the two PDN connections.

-	PCRF internally associates contexts of the two PDN connections.

Proposed solution for the issue of coexistence of S11-U and S1-U is as follows:

-	Consider that UE has established an SGi PDN connection using S1-U and an SGi PDN connection using S11-U and that the UE has become in the idle mode. When the UE has pending data for either of these SGi PDN connections, the UE triggers the RRC Connection Resume procedure that is used to resume the SGi PDN connection using S1-U. Upon reception of UE Context Resume Request, MME sends to S-GW both S1-U eNB TEID, which MME has received in Initial Context Setup response and stored, and S11-U MME TEID, which MME reserves in itself, by using Modify Bearer Request. Then both these SGi PDN connections are activated. UE sends the pending data via the corresponding PDN connection. In order to allow UE to establish an SGi PDN connection using S1-U and an SGi PDN connection using S11-U, MME is configured to allow establishing these SGi PDN connections under a certain condition (e.g. UE has sent Preferred CIoT network behaviour and UE's usage setting=voice in the Attach procedure).

Aspects for further consideration are as follows:

-	Whether QCI=1 is sufficient or a new QCI value needs to be introduced in order to express the difference between the usual voice media that assumes ptime=20ms and the GEO voice media that assumes ptime= 80ms or 100ms.

-	Whether and how to perform authorization check.

Editor's note:	FFS whether and how to perform authorization check.

### 6.10.2	Procedures

#### 6.10.2.1	General

The procedures in the below are an example and depict the case where SIP messages are transferred in CP and voice media is transferred in UP.

#### 6.10.2.2	Establishment of PDN connection for SIP message exchange

Figure 6.10.2.2-1: Establishment of PDN connection for SIP message exchange

0.	UE performs Attach without PDN Connectivity. The Subscription-Data that MME has received from HSS contains APN-Configuration-Profile for APN=ims and QCI=5 and for APN=ims_media (a new value) and QCI=1 (or a new QCI value for the GEO voice media).

1.	UE that uses NB IoT recognizes that two PDN connections are required for the GEO voice call; one for SIP message exchange and the other for voice media. UE may decide to use Control Plane for SIP message exchange, while still having available DRB(s). UE sends the PDN connectivity request containing APN=ims, "Control Plane Only Indicator requested" (a new indicator), and PDN connectivity pair ID (a new parameter).

2.	Based on "Control Plane Only Indicator requested", MME decides to set Control Plane Only PDN Connection Indication. MME sends to S-GW the Create Session Request containing APN=ims, Control Plane Only PDN Connection Indication, and PDN connectivity pair ID.

3.	S-GW sends to P-GW the Create Session Request containing APN=ims, Control Plane Only PDN Connection Indication, and PDN connectivity pair ID.

4.	P-GW sends to PCRF the CC-Request containing APN=ims, "Control Plane Only Indicator" (a new indicator), and PDN connectivity pair ID. PCRF recognizes that this PDN connection is for SIP message exchange only and not for voice media.

5.	The rest is the same as the existing procedure.

6.	PDN connection is established. This PDN connection uses NAS for transferring data (i.e. SIP messages) between UE and MME and uses U-plane between MME and P-GW via S-GW.

#### 6.10.2.3	IMS voice call setup

##### 6.10.2.3.1	MO procedure

NOTE:	Steps 9 to 14 in the below for establishing a PDN connection for voice media can be performed any time after a PDN connection for SIP messages are established as per the procedure in clause 6.10.3.2 to shorten the call setup time.

Figure 6.10.2.3.1-1: IMS voice call setup MO

0.	A path for SIP message exchange is established as a result of the procedure depicted in clause 6.10.3.2.

1.	UE sends SIP INVITE to MME by using NAS.

2.	MME forwards SIP INVITE towards P-CSCF on U-plane via S-GW and P-GW.

3.	P-CSCF sends SIP INVITE to S-CSCF.

4.	P-CSCF receives SIP 183 Session Progress.

5.	P-CSCF sends to PCRF the AA-Request to establish a dedicated bearer for voice media.

6.	PCRF recognizes that the targeted PDN connection is for SIP message exchange only and that UE is expected to establish another PDN connection for voice media when the UE receives SIP 183 Session Progress. PCRF decides not to send any message to P-GW. PCRF sends to P-CSCF the AA-Answer with success. (new behaviour of PCRF)

7.	P-CSCF sends SIP 183 Session Progress. SIP 183 Session Progress is transferred to MME on U-plane via P-GW and S-GW.

8.	MME sends SIP 183 Session Progress to UE by using NAS.

9.	UE recognizes the need to establish PDN connection for voice media. UE sends the PDN connectivity request containing APN=ims_media and PDN connectivity pair ID. UE sets the value of the PDN connectivity pair ID to be the same as that for the PDN connection for SIP message exchange.

10.	MME recognizes the value of the PDN connectivity pair ID is the same as that for the PDN connection for SIP message exchange and selects P-GW that is the same as that for the PDN connection for SIP message exchange. MME confirms that UE has available DRB(s). MME sends to S-GW the Create Session Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID.

11.	S-GW sends to P-GW the Create Session Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID.

12.	P-GW recognizes the value of the PDN connectivity pair ID is the same as that for the PDN connection for SIP message exchange and selects PCRF that is the same as that for the PDN connection for SIP message exchange. P-GW sends to PCRF the CC-Request containing APN=ims_media, QCI=1(or a new QCI value for the GEO voice media), and PDN connectivity pair ID. PCRF internally associates contexts of the two PDN connections having the same PDN connectivity pair ID. PCRF recognizes that this newly established PDN connection is for voice media.

13.	PCRF sends to P-GW the CC-Answer.

14.	The procedure for PDN connectivity establishment continues.

15.	PDN connection is established. This PDN connection uses U-plane for transferring data (i.e. voice media) between UE and P-GW via S-GW.

16.	The procedure for IMS voice call setup continues.

17.	S-CSCF sends SIP 200 OK to P-CSCF.

18.	When the procedure of Enabling of IP Flows is used, P-CSCF sends the AA-Request to PCRF.

19.	PCRF receives the AA-Request to the PDN connection for SIP message exchange, checks the context for the PDN connection for voice media as well, and decides to send the RA request to P-GW for PDN connection for voice media. PCRF sends to P-CSCF the AA-Answer with success. (new behaviour of PCRF)

20.	PCRF sends the RA-Request to P-GW for the PDN connection for voice media.

21.	P-GW sends the RA-Answer to PCRF.

22.	The procedure for IMS voice call setup continues. Then, IMS voice call is setup.

##### 6.10.2.3.2	MT procedure

NOTE:	Steps 9 and 10 in the below for establishing a PDN connection for voice media can be performed any time after a PDN connection for SIP messages are established as per the procedure in clause 6.10.3.2 to shorten the call setup time.

Figure 6.10.2.3.2-1: IMS voice call setup MT

0.	A path for SIP message exchange is established as a result of the procedure depicted in clause 6.10.3.2.

1.	P-CSCF receives SIP INVITE.

2.	P-CSCF sends SIP INVITE. SIP INVITE is transferred to MME on U-plane via P-GW and S-GW.

3.	MME sends SIP INVITE to UE by using NAS.

4.	UE sends SIP 183 Session Progress to MME by using NAS.

5.	MME forwards SIP 183 Session Progress towards P-CSCF on U-plane via S-GW and P-GW.

6.	P-CSCF sends SIP 183 Session Progress to S-CSCF.

7.	P-CSCF sends to PCRF the AA-Request to establish a dedicated bearer for voice media.

8.	PCRF recognizes that the targeted PDN connection is for SIP message exchange only and that UE is expected to establish another PDN connection for voice media when the UE receives SIP 183 Session Progress. PCRF decides not to send any message to P-GW. PCRF sends to P-CSCF the AA-Answer with success. (new behaviour of PCRF).

9.	Upon reception of SIP INVITE in step 3, UE recognizes the need to establish PDN connection for voice media. UE sends the PDN connectivity request containing APN=ims_media and PDN connectivity pair ID. UE sets the value of the PDN connectivity pair ID to be the same as that for the PDN connection for SIP message exchange.

10.	The procedure for PDN connectivity establishment continues.

11.	PDN connection is established. This PDN connection uses U-plane for transferring data (i.e. voice media) between UE and P-GW via S-GW.

12.	The procedure for IMS voice call setup continues. Then, IMS voice call is setup.

### 6.10.3	Impacts on Services, Entities and Interfaces

UE:

-	Handling of a new parameter "PDN connectivity pair ID" and of a new indicator "Control Plane Only Indicator requested".

-	In order to solve the issue of coexistence of S11-U and S1-U:

-	Handling of the order of activation of an SGi PDN connection using S1-U and activation of an SGi PDN connection using S11-U.

MME:

-	Selection of the same P-GW for two PDN connections that have the same value for a new parameter "PDN connectivity pair ID".

-	Handling of a new indicator "Control Plane Only Indicator requested".

-	In order to solve the issue of coexistence of S11-U and S1-U:

-	Making sure that S-GW receives all the necessary S1-U eNB TEID and S11-U MME TEID when SGi PDN connections are activated.

-	Allowing coexistence of an SGi PDN connection using S1-U and an SGi PDN connection using S11-U under certain conditions.

S-GW:

-	Transparent forwarding of a new parameter "PDN connectivity pair ID"

P-GW:

-	Selection of the same PCRF for two PDN connections that have the same value for a new parameter "PDN connectivity pair ID"

PCRF:

-	Handling of a new parameter "PDN connectivity pair ID" and of a new indicator "Control Plane Only Indicator"

EPS as a whole:

-	Using GBR for the default bearer has a system-wide impact.

IMS nodes (e.g., P-CSCF):

-	No impact if accepting some features (e.g., IMS Local Breakout roaming) do not work.

********** End of Changes **********