SA WG2 Meeting #170	S2-2507325

25 - 29 August, 2025, Goteborg, Sweden                  	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0062 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on reader selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some description in Clause 5.3.3 “NG-RAN Node and RAN Reader Selection” is not accurate. - In this sentence, “If the RAN Readers within an AIoT Area is configured in AIOTF, the AIOTF can directly select the RAN Reader(s).”, it’s not accurate to use the wording “within an AIoT Area” as a RAN Reader may serve multiple AIoT areas, and such RAN Reader should be selected if any of its served AIoT area is within the Requested Service Area. - In this sentence, “If an AIoT service request includes information about individual target AIoT Device(s), the AIOTF may consider the last known serving RAN Reader(s) from the AIoT Device context to determine the NG-RAN node and RAN Reader(s) for the request.” It should be clarified that “last known RAN Reader” is sent as part of Requested Service Area to the NG-RAN as this is the only information element expected by the NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarify how AIOTF selects RAN Readers. - Clarify last known RAN Reader is sent to NG-RAN as Requested Service Area. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In accurate description of RAN Reader selection mechanism may cause confusion in other work groups and implementation issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.3.3	NG-RAN Node and RAN Reader Selection

The AIOTF selects NG-RAN node(s) and optionally RAN readers based on the configured NG-RAN node selection information. The AIOTF may also select the RAN reader based on the stored last known RAN Reader information.

The AIOTF obtains the NG-RAN selection information (AIoT Area list, RAN Reader list, and, optionally, the location served by each RAN Reader and each AIoT Area) via OAM or local configuration.

When the AIOTF receives an AIoT service request, based on the received Target Area information in the request and the NG-RAN selection information, the AIOTF selects the NG-RAN node(s) to handle the request, and determines a Requested Service Area Information for each selected NG-RAN node. If the RAN Readers serving the Requested Service Area can be determined by the AIOTF based on the NG-RAN selection information, the AIOTF can directly select the RAN Reader(s).

The Requested Service Area Information provided to each NG-RAN node can be empty, a list of AIoT Areas, a list of RAN Readers, or both a list of AIoT Areas and a list of RAN Readers.

NOTE 1:	The Target Area information in a received request can span multiple NG-RAN node(s) or can be a subset of the supported AIoT areas of a single NG-RAN node.

The AIOTF sends the AIoT service request to each selected NG-RAN node, either directly or through the selected AMF, the AIOTF sends the AIoT service request to each selected NG-RAN node along with the corresponding Requested Service Area Information for the NG-RAN node.

If the AIOTF provides an empty Requested Service Area Information in the AIoT service request to the NG-RAN node, then the NG-RAN node may use all available RAN Readers under its coverage.

If an AIoT service request includes information about  individual target AIoT Device(s), the AIOTF may consider the last known serving RAN Reader, if available from the AIoT Device context, to determine the NG-RAN node and RAN Reader for the request. In this case the last known serving RAN Reader ID is sent as the Requested Service Area to the selected NG-RAN node.

NOTE 2:	The RAN Reader ID is not exposed to the AF.

NOTE 3:	An AIOTF receives the Inventory Report from an NG-RAN node includes a RAN Reader ID that represents the AIoT Device’s location at Reader granularity.

NOTE 4:	The AIOTF uses the RAN Reader ID and the NG-RAN node to update the last known serving RAN reader information in the local AIoT Device context.

END OF CHANGES