3GPP TSG SA WG2 Meeting #170	S2-2507333

25 – 29 Aug, 2025, Goteborg , SE	(revision of  )

# Source:	China Mobile

# Title:	New WID on 5G VN group enhancement

# Document for:	Approval

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	5G VN group enhancement

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	TEI20_e5GVN

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | x |  | x |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820017 | Stage 2 of Vertical_LAN | Vertical_LAN |
|  |  | {optional free text} |

Dependency on non-3GPP (draft) specification:  None

3          Justification

During the deployment of 5G-LAN technology, vertical industries require that a single DNN/S-NSSAI support multiple 5G VN Groups. This need arises from two critical constraints:

(1) Operation limitation: DNN/S-NSSAI availability is restricted because each network slice requires dedicated resource configuration and cannot be dynamically instantiated.

(2) UE capability constraints: UEs support a limited number of DNN/S-NSSAIs (typically two – one for IMS and another for default eMBB services).

Consequently, vertical enterprises typically request one DNN/S-NSSAI to provision an entire 5G LAN service (i.e., enabling local switching or N19 transmission for multiple 5G VN Groups), rather than allocating separate DNN/S-NSSAIs per 5G VN Group.

This approach fundamentally contrasts with the current 5G LAN mechanism where each DNN/S-NSSAI exclusively serves one 5G VN Group. To address this gap, in this TEI 20, it is suggested to standardize the mechanism of one DNN/S-NSSAI can support multiple 5G VN groups.

From UE perspective, that is the UE uses a DNN/S-NSSAI (configured for one vertical company) which belongs to 5G VN group1, while another UE uses the same DNN/S-NSSAI but belongs to 5G VN group2, and there is no specification impact for UE.

明确说一个UE只属于一个组。基本机制

4	Objective

The objective is to specify the following enhancements to 5G VN group management:

- Study enhancement for 5G VN group management to support that a DNN/S-NSSAI can support multiple 5G VN groups.

5	Expected Output and Time scale

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | The feature description of a DNN/S-NSSAI support multiple 5G VN groups. | SA#110 (Dec 2025) |  |
| 23.502 | Impacts to procedures according to impacts identified in 23.501. | SA#110 (Dec 2025) |  |

This TEI20_WID requires 0.5 TU.

6	Work item Rapporteur(s)

Dan Wang(wangdanyjy@chinamobile.com)

7	Work item leadership

SA WG2

8	Aspects that involve other WGs

9	Supporting Individual Members

Supporting IM name

China Mobile

Huawei

ZTE