3GPP SA WG2 Meeting #170	S2-2507337

# 25-29 Aug, 2025, Goteborg, SE											    (Revision of S2-2506987)

Source:	China Telecom

Title:	Interim principle for KI#1

Document for:	Approval

Agenda item:	20.4.1

Spec:	3GPP TR 23.700-67

Version:	0.2.0

Work Item:	FS_EnergySys_Ph2

Abstract: This pCR proposes interim agreements on KI#1 in TR23.700-67.

Introduction

Up to SA2#169 meeting, there are 8 solutions are documented in TR23.700-67 regarding KI#1. These solutions propose enhancements of collection, determination and exposure of Energy-related information from different aspects in the following table.

|  | Key Issues#1 |  |
| --- | --- | --- |
| Solution# | New Category/granularity of Energy information | Improvement in the calculation of Energy Consumption |
| 1 | Energy consumption percentile, Energy consumption per bit | - |
| 2 | Slice Level EnC from OAM system | - |
| 8 | OAM provides the renewable energy for UPF and gNB | Calculation of renewable and non-renewable energy consumption for different granularities |
| 9 | exposure of renewable energy with the AF | - |
| 10 | - | to calculate EnC based on time interval difference of data volume and energy |
| 11 | - | Considering NF's basic energy consumption |
| 12 | Exposure of energy consumption ranking of applications | - |
| 22 | Slice Level EnC from OAM system | - |

The calculation of precise energy consumption is complicated based on the nature of cloud-native deployment of 5GC NFs. Clause 6.7 in TR28.554 specifies many formulae of calculating the energy consumption of both physical NF and virtual NF, where the energy consumption of virtual NF is the sum of the energy consumption of all its consistent by estimation. Thus, all the energy consumption and energy efficiency calculation based on TR28.554 for VNFs is not the accurate value in the network even from the OAM, and might not be appropriate for operators to use as charging merits.

However, the solutions proposing relative results, e.g., percentile in Solution#1 and ranking in Solution#12, reflecting the trends of EnC of UEs and applications, is good reference for network to decide energy saving policies.

Proposed Changes

* * * First Change * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#1:

- The enhancement of EIF providing relative energy consumption and energy efficiency result, e.g., percentile and ranking of UE or application EnC is agreed for R20. And this information could be the input for PCF to decide energy saving behaviours.