3GPP TSG-WG SA2 Meeting #170	S2-2507340

# Goteborg, Sweden, 25 – 29 August 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0063 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to the Filtering Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 17) Rel-17 (Release 18) Rel-18 (Release 19) Rel-19 (Release 20) |  |
| Reason for change: |  | The description agreed during the SA#169 meeting to clause 5.8 has some ambiguities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications are proposed to clause 5.8 to fix ambiguities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification regarding filtering information. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

## 5.8	Filtering Information

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple filtering elements each one referring to a component (i.e. PLMN Identifier, NID, third party identifier and Identification Information) of the AIoT Device Permanent Identifier as defined in clause 5.7.

The filtering information includes a list of filtering elements. A filtering element includes:

-	Information indicating a component of the AIoT Device Permanent Identifier that is used to match the bitstring.

-	A bitstring value which is used to compare with the component.

-	An Offset from the beginning of the indicated component, which indicates the start location in the indicated component to be used to compare with the corresponding bitstring.

When the filtering element indicates to a component type PLMN ID, NID or Third Party Identifier, then the Offset is not included in the filtering element, i.e. the whole component is compared with the bitstring.

When the filtering element indicates to a component type Identification Information the Offset is always included.

There may be multiple filtering elements related to the component type Identification Information.

Each bitstring is corresponding to one component of the AIoT Device Permanent Identifier.

NOTE 1:	It is assumed that the AIOTF includes at most one filtering element information within the Filtering Information for ID Type, PLMN ID, NID or Third Party Identifier. It is assumed that the AIoT Device does not validate the number of filtering element(s) for any indicated component within the Filtering Information.

NOTE 2:	The maximum number of the filtering element for Identification Information is left to stage3.

Editor's note:	Whether and how to secure the filtering information is up to SA WG3.

Figure 5.8-1: The example of the comparing the bitstring to the component

To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every filtering element information within filtering information by comparing the whole bitstring within a filtering element information with the indicated component of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

* * * * End of Changes * * * *