3GPP TSG-SA2 Meeting #170	S2-2507344

Goteborg, Sweden, 25 - 29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.369 | CR | 0064 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the Filtering Information and Clear the EN for AIoT device ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AmbientIoT-ARC |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - There an EN left for AIoT device ID and CT4 has define it in 23.003. Editor's note: The reference in NOTE 3 needs to be updated, when the appropriate stage 3 document is identified. 2. Filtering information has been defined in clause 31.3 TS 23.003 as follows: Figure 31.3.1: Structure of Filtering Information The Filtering Information is composed of the following parts, with a maximum length of 256 bits.: 1) ID Type shall have a fixed length of 8 bits, as specified in clause 31.2. 2) Optional filtering information for PLMN ID as specified in clause 12.1. 3) Optional filtering information for NID as specified in clause 12.7.1. 4) Optional filtering information for Third Party ID shall have a fixed length of 24 bits. 5) Optional Identification Information Filter(s), there may be multiple filtering element within the Filtering Information for the Identification Information, as specified in clause 5.8 of 3GPP TS 23.369 [152]. The structure is show in Figure 31.3.2. It is used to filter the target devices with bitstring of Identification Information equal to a specific value. Figure 31.3.2: Structure of Identification Information Filter - Offset consisting of 7 bits. It represents the position of the Identification Information's bitstring to be compared. Value 0 means the starting position is 1, and value 127 means the starting position is 128. - Length consisting of 7 bits. It represents the length of value of the bitstring to be compared. - Component Value consisting of 1 - 128 bits. It represents the component value of the bitstring to be compared. According to clause 5.8 of TS 23.369, Filtering Information is composed of a list of filtering elements. Each filter element can be ID type, PLMN ID, NID, Third Party Identifier, or Identification information. However, since only one of the ID type, PLMN ID, NID, Third Party Identifier can be included, and only identification information can be included multiple times, it is proposed to align CT4 coding of Filtering Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clear the EN and refer to 23.003. Align the filter Information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is left over EN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

### 5.7.2	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT Device Permanent Identifier shall be allocated to each AIoT Device. An AIoT Device Permanent Identifier is assigned either by an operator or by a third party. The AIoT Device Permanent Identifier is used to identify an AIoT Device and locate the entity where the AIoT Device related information is stored.

NOTE 1:	How to configure an AIoT Device with the AIoT Device Permanent Identifier is out of scope of this specification.

The AIoT Device Permanent Identifier includes the following components:

-	The ID Type, including:

-	Information indicating whether a PLMN ID is included.

-	Information indicating whether a NID is included.

-	Information indicating whether a third party identifier is included.

-	Identification Information Type, indicating whether the Identification Information contains an EPC or unstructured information.

-	The Domain Information includes none, one or more of the following:

-	A PLMN Identifier (i.e., MCC and MNC) as specified in TS 23.003 [6] when the information in the ID type indicates it is included

-	A Network Identifier (NID) as specified in TS 23.003 [6] when the information in the ID type indicates it is included.

-	A third party identifier used to identify a third party when the information in the ID type indicates it is included.

-	The Identification Information is used to distinguish different AIoT Devices within the scope identified by Domain Information (if available) and can contain either:

-	An EPC, as defined in clause 14 of GS1 TDS Release 2.1 [7].

-	Unstructured information, where the content is defined by the allocator.

Figure 5.7.2-1: AIoT Device Permanent Identifier Structure

An operator allocated AIoT Device Permanent Identifier should include the identifier of the network for the operator. The identifier of the network is present as either a PLMN Identifier, NID or both in the AIoT Device Permanent Identifier.

A third party allocated AIoT Device Permanent Identifier may include none of the following information or include any combination of at least one kind of the following information: a PLMN Identifier, NID or the third party identifier.

NOTE 3:	The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in clause 31.2 of TS 23.003 [6].

NOTE 4:	When the Domain Information is empty, the AIOTF uses, e.g., Identification Information (i.e EPC) to discover and select the ADM instance or the external server for the AIoT Device Profile Data.

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

NOTE 5:	The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

NEXT CHANGE

## 5.8	Filtering Information

The filtering information is used to identify or filter multiple AIoT Devices and is constructed by one or multiple components(i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information) of the AIoT Device Permanent Identifier as defined in clause 5.7.

The Filtering Information includes  one or more of the following components:

-	The ID Type,

-	A PLMN Identifier

-	A Network Identifier (NID).

-	A third party identifier.

-	A list of Identification Information Filter(s), which is used to filter the component of Identification Information. Each Identification Information Filter includes:

-	An Offset field, which indicates the start location of the Identification Information to be compared with the corresponding bitstring.

-	A Length field, which indicates the length of value of the bitstring to be compared.

-	A bitstring field, which is used to compare with the component.

NOTE 1:	It is assumed that the AIOTF includes at most one  ID Type, PLMN ID, NID or Third Party Identifier component in the Filtering Information (i.e. the ID Type, PLMN ID, NID or Third Party Identifier component does not appear twice in the Filtering Information). It is assumed that the AIoT Device does not validate the number of component(s)  for any indicated component within the Filtering Information.

NOTE 2:	The maximum number of the Identification Information Filter(s)for Identification Information is left to stage3.

Editor's note:	Whether and how to secure the filtering information is up to SA WG3.

Figure 5.8-1: The example of the comparing the bitstring to the component

To determine whether an AIoT Device Permanent Identifier matches the filtering information, it is compared with every component within filtering information by comparing the component(s) in the filtering information with the corresponding bitstring of its AIoT Device Permanent Identifier. If all the compared bitstrings match the AIoT Device Permanent Identifier then an AIoT Device Permanent Identifier matches the filtering information. If an AIoT Device Permanent Identifier does not contain an indicated component then it does not match the filtering information.

ND OF CHANGES