SA WG2 Meeting #S2-170	S2-2507349

25 - 29 August, 2025, Goteborg, Sweden

Source:	Samsung

Title:	KI#1 Interim Agreements

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes the interim agreements for AIML_CN KI#1.

- Discussion

This pCR proposes the interim agreements with the consideration of RAN-provided input, solution, and discussion so far.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel (Note 7) | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel (Note 7) |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer (Note 1) | No AI/ML specific controllability | FFS: level of controllability (Note 5) | Full controllability | Full controllability |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure or FFS other procedures | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS on level of visibility (Note 5) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) | Opt A) Full visibility for standardized data contents. Opt B) Partial visibility for partially standardized data contents. (Note 6) Opt C) No standardized visibility. (Note 6) |

- Proposal

It is proposed to adopt the following changes into TR 23.700-04.

*** Start of the change ***

7	Interim agreements

7.1	Agreed Principles

7.1.X	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

First termination entity of the data transfer:

- NF located inside the CN, which supports the direct UP connection with the UE

Editor’s Note:	It is FFS whether the DCF or other 5GC NF will be perform the control data transfer over UP for UE data collection.

AIML-specific collected UE Data Transfer Path via UP and its protocol layer for data transfer:

- Path of the UE Data Transfer is as follows: The data is transferred from UE to the first termination NF in the CN via UPF.

Editor’s Note:	It is FFS how the UE Data is transferred from the first termination NF to the UE-side model training/OTT server

- A dedicated protocol layer used for AIML-specific UE Data Transfer over UP tunnel is assumed to be located above TCP/IP layer.

Controllability of MNO on data transfer:

- MNO can have controllability via the first termination entity in the CN (e.g., Data Collection Function)

- Subject to user consent, DCF controls data transfer over UP, including trigger and terminate data transfer. The DCF may trigger or terminate the data transfer based on its internal logic or request from UE model training entity server.

Editor’s Note:	It is FFS whether the DCF or other 5GC NF will be perform the control data transfer over UP, acting as, the first termination entity for UE data collection.

Editor’s Note:	It requires coordination with RAN regarding the details on how the first termination entity control the data transfer over UP.

- How to provision UE with the information for the first termination entity.

  - NAS signaling is used to provide information required for UE to discover the first termination entity and establish the UP connection to it.

- How the UP connection is established with the UE:

  - The UP connection is established based on IP address/FQDN of the first termination entity.

  - The UP connection establishment may be triggered by DCF or UE.

- How to differentiate the traffic for UE Data Transfer from the UE’s regular traffic (i.e., application traffic over UP) in the 5GC.

  - Dedicated PDU session(s) may be used for data transfer over UP for UE data collection for model training. The mechanism of existing URSP rule can be reused with specifying specific traffic descriptor(s) and Route Selection Descriptor(s).

  - UE data collection for model training may be differentiated from other regular traffic by the core network in PDU session and/or QoS flow.

- How the UE knows about what the standardized data to be transferred are: FFS

Editor’s Note:	It is FFS how the UE knows about what the standardized data to be transferred are and it may require coordination with RAN

Visibility of data content in MNO and Data format

- In this study, it is assumed that the collected data refers to the data content fully standardized by RAN1.

Editor’s Note:	It is FFS how to support visibility of data content in MNO.

*** End of the change ***