SA WG2 Meeting #170	S2-2507353

25 – 29 Aug, 2025, Goteborg , SE

Source:	Xiaomi

Title:	[WT#1.1] KI on UE-CN interaction for control signalling for 6G

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: The clarification on WT#1.1 on UE-CN interaction for control signalling for 6G.

1. Justifications

The contribution provides clarification on WT#1.1 as agreed in the SID of FS_6G_ARC(SP-250806).

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

1.1	Architectural Motivation

For 6G, the 6G system shall be able to support the following services:

- the basic connectivity services as supported by 5G

- legacy beyond connectivity services, e.g. the SMS service, Location and positioning services as indicated in clause 5.2.3.2 of 22.870(as following).

The 6G system shall be able to support the following services:

-	Mission Critical Services, i.e., MCPTT, MCData, MCVideo, ref TS 22.179 [53], TS 22.280 [54], TS 22.281 [55]. and TS 22.282 [56],

-	Message service, ref TS 22.262 [57],

-	SMS, ref TS 22.101 [58],

-	Multimedia communication services, ref TS 22.101 [5f], TS 22.261 [14], TS 22.173 [59],

-	IMS Multimedia Telephony Service, ref TS 22.261 [14], TS 22.173 [59],

-	Roaming services, ref TS 22.011 [60], TS 22.101 [58], TS 22.261 [14],

-	Location and positioning services, ref TS 22.261 [14], TS 22.071 [61],

-	Broadcast and Multicast Services, ref TS 22.261 [14],

-	Emergency Services, ref TS 22.101 [58],

-	PWS, ref TS 22.268 [62],

-	MPS, ref TS 22.153 [63],

-	Lawful Intercept, ref TS.22.261 [14], and

-	Other regulatory services, based on regional/ national regulatory requirements.

Editor’s note: other services can be added, FFS.

- Emerging beyond connectivity services, e.g. the operator provided service for AI, Integrated Sensing and Communication service, Immersive Communication as discussed in SA1 for 6G.

The network simplification for rolling out new services is also discussed in SA1 with the following requirement:

[PR 5.9.5.6-1] The 6G system shall support on-demand rollout (e.g. within hours) of new or updated services/capabilities with minimal disruption to existing services, including the ability to efficiently rollback those services/capabilities, as needed (e.g. in case of failures or demand from other services).

Furthermore, the following is also reflected in the SID of FS_6G_ARC(SP-250806):

The study should consider at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.

1.2	Gap Analysis-UE-CN interaction for control signalling within 5G architecture

5G re-architects 4G by disaggregating core network functions into cloud-native micro-services, fully separating user and control planes, introducing end-to-end slicing, unifying all access technologies under a single service-based framework and supporting native edge computing & exposure and other enhancements. With regards to UE-CN interaction for control signalling, the following are supported:

- Control signalling transportation via control plane NAS to support UE-CN interaction for connectivity services e.g. the access management, security management, mobility management, session management, UE policy management, etc.

- Control signalling transportation via control plane NAS to support UE-CN interaction for beyond connectivity services, which can also be named as services other than connectivity services provided by operator, e.g. the SMS service, Location and positioning services, IoT service, etc.

- Control signalling transportation via user plane to support UE-CN interaction for beyond connectivity services, which can also be named as services other than connectivity services provided by operator, e.g. the Location and positioning services via user plane, etc.

With regards to the 5G NAS design, it is architecturally modular, but implementation monolithic.

- MM vs. SM split: 5G NAS cleanly separates Mobility Management (5GMM) and Session Management (5GSM) into two independent sub-layers. Each sub-layer can be updated or extended without touching MM procedures.

- Transport coupling: All messages ride on a NAS-MM, including NAS-MM itself with detailed information elements, NAS-SM and other messages for each specific purpose encapsulated into the Payload container of NAS transport message.

Observation#1: Considering above, in order to facilitate commercialization, achieve better customer experience and make a more successful generation, 6G design for UE-CN interaction should ensure that

- the system being architecturally modular, and implementation modular with as little independency from each self-contained module as possible;

- the system supports control signalling for connectivity services and beyond connectivity services;

- the system supports UE-CN interaction for control signalling over control plane;

- the system supports UE-CN interaction for control signalling over user plane;

- the design UE-CN interaction should be conceived considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.

1.3	Technical Impact

As the UE-CN interaction is the basic part for the UE accessing the mobile network to any service involving UE-CN interaction, so this WT/KI influence:

- Overall system architecture (e.g., UE, functional models in core network, UE-CN reference points)

- Interaction between UE and network via control plane and user plane

- All other WTs involving UE-CN interaction

- Interworking or migration from 5G to 6G

Proposal

It is proposed to add the following to the TR 23.801-01 “Study on Architecture for 6G System”

**** First Change ****

Annex A.X. WT1.1 Scope

WT#1.1 focuses on UE-CN interaction for control signalling. The 6G design for UE-CN interaction for control signalling should consider the following:

- the system is architecturally modular, and implementation modular with as little independency from each self-contained module as possible;

- the system supports control signalling for connectivity services and beyond connectivity services;

- the system supports UE-CN interaction for control signalling over control plane;

- the system supports UE-CN interaction for control signalling over user plane;

- the design UE-CN interaction should be conceived considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.

**** Second Change ****

5.X. Key Issue #1.1: UE-CN interaction for control signalling

In order to facilitate commercialization, achieve better customer experience and make a more successful generation, 6G design for UE-CN interaction for control signalling should consider the following:

- the system is architecturally modular, and implementation modular with as little independency from each self-contained module as possible;

- the system supports control signalling for connectivity services and beyond connectivity services;

- the system supports UE-CN interaction for control signalling over control plane;

- the system supports UE-CN interaction for control signalling over user plane;

- the design UE-CN interaction should be conceived considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.

Comparing to the existing 5G architecture on UE-CN interaction for control signalling, the following questions need to be addressed during the study on UE-CN interaction for control signalling in 6G:

-	Whether and how to further modulize connectivity services with self-contained modules which are independent from each other;

-	Whether and how to further modulize beyond connectivity services with self-contained modules which are independent from each other;

NOTE:	This WT discusses generic mechanism for all beyond connectivity services, specific details for beyond connectivity services covered by other WTs are not covered in this WT, specific details for beyond connectivity services not covered by other WTs are covered in this WT.

-	whether and how to enhance UE-CN interaction for control signalling over control plane considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency;

-	whether and how to enhance UE-CN interaction for control signalling over user plane considering at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency;

-	whether and how to define the principle to support the modulized connectivity services and beyond connectivity services on UE-CN interaction via control plane or via user plane with potential enhancements in the related study in this WT.

**** End of Changes ****