SA WG2 Meeting #170	S2-2507364

25 – 29 Aug, 2025, Goteborg, SE

Source:	Xiaomi

Title:	[WT#8, cellular IoT] WTs and KIs for cellular IoT enablers in 6G

Document for:	Agreement

Agenda Item:	20.6.8

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: It is proposed the WTs and KIs of cellular IoT enablers in 6G in the TR 23.801, considering the requirements, on the WT#8 cellular IoT enablers in 6Gapproved in SP-250806 (SA2 6G SID).

Editor's Note: We encourage companies to include a short summary in their abstract of the contributions, mainly to help readers quickly understand the intent and scope of the contribution.

1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

WT#8: Study whether and how to support cellular IoT enablers in 6G, based on RAN decision for 6G IoT.

NOTE 9:	The detailed scope for WT#8 will be coordinated and aligned with RAN. Ambient IoT is not in the scope of the study.

The WT#8 was agreed in SP-250806 (SA2 6G SID) approved in SA#108, including:

With respect to the cellular IoT enablers in 6G, it is justified and proposed as follows:

Architectural Motivation:

The 5GS supports the Control Plane CIoT 5GS Optimisations and User Plane CIoT 5GS Optimisations over E-UTRA as described in TS 23.501[2]. The Cellular IoT is in earlier 3GPP releases also referred to as Machine Type Communication (MTC) (see clause 4.3.17 of TS 23.401 [x]).

Though motivated by scenarios and use cases defined in TS 22.261 [2], in 5GS the functions added to support CIoT have general applicability and are in no way constrained to any specific scenario, use case or UE types, except where explicitly stated. In the context of CIoT the term AF denotes an SCS/AS as defined TS 23.682 [36].

While 5G has made significant strides for CIoT, challenges remain including the CIoT 5GS optimisations is only supported over E-UTRA, CIoT 5GS optimisations are not supported over Non-3GPP RAT type accesses, and the new services (e.g., AI, sensing) supporting for the CIoT is FFS.

How to support cellular IoT enablers in 6G to improve the CIoT should be studied on 6G architecture.

Gap Analysis:

As described above, while 5G has made significant strides for CIoT, challenges remain including the CIoT 5GS optimisations is only supported over E-UTRA, CIoT 5GS optimisations are not supported over Non-3GPP RAT type accesses, and the new services (e.g., AI, sensing) supporting for the CIoT is FFS.

There are new use cases with requirements about the CIoT in 6G, are proposed in 3GPP TR 22.870, the existing stage 1 specifications of 3GPP, some examples of potential new requirements related to the 6G CIoT, as follows:

- [PR 10.1.6-1] The 6G system shall support UEs with both wide-area (i.e. rural area and deep indoor) coverage capabilities and broadband capabilities.

NOTE: The wide-area coverage performance target will need to be confirmed by RAN working groups.

- [PR 10.1.6-2] The 6G system shall provide emergency services (e.g. emergency calls) support from rural area and deep indoor coverage scenarios, including leveraging wide-area coverage capabilities.

- [PR 10.1.6-3] The 6G system shall support MO and MT basic services with low-capacity demands (e.g. messaging, SMS, and small data).

- [PR 10.2.6-1] The 6G system shall support communication service with the following KPI’s

| Profile | Characteristic parameter |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit rate down link (Mbit/s) | Bit rate up link (Mbit/s) | End-to-end latency: maximum (ms) | Payload Message size (Kbyte) | # of UEs connection | Communication service availability: target value | Transfer Interval |
| Smart Grid monitor and control | Peak [5] | Peak [5] | TBD | TBD (note) | TBD | [> 99.99] | TBD |
| NOTE: Typical message sizes are 500-byte payload meter readings to 1 MB differential and 20 MB complete firmware upgrades |  |  |  |  |  |  |  |

Technical Impact:

- Whether the 6G CIoT is supported over multiple Access (e.g. not or not only the E-UTRA)

- Whether and how to support the 6G CIoT considering the exist and new services capability (e.g., AI, sensing).

- How to enable 6G CIoT to ensure the 6G LPWA devices supporting basic network connectivity which can allow them to be connected to future generations (e.g. new monitoring or power saving mechanism).

- Whether and how to support the 6G CIoT considering the wide-area (i.e. rural area and deep indoor) coverage capabilities.

**** First Change (all new text) ****

Annex A.X. WT#8 Scope: Cellular IoT enablers in 6G

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#8	Study how to support cellular IoT enablers in 6G:

-	Whether and how the 6G CIoT is supported over multiple Access (e.g. not or not only the E-UTRA).

-	Whether and how to support the 6G CIoT considering the exist and new services capability (e.g., AI, sensing).

-	How to enable 6G CIoT to ensure the 6G LPWA devices supporting basic network connectivity which can allow them to be connected to future generations (e.g. new monitoring or power saving mechanism).

-	Whether and how to support the 6G CIoT considering the wide-area (i.e. rural area and deep indoor) coverage capabilities.

**** Second Change (all new text) ****

5.X. Key Issue #8: Study how to support cellular IoT enablers in 6G

The 5GS supports the Control Plane CIoT 5GS Optimisations and User Plane CIoT 5GS Optimisations over E-UTRA as described in TS 23.501 [2]. The Cellular IoT is in earlier 3GPP releases also referred to as Machine Type Communication (MTC) (see clause 4.3.17 of TS 23.401 [x]).

Though motivated by scenarios and use cases defined in TS 22.261 [y], the functions added to support CIoT have general applicability and are in no way constrained to any specific scenario, use case or UE types, except where explicitly stated. In the context of CIoT the term AF denotes an SCS/AS as defined TS 23.682 [z].

While 5G has made significant strides for CIoT, challenges remain including the CIoT 5GS optimisations is only supported over E-UTRA, CIoT 5GS optimisations are not supported over Non-3GPP RAT type accesses, and the new services (e.g., AI, sensing) supporting for the CIoT is FFS.

TR 22.870 [w] has several requirements for the 6G CIoT. This key issue will study how to support cellular IoT enablers in 6G.

In particular, this key issue will study:

-	Whether and how the 6G CIoT is supported over multiple Access (e.g. not or not only the E-UTRA).

-	Whether and how to support the 6G CIoT considering the exist and new services capability (e.g., AI, sensing).

-	How to enable 6G CIoT to ensure the 6G LPWA devices supporting basic network connectivity which can allow them to be connected to future generations (e.g. new monitoring or power saving mechanism).

-	Whether and how to support the 6G CIoT considering the wide-area (i.e. rural area and deep indoor) coverage capabilities.

**** Third Change ****

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3] 	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4] 	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[5] 	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[6] 	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and Systems Aspects; IP Multimedia Subsystem (IMS) emergency sessions".

[x]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[y]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[z]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[w]	3GPP TS 22.870: " Study on 6G Use Cases and Service Requirements; Stage 1".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

**** End of Changes ****