3GPP TSG-SA2 Meeting #170	S2-2507372

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6419 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support PDU Set QoS exposure to AF by SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY |  |  |  |  |  | Date: |  |  | 2025-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on Rel-18 and 19 XR enhancements, an AF may provide PDU set QoS parameters (PSDB, PSER) along with normal QoS parameters. Depending on whether the serving NG-RAN supports PDU set based QoS handling (and corresponding Rel-19 notification, AQP enhancements) or not, either PDU set QoS handling or normal QoS handling will be applied, respectively. The AF has no control over the selected NG-RAN and cannot be aware of its capabilities regarding PDU set QoS. Nevertheless, the application needs to be aware of whether the requested PDU set QoS or normal QoS has been used (and achieved or not), to adapt accordingly and/or properly evaluate the usefulness of PDU set QoS enhancements. Notice that based on Rel-19 XRM changes to QoS notification control, the AF cannot know whether lack of direction indication means i) both directions (rel-19) or ii) at least one direction (Rel-18). Thus, it is proposed to expose this information to the AF. This CR updates TS23.501 to support an explicit indication by the SMF on whether the normal QoS or PDU Set QoS parameters has been used (depending on NG-RAN support). More details on the issue can be found in the discussion paper S2-2503436. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support SMF exposure of whether PDU set QoS has been enabled or not. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The application cannot identify whether the resulting performance (QoS, QoE) results from using the normal QoS or the PDU Set QoS parameters and hence can neither adapt accordingly nor properly evaluate the usefulness of PDU set QoS enhancements |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.5.1; 5.37.5.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.37.5.1	General

A PDU Set is comprised of one or more PDUs carrying an application layer payload such as a video frame or video slice. The PDU Set based QoS handling by the 5G-AN is determined by PDU Set QoS Parameters in the QoS profile of the QoS Flow (specified in clause 5.7.7) and PDU Set Information provided by the PSA UPF via N3/N9 interface as described in clause 5.37.5.2. The PDU Set based Handling can be applied for GBR and non-GBR QoS Flows. The AF should provide PDU Set related assistance information for dynamic PCC control. One or more of the following PDU Set related assistance information may be provided to the NEF/PCF using the AF session with required QoS procedures in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3].

-	PDU Set QoS Parameters as described in clause 5.7.7

-	Protocol Description: Indicates the transport protocol used by the service data flow (e.g. RTP, SRTP) and information, e.g. the following:

-	RTP [185] or SRTP [186];

-	RTP or SRTP with RTP Header Extensions, including:

-	RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179];

-	Other RTP Header Extensions as defined RFC 8285 [189], RFC 9143 [207];

-	RTP or SRTP without RTP Header Extensions, but together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with RTP Header Extensions for PDU Set Marking as defined in TS 26.522 [179] and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]);

-	RTP or SRTP with other RTP Header Extensions following RFC 8285 [189] and together with RTP Payload Format (e.g. H.264 [187] or H.265 [188]).

NOTE 1:	With the Protocol Description options combining SRTP together with RTP Payload Format the UPF can still obtain some of the PDU Set information from the RTP Header (refer to Annex A of TS 26.522 [179]).

-	Media over QUIC (MoQ) Transport, IETF draft-ietf-moq-transport [201] (as further described in clause 5.37.9.2).

When RTP Header Extensions for PDU Set Marking (as defined in TS 26.522 [179] or other RTP header extensions as defined in RFC 8285 [189] is included, the differentiation between different RTP Header Extension Types should be supported.

When RTP Payload Format is included, the differentiation between different RTP Payload Formats should be supported.

NOTE 2:	Multiplexing of different transport protocols and different media traffic for differentiated PDU Set based handling is not supported in the current Release.

The Protocol Description can be UL only, DL only or UL and DL. The Protocol Description for UL and DL traffic may be different.

For end-to-end encrypted traffic, PDU Set Information is received as media related information from the Application Server, see clause 5.37.9.

NOTE 3:	Identification of PDU Set information for end-to-end encrypted traffic is supported in the DL direction only.

AF provided PDU Set QoS Parameters and UL and/or DL Protocol Description may be used in determining the PCC Rule by the PCF as defined in clause 6.1.3.27.4 of TS 23.503 [45] and the DL Protocol Description may be used for identifying the PDU Set Information and PDU Set Information marking by the PSA UPF. An AF may subscribe to notification of the status changes for the PDU Set based QoS handling as described in clause 5.37.5.x.

When the SMF receives the PCC rule, the SMF performs binding of the PCC rule to one QoS Flow as described in clause 6.1.3.2.4 of TS 23.503 [45]. At least one of the following shall be included in the PCC rule to enable PDU Set based QoS handling: 1) a PSIHI and/or 2) both PSDB and PSER. Based on the PCC rule, the SMF adds the PDU Set QoS Parameters to the QoS Profile of the QoS Flow as described in clause 6.2.2.4 of TS 23.503 [45]. Alternatively, the SMF may be configured to support PDU Set based Handling without receiving PCC rules from a PCF.

If no DL PDU Set QoS parameters are included in the PCC rule, but a DL Protocol Description is included in the PCC rule received by the SMF and the PSA UPF supports PDU Set  basedhandling (i.e. supports to perform PDU Set Information marking), the SMF may, based on the local operator policy, send to the NG-RAN, a DL PDU Set Information Marking Support Indication indicating that the DL PDU Set Information marking is supported.

For the downlink direction, the PSA UPF identifies PDUs that belong to PDU Sets and marks them accordingly as described in clause 5.37.5.2. If the PSA UPF receives a PDU that does not belong to a PDU Set based on Protocol Description for PDU Set identification, then the PSA UPF still maps it to a PDU Set and determines the PDU Set Information as described in clause 5.37.5.2.

NOTE 4:	If the PSA UPF receives a PDU that does not belong to a PDU Set, then it is assumed that the UPF determines the PDU Set Importance value based on pre-configuration.

For the uplink direction, the UE may identify PDU Sets and how this is done is left up to UE implementation. The SMF may send the UL Protocol Description associated with the QoS rule to UE.

NOTE 5:	Using the Protocol Description or not is left to UE implementation. The use of Protocol Description does not impact QoS Flow Mapping in the UE.

In this Release, the PDU Set based handling is supported in 5GS for a UE registered in 3GPP access for single access PDU Session with IP PDU Session Type, for a UE registered in untrusted or trusted non-3GPP accesses for single access PDU Session with IP PDU Session Type, and for a 5G-RG registered in W-5GAN for single access PDU Session with IP PDU Session Type. The support of PDU Set based handling in 5G-RG is specified in TS 23.316 [84].

# * * * * Second change * * * *

#### 5.37.5.x	Exposure of PDU Set based QoS handling status

The SMF, by sending PDU Set QoS parameters (as described in clause 5.7.7.1) to the NG-RAN, requests the NG-RAN to activate PDU Set based QoS handling for a given QoS Flow and the NG-RAN provides the SMF with a PDU Set Based Handling Support Indication, if the PDU Set based handling is supported. Based on this indication, SMF may activate (or deactivate) the PDU Set identification and marking in the PSA UPF as described in clause 5.37.5.2. An AF may subscribe to PDU Set QoS handling status notifications by the SMF as defined in clause 6.1.3.27.4 of TS 23.503 [45].

# * * * End of changes * * * *