SA WG2 Meeting #170	S2-2507378

Stor-Göteborg, Sweden, August 25 – 29th, 2025

Source:	NEC

Title:	WT#1.1, Study proposal to enhance a NAS functionality

Document for:	Discussion / Approval

Agenda Item:	WT#1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the study considerations related to the WT1which proposes to enhance the NAS functionality so that introduction of a new NAS will not impact existing NAS procedure.

# 1. Justifications

S2-2506096 includes WT#1.1

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

The discussion paper discusses following aspects support WT#1.1 bullet a, and b.

Architectural Motivation: A key architectural limitation in 5G (5GS) and its predecessors (EPS, UMTS, GPRS) is the reliance on a single mobility management node as the gateway to the core network. In the 5GS, this role is filled by the Access and Mobility Management Function (AMF).

The AMF serves as the sole entry point for all Non-Access Stratum (NAS) control signaling from the User Equipment (UE). This includes not only its primary Mobility Management (5GMM) messages but also messages destined for other Network Functions (NFs), such as:

- Session Management (5GSM) messages for the SMF

- SMS messages

- Policy Control Function (PCF) related messages

- Location Services (LCS) messages

This central-proxy model creates two significant problems:

- Performance Bottleneck: The AMF must process its own 5GMM traffic while simultaneously routing messages for all other core network services. This dual workload makes the AMF a potential performance bottleneck.

- Service Dependency: Since all service-related signaling passes through the AMF, its performance directly impacts the reliability and efficiency of every other network function.

Gap Analysis: This centralized design introduces serious performance issues:

- Single Point of Congestion: By forcing one function to manage its own workload plus the signaling traffic for all other services, the AMF becomes a natural bottleneck.

- Negative Feedback Loop: The architecture creates a cycle where congestion feeds itself. When the AMF is overloaded, the continuous stream of signaling required for other essential services (like activating PDU session,  setting up a user plane) only worsens the congestion.

- Degraded Quality of Experience: Ultimately, any congestion or failure at the AMF inevitably cascades to all dependent services. This directly impacts the end-user, causing delays, failed service attempts, and an overall unreliable network experience.

In short, the tight coupling of mobility and session-related signaling at the AMF creates a fragile system where a single point of failure can impact the entire network.

.

Technical Impact: How would this WT/KI influence:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

**** First Change ****

# Annex A.X. WT#1.1 Scope

WT 1.1 - support for control signalling for 6G for connectivity services and/or beyond connectivity services:

1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

i)   Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

ii).	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

**** Second Change ****

# 5.X. Key Issue #X (1.1): support for control signalling for 6G for connectivity services and/or beyond connectivity services.

This key issue (KI) will study to cover the following aspects:

- 	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities

- 	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

**** End of Changes ****