SA WG2 Meeting #170	S2-2507379

25 – 29 Aug, 2025, Goteborg , SE

Source:	NEC

Title:	[WT#1.2] User Plane architecture

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides scope for WT #1.2 for supporting enhancement of user plane architecture and proposes corresponding KI description.

# 1. Justifications

The 6G SID approved at TSG SA #108 has the following text in WT #1.2 for supporting essential capabilities and high-level functionalities for overall 6G architecture.

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Architectural Motivation

In Current 5GS architecture, the CU-UP and UPF are designed as a separate entity for PLMN, Non-Public Network (NPN), Multi-Access Edge Computing (MEC). The UPF deployment are distributed which is best suited for the large scale deployment of 5GS e.g. in a PLMN providing services to the user through out larger geographical area (country wide). It is also observed that the CU are moving closer to the core network.

Gap Analysis

For NPN more specifically SNPN or CIoT where the UE is basically stationary, there is no technical justification why UPF and CU-UP needs to be separately assigned even UPF and CU-UP seldom change for the life time of PDU Session. I.e. Use of both F1-U tunnel and N3 tunnel is very inefficient in respect of network resource usage in this deployment case. In addition, the procedure to setup two tunnels will take a lot of time and signnalling in the network. The user plane packet encapsulated and decapsulated unnecessary which requires extra processing and overheads in this case. The management function to maintain the GTP-U tunnel is not required which brings extra complexity.

Technical Impact

How would this WT/KI influence:

1.2.	Study whether and how to enhance the following aspects in 6G: user plane architecture.

**** First Change ****

# Annex A.1.2.X WT#1.2 Scope for user plane architecture

WT 1.2 covers whether and how to enhance user plane architecture in 6GS.

**** Next Change (all text is new)****

# 5.X. Key Issue #1.2.X: Support for network slicing

This Key Issue aims to study whether and how to enhance user plane architecture in 6GS.

**** End of Changes ****