3GPP SA WG2 Meeting #170	S2-2507380

25 - 29 August, 2025, Goteborg, Sweden

Source:	Nokia

Title:	Discussions on CT3 Question 1 in S2-2506127/C3-252543

Document for:	Discussion

Agenda Item:	19.9.2

Work Item / Release:	eEDGE_5GC_Ph3 / Rel-19

Abstract of the contribution: This paper proposes a discussion and a way forward regarding CT3 Question 1 in C3-252543.

# 1	Discussion

In C4-250555, CT4 asked SA3 to clarify the security related protocol-specific configuration parameters that PFCP should support providing to the UPF over N4.

In S3-251667, SA3 provided required security parameters that are part of protocol-specific parameters in N6 delay measurement assistance information in Clause 6.2.3.4 (EAS Deployment Information Management) of TS 23.548. The required security parameters are provided for the example protocols highlighted in clause 5.8.2.23 of TS 23.501 (please see below Table 1 excerpted from S3-251667 for convenience).

Table 1: Exchanged security and configuration parameters to enable security mechanisms in S3-251667

|  | OWAMP (RFC 4656) | TWAMP (RFC 5357) | STAMP (RFC 8762) |
| --- | --- | --- | --- |
| Security related parameters | Shared secret Security mode | Shared secret Security mode | Shared Secret Security mode |

Observation 1: As per SA3 reply LS to CT4 in S3-251667, the protocol-specific security parameters (as captured in Table 1 above) are provided as part of protocol-specific configuration parameters.

In Clause 5.8.2.23 of TS 23.501, it is clarified that AF would trigger N6 delay measurement by providing Indication of considering N6 delay measurement in AF traffic influence (described in clause 5.6.7 of TS 23.501), and SMF would request candidate UPF(s) to perform the N6 delay measurement by including measurement endpoint address, measurement protocol and protocol-specific configuration parameters that are provided by AF via EAS Deployment Information (described in clause 6.2.3.4 of TS 23.548).

Observation 2: N6 delay measurement assistance information as part of EAS Deployment Information is provided by AF and includes measurement endpoint address, measurement protocol and protocol-specific configuration parameters.

In this meeting, CT3 has asked SA2 in C3-252543 to clarify whether the protocol-specific configuration parameters contain the contents in S3-251667 / Table 1 above. Based on the observations 1 and 2 above, it is clarified that the protocol-specific configuration parameters include security parameters, and the protocol-specific configuration parameters are provided by AF as part of EAS Deployment Information.

Proposal 1: Answer the Question 1 in CT3 LS in C3-252543 to confirm that protocol-specific configuration parameters contain the contents that are described in the CT4 LS reply in S3-251667 as proposed in S2-2507381.

# 2	Proposals

Proposal 1: Answer the Question 1 in CT3 LS in C3-252543 to confirm that protocol-specific configuration parameters contain the contents that are described in the CT4 LS reply in S3-251667 as proposed in S2-2507381.