3GPP TSG SA WG2#168	            S2-2507383

Goteborg, Sweden, 07 – 11 April 2025	(revision of S2-2501062, S2-2502895)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0297 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on SMF triggered I-SMF insertion or removal for Local Offloading Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For an ongoing PDU session, PCF may send updated PCC rule(s) which may include target DNAI or DNAI(s) for the PDU session by invoking Npcf_ SMPolicyControl_UpdateNotify to SMF based on either internal mechanisms or an AF request. This is described in clause 5.34.3 of TS 23.501 as follows: According to the PCC rules related with AF influence traffic mechanism regarding DNAI(s), the SMF determines the target DNAI which is applicable to the current UE location and which can be based on the common DNAI (if applicable) as described in TS 23.548 [130]. If current (I-)SMF cannot serve the target DNAI or if the SMF can serve the target DNAI and existing I-SMF is not needed, the SMF may send the target DNAI information to the AMF for triggering I-SMF (re)selection or removal, e.g. the AMF performs I-SMF (re)selection or removal based on the target DNAI and supported DNAI(s) of (I-)SMF. If the SMF determines that target DNAI currently served by I-SMF should not be used for the PDU Session hence the existing I-SMF is not needed (e.g. due to the updated PCC rules removes DNAI(s) that was provided in the previous PCC rules), the SMF sends the target DNAI information without including target DNAI to AMF, which may trigger the I-SMF removal. Such procedure should be handled for Local Offloading Management scenario to not to interfere with the existing procedures. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifies SMF triggered I-SMF selection or removal for Local Offloading Management |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification related to Local Offloading Management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.10.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5568 TS 23.501 CR 6420 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev02: changes submitted to SA2#170 are highlighted in yellow. |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.10.2.X	SMF triggered I-SMF insertion/change/removal for Local Offloading Management

For a PDU session supporting Local Offloading Management, handling of updated PCC rule(s) which may include target DNAI or DNAI(s) for the PDU session is executed same as the respective SMF triggered I-SMF selection or removal defined in clause 4.23.5.4 of TS 23.502 [3] with the following difference:

- At step 1, when SMF receives Npcf_SMPolicyControl_UpdateNotify with a target DNAI, SMF decides whether proceeding to step 2 is required or not based on implementation.

NOTE: 	It is SMF logic to consider whether the exiting I-SMF is inserted to support Local Offloading Management and to take into account whether the UE has other application(s) running on the same PDU session towards different DNAI(s). In case the SMF decides not to change to the target DNAI provided in the PCC rule, the SMF may issue a failure notification.

- At step 2, when the content of Nsmf_PDUSession_SMContextStatusNotify or Nsmf_PDUSession_StatusNotify message includes the target DNAI information, the existing I-SMF of the PDU session is removed/(re)-selected only if:

  - AMF decides that the target DNAI is not supported by the existing I-SMF, or

  - AMF receives a target DNAI information without target DNAI.

The target DNAI, if included, is used for selecting I-SMF at step 3.

- At step 3,

  - the AMF selects a new I-SMF which serves the target DNAI, supports Local Offloading Management, serves the UE location and local offloading management service area, or

  - the AMF removes the I-SMF if the AMF receives a target DNAI information without including the target DNAI.

In case if the AMF cannot determine a new I-SMF that supports Local Offloading Management and local offloading management service area based on the received target DNAI, the AMF performs the procedure defined in clause 4.23.5.4 of TS 23.502 [3] from step 3 onwards. The remaining below steps in this clause are skipped.

- At step 4, the AMF also includes Local Offloading Allowed indication to the new I-SMF as part of Nsmf_PDUSession_CreateSMContext Request.

- At step 5 and 6, the procedure defined in clause 6.10.2.4 for I-SMF insertion/change/removal during UE mobility is used with the following difference:

  - For I-SMF insertion or I-SMF change case, the I-SMF selects the I-UPF based on the target DNAI,

  - For I-SMF removal case, the SMF selects a new I-UPF based on the target DNAI.

End of CHANGES