3GPP TSG SA WG2#170	            S2-2507394

Goteborg, Sweden, 25 – 29 August 2025	(revision of S2-2505366)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | zzzz | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Service Experience and DN performance analytics with N6 delay measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | DUMMY, TEI20 |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In clause 6.9 of TS 23.548, it is noted that before a UE sends a DNS query, the SMF may create or update the DNS message handling rule on EASDF by considering N6 delay measurements. In clause 6.2.3.2.2 of TS 23.548, it is noted that SMF may determine the DNAI based on EAS information received from the EASDF in Neasdf_DNSContext_Notify, other UPF selection criteria and possibly Service Experience or DN performance analytics for an Edge Application as described in TS 23.288. Using N6 delay measurements as inputs and obtaining N6 delay statistics / predictions can be useful in determining the DN performance and the Quality of Experience analytics. The SMF may use these analytics to determine the UPF and DNAI that offers the best user plane performance or ECS option at EASDF to improve DNS query resolution. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Includes N6 delay measurements to input/output data of Service Experience and DN performance analytics for SMF to (re)select UPF/DNAI or create/update DNS message handling rule on EASDF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not utilizing the existing capability/information for better performing system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.4.2, 6.4.4, 6.14.2, 6.14.3, 6.14.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

… Omitted for simplicity

[56]	3GPP TS 28.406: "Quality of Experience (QoE) measurement collection; Information definition and transport".

[X]	3GPP TS 23.548: "5G System Enhancements for Edge Computing; Stage 2".

SECOND CHANGE

### 6.4.2	Input Data

The service data and performance data collected from the AF (including the service data collected from the UE through the AF), the network data from other 5GC NFs and the network data from OAM and MDAS/MDAF for observed service experience are defined in Table 6.4.2-1, 6.4.2-1a, Table 6.4.2-2, Table 6.4.2-3, Table 6.4.2-4 and Table 6.4.2-5 respectively.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| IP filter information | AF | Identify a service flow of the UE for the application |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAIs. The NEF may map the AF-Service-Identifier information to a list of DNAI when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF | Refers to the QoE per service flow as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS for any kind of service including those not related to video or voice. |
| UE ID | AF | The list of UE IDs that are associated with the Service Experience value(s). When the AF is untrusted, GPSIs will be provided. When the AF is trusted SUPIs will be provided. |
| Service Experience Contribution Weight | AF | The list of Service Experience Contribution Weights that are associated with each of the provided UE IDs. |
| QoE metrics | UE (via AF) | QoE metrics observed at the UE(s). QoE metrics and measurement as described in TS 26.114 [27], TS 26.247 [28], TS 26.118 [29], TS 26.346 [30], TS 26.512 [31] or ASP specific QoE metrics in TS 26.512 [31], as agreed in the SLA with the MNO, may be used. |
| Timestamp | AF | A time stamp associated to the Service Experience provided by the AF, mandatory if the Service Experience is provided by the ASP. |
| Application Server Instance | AF | The IP address or FQDN of the Application Server that the UE had a communication session when the measurement was made. |

NWDAF subscribes to the service data from AF in the Table 6.4.2-1 either directly for trusted AFs by invoking Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Event Filter information = Area of Interest, Application ID) as defined in TS 23.502 [3], or indirectly for untrusted AFs via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = Service Experience information, Event Filter information = Area of Interest, Application ID) where NEF translates the Area of Interest into geographic zone identifier(s). For the information whose source is UE (via AF), the AF collects data from the UE as defined in clause 6.2.8.

NOTE:	When the Service Experience is expressed as a customized MOS, the customized MOS might be defined by the content provider or by the MNO and might be based on the nature of the targeted service type (e.g. web browsing, gaming, augmented reality, V2X, SMS).

Table 6.4.2-1a: Performance Data from AF

| Information | Source | Description |
| --- | --- | --- |
| UE identifier | AF | IP address of the UE at the time the measurements was made. |
| UE location | AF | The location of the UE when the performance measurement was made. |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service). |
| IP filter information | AF | Identify a service flow of the UE for the application. |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAIs. The NEF may map the AF-Service-Identifier information to a list of DNAIs when the DNAIs being used by the application are statically defined. |
| Application Server Instance address | AF/NEF | The IP address/FQDN of the Application Server that the UE had a communication session when the measurement was made. |
| Performance Data | AF | The performance associated with the communication session of the UE with an Application Server that includes: Average Packet Delay, Average Loss Rate and Throughput. |
| Timestamp | AF | A time stamp associated to the Performance Data provided by the AF. |

NWDAF subscribes to the performance data from AF in the Table 6.4.2-1a either directly for trusted AFs by invoking Naf_EventExposure_Subscribe service (Event ID = Performance Data, Event Filter information = Area of Interest, Application ID) as defined in TS 23.502 [3], or indirectly for untrusted AFs via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = Performance Data, Event Filter information = Area of Interest, Application ID) where NEF translates the Area of Interest into geographic zone identifier(s).

Table 6.4.2-1b: QoE measurements from OAM related to specific service type

| Information | Source | Description |
| --- | --- | --- |
| UE ID/Area scope | OAM | The UE Identity (i.e., qoETarget ("IMSI" or "SUPI") or areaScope as defined in TS 28.405 [55]) that are associated with the QoE measurements. |
| Service Type | OAM | Identify the service type for the QoE measurements, i.e., Dynamic Adaptive Streaming over HTTP (DASH), Multimedia Telephony Service for IMS (MTSI) or Virtual Reality (VR). (NOTE) |
| Area scope | OAM | Areas of the UEs with the QoE measurements. |
| QoE metrics | OAM | QoE metrics observed at the UE(s). QoE metrics and measurement as described in TS 28.406 [56]. |
| Timestamp | OAM | A time stamp associated to the QoE measurements data collection provided by the UE. |
| NOTE: The NWDAF maps the Application ID into a Service Type that is understood by OAM. |  |  |

NWDAF subscribes the network data from OAM in the Table 6.4.2-1b by using the services provided by OAM as described in clause 6.2.3.

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| Finer granularity location (1...max) | GMLC | UE positions. |
| ....>UE location |  | GAD shape or location coordinates (see TS 23.032 [34]). |
| ....>Timestamp |  | A time stamp when the location was measured. |
| ....>LCS QoS |  | The accuracy of the measurement. |
| UE ID | AMF | List of SUPIs. If UE IDs are not provided as Target of Analytics Reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| DNAI | SMF | Identifies the access to DN to which the PDN session connects. |
| PDU Session type | SMF | Type of the PDU Session. |
| SSC Mode | SMF | SSC Mode selected for the PDU Session. |
| Access Type | SMF | List of Access Types used for the PDU Session. |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Packet transmission | UPF | The observed number of packet transmission. |
| Packet retransmission | UPF or AF | The observed number of packet retransmission. |
| N6 delay measurements | SMF/UPF | The measured N6 delay measurements between the candidate UPF(s) and measurement endpoint in the DN(s). |

NOTE:	Care needs to be taken with regards to load and major signalling caused when requesting Any UE. This could be achieved via utilization of some event filters (e.g. Area of Interest for AMF), Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

NWDAF subscribes to the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation with the following Event IDs as input parameters:

-	AMF Source: Namf_EventExposure_Subscribe (Event IDs = Location Changes, Area of Interest).

-	SMF Source: Nsmf_EventExposure_Subscribe (Event ID = QFI allocation).

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM (see NOTE 1) | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15]. |
| Reference Signal Received Quality | OAM (see NOTE 1) | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15]. |
| Signal-to-noise and interference ratio | OAM (see NOTE 1) | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12]. |
| RAN Throughput for DL and UL | OAM (see NOTE 1) | The per UE measurement of the throughput for DL and UL as specified in clauses 5.2.1.1 and 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet delay for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet delay for DL and UL, including per QCI per UE packet delay as specified in clause 5.2.1.1 of TS 37.320 [20] and per DRB per UE packet delay as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| RAN Packet loss rate for DL and UL | OAM (see NOTE 1) | The per UE measurement of the packet loss rate for DL and UL, including the per QCI per UE packet loss rate as specified in clause 5.2.1.1 of TS 37.320 [20] and the per DRB per UE packet loss rate as specified in clause 5.4.1.1 of TS 37.320 [20]. |
| The mapping information between cell ID and frequency | OAM | The mapping information between cell ID and frequency (NOTE 2). |
| Cell Energy Saving State | OAM | List of the cells which are within the area of interest and are in energy saving state, as specified in clauses 3.1 and 6.2 of TS 28.310 [24]. |
| NOTE 1: Per UE measurement for a specific UE from OAM (via MDT), is as captured in clause 6.2.3.1. NOTE 2: The MDT measurement report provides the cell identity and carrier frequency information for UE's serving cell and neighbour cell(s). The NWDAF can get the mapping information between cell ID and frequency using OAM service as described in clause 6.2.3. |  |  |

NWDAF subscribes the network data from OAM in the Table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

Table 6.4.2-4: UE level Network Data from 5G NF related to the Service Experience

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| Finer granularity location | GMLC | UE positions. |
| UE ID | AMF | List of SUPIs. |
| RAT Type | SMF | The RAT type the UE camps on. |

The Event Filters for the service data collection from SMF, AMF and AF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

The NWDAF collects the following MDAF analysis result listed in Table 6.4.2-5, as defined in clauses 8.4.2.1.3 and 8.4.4.1.3 of TS 28.104 [45].

Table 6.4.2-5: Data collection from MDAS/MDAF of service experience and energy saving state analysis

| Information | Source | Description |
| --- | --- | --- |
| ServiceExperienceIssueType | MDAF | Indication of the service experience issue type. The allowed value is one of the enumerated values: RAN issue, CN issue, both. |
| AffectedObjects | MDAF | The managed object instances where the service experience is applicable, e.g. SubNetwork Instance, NetworkSlice Instance, S-NSSAI. |
| ServiceExperienceStatistics | MDAF | The statistics of the level of service experience for a service in a certain time period, e.g. there are five levels which are represented by 1, 2, 3, 4, 5 where level 1 represents the users are enduring bad experience while level 5 represents the users' requirements are perfectly satisfied. |
| ServiceExperiencePredictions | MDAF | The predictions of the level of service experience for a service in a certain time period. |
| StatisticsOfCellsEsState | MDAF | The statistic result of current energy saving state of the cells at a certain time. |

THIRD CHANGE

### 6.4.4	Procedures to request Service Experience for an Application

Figure 6.4.4-1: Procedure for NWDAF providing Service Experience for an Application

This procedure allows the consumer to request Analytics ID "Service Experience" for a particular Application. The consumer includes both the Application ID for which the Service Experience is requested and indicates that the Target of Analytics Reporting is "any UE". If the Target for Analytics Reporting is either a SUPI or an Internal-Group-Id the procedure in clause 6.4.6 applies. At the same time, for an Application ID, a set of initial QoS parameter combinations per service experience window (e.g. one is for 3<Service MOS<4 and another is for 4<Service MOS<5) is defined in PCF (e.g. by configuration of operator policies) that may be updated based on the Service Experience reported by NWDAF.

1.	Consumer NF sends an Analytics request/subscribe (Analytics ID = Service Experience, Target of Analytics Reporting = any UE, Analytics Filter Information that may include one or more of the following as defined in Table 6.4.1-1 (Application ID, S-NSSAI, DNN, Application Server Address(es), Area of Interest, RAT type(s), Frequency value(s)), Analytics Reporting Information=Analytics target period) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe.

2a.	The NWDAF may subscribe to the service data from AF in the Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Application ID, Event Filter information, Target of Event Reporting = Any UE) as defined in TS 23.502 [3].

NOTE 1:	In the case of trusted AF, NWDAF provides the Area of Interest as a list of TAIs to AF. In the case of untrusted AF, NEF translates the requested Area of Interest provided as event filter by NWDAF into geographic zone identifier(s) that act as event filter for AF.

2b.	If the NWDAF maps the provided Application ID is DASH, MTSI or VR in step 1, the NWDAF may act as an MnS Consumer to collect the QoE measurements data for all the UEs in the interested area from OAM as defined in TS 28.404 [54], TS 28.405 [55] and TS 28.406 [56].

2c.	The NWDAF subscribes the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation.

2d. With these data, the NWDAF estimates the Service experience for the application.

NOTE 2:	Care needs to be taken to avoid end to end excessive signalling for collecting the QoE measurements from the applications in the UE.

3.	The NWDAF provides the data analytics, i.e. the observed Service Experience (which can be a range of values) to the consumer NF by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1, indicating how well the used QoS Parameters satisfy the Service MoS agreed between the MNO and the end user or between the MNO and the external ASP.

NOTE 3:	The call flow only shows a request-response model for the interaction of NWDAF and consumer NF for simplicity instead of both request-response model and subscription-notification model.

NOTE 4:	The non-real time data information from AF includes the service experience data (see Table 6.4.2-1), which indicates the service quality during the service lifetime.

If the consumer NF is a PCF and it determines that the application SLA is not satisfied, it may take into account the Observed Service Experience and the operator policies including SLA and required Service Experience (which can be a range of values) to determine new QoS parameters to be applied for the service, as defined in clause 6.1.1.3 and clause 6.2.1.2 of TS 23.503 [4].

If the consumer NF is an AF (e.g. MEC or other Application Server), it may use the Observed Service Experience related network data analytics to determine whether the user experience can be satisfied. If not, the AF may determine to adjust service parameters, e.g. for a video service this may be bit rate, frame rate, codec format, compression parameter, screen size, etc. to better match the network conditions and achieve better user experience.

If the consumer NF is SMF, PCF or AF/Application Server, it may take into account the Observed Service Experience analytics per UP path (i.e. UPF and/or DNAI and/or AS instance address as defined in Table 6.4.3-1) to perform the following procedures:

-	The consumer SMF determines to (re)selects UP paths, including UPF and DNAI, as described in clause 4.3.5 of TS 23.502 [3] or ECS option as described in clause 6.2.3.2.2 of TS 23.548 [X]. In addition, the SMF may (re)configure traffic steering, updating the UPF regarding the target DNAI with new traffic steering rules or (re)configure DNS handling rules at EASDF.

-	The consumer AF/Application Server determines to adjust service parameters, e.g. service parameters of video for adjustment may be bit rate, frame rate, codec format, compression parameter, screen size, etc. or service parameters for the AI/ML operations described in clause 6.40 of TS 22.261 [33]. In addition, the AF/ Application Server may provide an updated list of DNAI(s) for SMF to perform relocation when appropriate.

-	The consumer PCF may provide an updated list of DNAI(s) for SMF to perform relocation upon AF request.

If the consumer NF is a NEF, it may take into account the Observed Service Experience analytics to support Member UE selection as detailed in clause 4.15.13 of TS 23.502 [3].

FIFTH CHANGE

### 6.14.2	Input Data

The data collected from the AF are defined in table 6.14.2-1

Table 6.14.2-1: Performance Data from AF

| Information | Source | Description |
| --- | --- | --- |
| UE identifier | AF | IP address of the UE at the time the measurements was made. |
| UE location | AF | The location of the UE when the performance measurement was made. |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service). |
| IP filter information | AF | Identify a service flow of the UE for the application. |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Application Server Instance address | AF/NEF | The IP address/FQDN of the Application Server that the UE had a communication session when the measurement was made. |
| UL/DL Performance Data | AF | The performance associated with the communication session of the UE with an Application Server that includes: UL/DL Average/Maximum Packet Delay, UL/DL Average/Maximum Loss Rate and UL/DL Average/Minimum/Maximum Throughput. |
| Timestamp | AF | A time stamp associated to the Performance Data provided by the AF. |

The data collected by the SMF or UPF are described in table 6.4.2-2.

The NWDAF subscribes to network data as defined in clause 6.4.2.

Data may be collected from OAM as described in table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

The Event Filters for the service data collection from SMF, AMF and AF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

SIXTH CHANGE

### 6.14.3	Output Analytics

The DN performance analytics is shown in table 6.14.3-1 and table 6.14.3-2.

Table 6.14.3-1: DN service performance statistics

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See note 1. |
| DNN | Identifies the data network name (e.g. "internet") for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performances for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF info | The UPF ID/address/FQDN information for the involved anchor UPF. See NOTE 1. |
| > Candidate UPF Info | Indicating the candidate UPF information (e.g., UPF ID/address/FQDN) that can serve the UE. See NOTE 1. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance (NOTE 4) | Performance indicators. |
| >> Aggregated Traffic rate (NOTE 2, NOTE 5) | Aggregated traffic rate observed for the UE group or all UEs (i.e. any UE) communicating with the application. |
| >> Average Traffic rate (NOTE 2) | UE granularity level Average traffic rate observed for the UE group or all UEs communicating with the application, or Average traffic rate observed for the specific UE in the statistics period. |
| >> Maximum Traffic rate (NOTE 2) | UE granularity level Maximum traffic rate observed for the UE group or all UEs communicating with the application, or Maximum Traffic rate observed for the specific UE in the statistics period. |
| >> Minimum Traffic rate (NOTE 3) | UE granularity level Minimum traffic rate observed for the UE group or all UEs communicating with the application, or Minimum Traffic rate observed for the specific UE in the statistics period. |
| >> Variance Traffic rate (NOTE 2, NOTE 5) | UE granularity level Variance of the traffic rate observed for the UE group or all the UEs communicating with the application, or Variance Traffic rate observed for the specific UE in the statistics period. |
| >> UE ID or list of UE IDs for traffic rate performance (1..SUPImax) (NOTE 2, NOTE 5) | Identifies a UE or a list of UEs whose observed traffic rate is higher or lower than the Reporting Threshold. |
| >> Average Packet Delay (NOTE 2) | Average packet delay observed for the specific UE, the UE group or all UEs communicating with the application. |
| >> Maximum Packet Delay (NOTE 2) | Maximum packet delay observed for the specific UE, the UE group or all UEs communicating with the application. |
| >> Variance Packet Delay (NOTE 2, NOTE 5) | Variance of packet delay observed for the specific UE, the UE group or all UEs communicating with the application. |
| >> UE ID or list of UE IDs for packet delay performance (1..SUPImax) (NOTE 2, NOTE 5) | Identifies a UE or a list of UEs whose observed packet delay is higher or lower than the Reporting Threshold. |
| >> Average Packet Loss Rate (NOTE 2) | Average packet loss observed for the specific UE, the UE group or all UEs communicating with the application. |
| >> Maximum Packet Loss Rate (NOTE 2) | Maximum packet loss observed for the specific UE, the UE group or all UEs communicating with the application. |
| >> Variance Packet Loss Rate (NOTE 2, NOTE 5) | Variance of packet loss rate observed for the specific UE, the UE group or all UEs communicating with the application. |
| >> UE ID or list of UE IDs for packet loss performance (1..SUPImax) (NOTE 2, NOTE 5) | Identifies a UE or a list of UEs whose observed packet loss rate is higher than the Reporting Threshold. |
| >> Number of UEs (NOTE 2) | The observed number of UEs for the UE group or all UEs (i.e. any UE) communicating with the application in the DNAI. |
| >> Average N6 delay (NOTE 2) | Average N6 delay observed for the specific UPF or candidate UPF(s) having an N6 connection with the application server instance/DNAI. |
| >> Maximum N6 delay (NOTE 2) | Maximum N6 delay observed for the specific UPF or candidate UPF(s) having an N6 connection with the application server instance/DNAI. |
| >> Minimum N6 delay (NOTE 2) | Minimum N6 delay observed for the specific UPF or candidate UPF(s) having an N6 connection with the application server instance/DNAI. |
| >> Variance N6 delay (NOTE 2) | Variance N6 delay observed for the specific UPF or candidate UPF(s) having an N6 connection with the application server instance/DNAI. |
| > Spatial Validity Condition | Area (i.e. list of TAIs) where the DN performance analytics applies. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. If a Temporal granularity size was provided in the request or subscription, the duration of this period is greater than or equal to the Temporal granularity size. |
| NOTE 1: The item "Serving anchor UPF info" and “Candidate UPF Info” shall not be included if the consumer NF is an AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 3: Minimum traffic rate measurements are only derived from active traffic. NOTE 4: Performance statistics may not be applicable to short group operation cycle for the given application. NOTE 5: Analytics subset that can be used to support aggregated UE performance monitoring and exposure for a group of UEs. |  |

Table 6.14.3-2: DN service performance predictions

| Information | Description |
| --- | --- |
| Application ID | Identifies the application for which analytics information is provided. |
| S-NSSAI | Identifies the Network Slice for which analytics information is provided. See NOTE 1. |
| DNN | Identifies the data network name (e.g. internet) for which analytics information is provided. See NOTE 1. |
| DN performance (0-x) | List of DN performance for the application. |
| > Application Server Instance Address | Identifies the Application Server Instance (IP address/FQDN of the Application Server). |
| > Serving anchor UPF info | The UPF ID/address/FQDN information for the involved anchor UPF. See NOTE 1. |
| > Candidate UPF Info | Indicating the candidate UPF information (e.g., UPF ID/address/FQDN) that can serve the UE. See NOTE 1. |
| > DNAI | Identifier of a user plane access to one or more DN(s) where applications are deployed as defined in TS 23.501 [2]. |
| > Performance | Performance indicators. |
| >> Aggregated Traffic rate (NOTE 2) | Aggregated traffic rate predicted for the UE group or all UEs (i.e. any UE) communicating with the application. |
| >> Average Traffic rate (NOTE 2) | UE granularity level Average traffic rate predicted for the UE group or all UEs communicating with the application, or Average traffic rate predicted for the specific UE in the prediction period. |
| >> Maximum Traffic rate (NOTE 2) | UE granularity level Maximum traffic rate predicted for the UE group or all UEs communicating with the application, or Maximum Traffic rate predicted for the specific UE in the prediction period. |
| >> Minimum Traffic rate (NOTE 3) | UE granularity level Minimum traffic rate predicted for the UE group or all UEs communicating with the application, or Minimum Traffic rate predicted for the specific UE in the prediction period. |
| >> Variance Traffic rate (NOTE 2) | UE granularity level Variance of the traffic rate predicted for the UE group or all the UEs communicating with the application, or Variance Traffic rate predicted for the specific UE in the prediction period. |
| >> Average Packet Delay (NOTE 2) | Average packet delay predicted for the specific UE, the UE group or all UEs communicating with the application. |
| >> Maximum Packet Delay (NOTE 2) | Maximum packet delay for predicted for the specific UE, the UE group or all UEs communicating with the application. |
| >> Variance Packet Delay (NOTE 2) | Variance of packet delay predicted for the specific UE, the UE group or all UEs communicating with the application. |
| >> Average Packet Loss Rate (NOTE 2) | Average packet loss predicted for the specific UE, the UE group or all UEs communicating with the application. |
| >> Maximum Packet Loss Rate (NOTE 2) | Maximum packet loss predicted for the specific UE, the UE group or all UEs communicating with the application. |
| >> Variance Packet Loss Rate (NOTE 2) | Variance of packet loss rate predicted for the specific UE, the UE group or all UEs communicating with the application. |
| >> Number of UEs (NOTE 2) | The predicted number of UEs for the UE group or all UEs (i.e. any UE) communicating with the application in the DNAI. |
| >> Average N6 delay (NOTE 2) | Average N6 delay observed for the specific UPF or candidate UPF(s) having an N6 connection with the application server instance/DNAI. |
| >> Maximum N6 delay (NOTE 2) | Maximum N6 delay observed for the specific UPF or candidate UPF(s) having an N6 connection with the application server instance/DNAI. |
| >> Minimum N6 delay (NOTE 2) | Minimum N6 delay observed for the specific UPF or candidate UPF(s) having an N6 connection with the application server instance/DNAI. |
| >> Variance N6 delay (NOTE 2) | Variance N6 delay observed for the specific UPF or candidate UPF(s) having an N6 connection with the application server instance/DNAI. |
| > Spatial Validity Condition | Area (i.e. list of TAIs) where the DN performance analytics applies. If a Spatial granularity size was provided in the request or subscription, the number of elements of the list is smaller than or equal to the Spatial granularity size. |
| > Temporal Validity Condition | Validity period for the DN performance analytics. If a Temporal granularity size was provided in the request or subscription, the duration of this period is greater than or equal to the Temporal granularity size. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: The item "Serving anchor UPF info" shall not be included if the consumer is an AF. NOTE 2: Analytics subset that can be used in "list of analytics subsets that are requested", "Preferred level of accuracy per analytics subset" and "Reporting Thresholds". NOTE 3: Minimum traffic rate measurements are only derived from active traffic. |  |

SEVENTH CHANGE

### 6.14.4	Procedures to request DN Performance Analytics for an Application

Figure 6.14.4-1: Procedure for NWDAF providing DN Performance analytics for an Application

The procedure illustrated in figure 6.14.4-1 allows an analytics consumer to request Analytics ID "DN Performance" for a particular Application. The analytics consumer includes the Application ID for which DN Performance is requested. The consumer indicates the Target of Analytics Reporting (e.g. "any UE") and may include as Analytic Filter Information the UPF anchor ID, DNAI, or Application Server instance that DN performance analytics are requested.

1.	Analytics consumer sends an Analytics request/subscribe (Analytics ID = DN Performance Target of Analytics Reporting, Analytics Filter Information = (Application ID, S-NSSAI, DNN, Area of Interest, UPF anchor ID, DNAI, Application Server Address(es)), Analytics Reporting Information = Analytics target period) to NWDAF by invoking a Nnwdaf_AnalyticsInfo_Request or a Nnwdaf_AnalyticsSubscription_Subscribe service.

2a.	NWDAF subscribes to the performance data from AF defined in table 6.14.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = Performance Data, Application ID, Event Filter information), Target of Event Reporting = Any UE) as defined in TS 23.502 [3].

NOTE 1:	In the case of trusted AF, NWDAF provides the Area of Interest as a list of TAIs to AF. In the case of untrusted AF, NEF translates the requested Area of Interest provided as event filter by NWDAF into geographic zone identifier(s) that act as event filter for AF.

2b.	NWDAF subscribes to the network data from 5GC NF(s) defined in table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service.

2c.	With the collected data, the NWDAF estimates the DN Performance for the application.

3.	NWDAF provides the data analytics, to the analytics consumer by means of either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

NOTE 2:	For simplicity, the call flow only shows a request-response model for the interaction of NWDAF and analytics consumer instead of both request-response model and subscription-notification model.

If the analytics consumer is an SMF, the SMF may use the analytics to determine the UPF and DNAI that offers the best user plane performance or ECS option at EASDF to improve DNS query resolution.

If the analytics consumer is an AF, the AF may use the analytics to determine the DNAI that has the best user plane performance if Application Server relocation is required.

End of CHANGES