3GPP TSG-SA WG2#170	S2-2507396

Gothenburg, Sweden, 25-29 August 2025	(was S2-2504979)

Source:	Qualcomm Incorporated

Title:	KI#3: Support for emergency calls over NB-IoT NTN

Document for:	Discussion/Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: Solution to address the support for emergency calls over NB-IoT NTN.

# 1.	Discussion

Proposes changes in the procedures to support IMS emergency calls over NB-IoT NTN connected to EPC.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-19:

# >>>>BEGINNING OF CHANGES<<<<

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G system, Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System".

[5]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[6]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[7]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[8]	3GPP TS 22.261: "Service requirements for the 5G system".

[9]	3GPP TS 22.101: "Service aspects; Service principles".

[10]	3GPP TS 29.211: "Rx Interface and Rx/Gx signalling flows".

[11]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[12]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[13]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[14]	3GPP TS 24.294: "IP Multimedia Subsystem (IMS) Centralized Services (ICS) protocol via I1 interface".

[15]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[16]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[17]	3GPP TS 23.203: " Policy and charging control architecture".

[18]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[19]	IETF RFC 4566: "SDP: Session Description Protocol".

[20]	IETF RFC 5049: "Applying Signaling Compression (SigComp) to the Session Initiation Protocol (SIP)".

[y]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1".

# >>>>NEXT CHANGE (new text)<<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  |  | X |  |

# >>>>NEXT CHANGE (new text)<<<<

## 6.X	Solution #X: Support for emergency services over NB-IoT NTN

### 6.X.1	High level principles

This solution has the following high level principles, in order to support emergency services over NB-IoT NTN connected to EPC:

- 	The only emergency service supported over NB-IoT NTN connected to EPC is for IMS voice;

-	MME needs to include an indication for Emergency Services Support within the NAS Accept to the UE also for UE accessing over NB-IoT NTN;

-	eNB needs to support the related broadcast indicator that the cell supports Emergency Services over NB-IOT NTN in order to support UEs in limited service state. Unlike TN where there is a single emergency support indicator per cell, since it is likely that the cell will be serving multiple PLMNs from different countries, the support indicator is preferrable per PLMN.

-	Regarding emergency domain selection:

-	If the UE is registered, use the current PLMN in any RAT including NB-IOT NTN if emergency services are supported.

-	If the UE is not registered, the UE uses either a PLMN 3GPP access (with emergency support), or an SNPN (with emergency support), and if neither are available the UE uses a PLMN non-3GPP access, if available.

### 6.X.2	Description

This is a solution for Key issue #3 "Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC".

The service requirements for support of emergency services are documented in TS 22.101 [9], TS 22.228 [y], and TS 22.261 [8] and define requirements only for emergency IMS voice calls.

Based on the above service requirements the following impacts for support of IMS emergency services by PLMN over NB-IoT NTN are foreseen for the specifications under SA WG2 control:

-	MME needs include an indication for Emergency Services Support within the NAS Accept to the UE also for UE accessing over NB-IoT NTN;

-	eNB needs to support related broadcast indicator that the cell supports Emergency Services over NB-IOT NTN in order to support UEs in limited service state. Unlike TN where there is a single emergency support indicator per cell, since it is likely that the cell will be serving multiple PLMNs from different countries, the support indicator is preferrable per PLMN.

The functionality that is already described in TS 23.401 [5] clause 4.3.12 for WB-EUTRAN access, need to apply NB-IoT and UE operating supporting emergency services over NB-IoT NTN. Therefore the following text in TS 23.401 [5] clause 4.3.12: “Support for emergency bearer services is not available when the UE is using NB-IoT, i.e. the MME shall not indicate support for emergency bearer services using the Emergency Service Support indicator in the Attach and TAU procedures to a UE that accesses the network using a RAT Type equal to NB-IoT, and an NB-IoT cell shall not indicate support for emergency services in any broadcast information in AS.” Needs to be modified.

For selection and priority where emergency services the principle should be following the same logic like in current TS 23.167 [7]. Given the UE may or may not be registered in PLMN at the time of initiating emergency services. The following are proposed:

-	If the UE is registered, use the current PLMN in any RAT including NB-IOT NTN if emergency services are supported.

-	If the UE is not registered, the UE uses either a PLMN 3GPP access (with emergency support), or an SNPN (with emergency support), and if neither are available the UE uses a PLMN non-3GPP access, if available.

NOTE :	The normative procedures for domain selection for emergency services can be documented eventually in Annex H in TS 23.167 [7].

### 6.X.3	Procedures

Existing procedures are reused.

### 6.X.4	Impacts to Services, Entities and Interfaces

Editor's note:	This clause captures impacts on existing 3GPP nodes and functional elements.

UE

-	Support procedures described in TS 23.401 [5] for emergency services.

-	Perform domain selection as described in clause 6.x.2.

MME

-	Include an indication for Emergency Services Support within the Attach and TAU Accept to the UE.

-	Needs Emergency configuration.

eNB of NB-IOT NTN

-	Include related broadcast indicator that the cell supports Emergency Services over NB-IoT NTN for UEs in limited service state, and if the eNB is shared by more than one PLMN, and the PLMNs do not have the same support for Emergency Services, the broadcast indicator is related to those networks that supports Emergency Services.

# >>>>END OF CHANGES<<<<