# SA WG2 Meeting #170, 																                  S2-2507398

Goteborg, Sweden, August 25 – 29, 2025

Source:	Lenovo

Title:	Using the NF Status to control NF traffic before activating energy saving state

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2/ Rel-20

Abstract of the contribution: This paper proposes a solution for KI#3 that controls how an EnergySaving State is applied on an NF without causing any service disruption or how energy related restrictions can be applied on selected NFs.

# 1.	 Discussion

The solution proposal in this contribution relates to Key Issue #3. It introduces a new solution taking advantage of NF Status attribute in the NF Profile, that allows an NF, which is selected by the OAM to enter an EnergySaving State, to process requests for existing resources or sessions but not accept new resource creation or session establishment assuring no service disruption. In addition, it allows an NF to apply energy related restrictions.

# 2. Proposals

It is proposed to adopt the following text in TR.23.700-67v0.2.0.

* * * * Start of 1st Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| 8 | X |  |  |
| 9 | X |  |  |
| 10 | X |  |  |
| 11 | X |  |  |
| 12 | X |  |  |
| 13 |  | X |  |
| 14 |  | X |  |
| 15 |  | X |  |
| 16 |  | X |  |
| 17 |  | X |  |
| 18 |  | X |  |
| 19 |  |  | X |
| 20 |  |  | X |
| 21 |  |  | X |
| 22 | X |  | X |
| 23 |  |  | X |
| x |  |  | X |

* * * End of 1st Change * * * *

* * * Start of 2nd Change (all new text)* * * *

## 6.X	Solution #X: Using the NF Status to control NF traffic before activating energy saving state

### 6.X.1	High-level solution principles

### This solution proposes the following principles to be agreed:

### Avoid discovering an NF in the 5G core, which is selected by the OAM to enter an energy saving state (as per TS 28.310) in the near future. For instance, avoid using a UPF that is selected by the OAM to enter an energy saving state as soon as it can finalize handling ongoing user plane data traffic or offload ongoing user plane data traffic to another UPF.

### Limit the energy usage of an NF, e.g., by restricting the user plane data traffic related to a UPF, or enhance the NF energy efficiency or the use of renewable energy.

## 6.X.2	Description

This solution assumes that the OAM configures the energy saving states of the network, i.e., EnergySaving /Not EnergySaving State as per TS 28.310. Once the OAM identifies and selects an NF to enter EnergySaving State, it will soon be powered off, i.e., not be able to handle any more user requests or user pane data traffic.

However, such NFs, e.g., UPFs, may already handle user data traffic, typically small amounts. At the same time the operator needs to make sure that such UPF is not further selected to handle any new request or user data traffic. To assure that there is no service disruption, there is a need to control that an NF that is going to enter an EnergySaving State, handles no user requests or user data traffic at the time instance that is powered off.

Currently, the NF Profile in TS 29.510 contains the NF Status parameter, which is responsible to provide information related to the operational status of the NF. NF Status information contains three operative status information, i.e., in where the NF is on Not EnergySaving State, including "REGISTERED", "UNDISCOVERABLE" and "CANARY_RELEASE" and one not operative status information "SUSPENDED" that deals with failures, e.g., as a result of a NF Heart-Beat failure and may trigger restoration procedures (see clause 6.2 of 3GPP TS 23.527).

In addition, the SHUTDOWN TIME although it represents a scheduled shut down it is related to the NF Status "UNDISCOVERABLE", i.e., it is present if the NF Status is set to "UNDISCOVERABLE" and represent the time until when an NF would be on "UNDISCOVERABLE" status, until it is shut down.

Although, the NF Status "UNDISCOVERABLE" cannot be adopted to represent an NF in EnergySaving State because it is inherently defined to represent an operative NF, it can be used for assuring that an NF selected and scheduled to be on an EnergySaving State is still able to process requests for existing resources or sessions but cannot accept new resource creation or session establishment. The respective SHUTDOWN TIME it can also complement "UNDISCOVERABLE" NF Status by indicating a timestamp when the NF Instance is planned to be shut down forcing the NF to offload current context or user data traffic. When a NF is shut down due to energy related reasons it needs to be reflected by indicating that the NF is on EnergySaving State.

If the SHUTDOWN TIME is not present, then the NF shall either wait for the ongoing user data traffic to be terminated or be offloaded before it can be shut down, i.e. enter an EnergySaving State.

The NF Status can also be used to limit the NF energy cost (i.e., consumption, or carbon emissions) or enhance the NF energy efficiency or the use of renewable energy by allowing an NF to be selected only by limited other NF types or NF types that belong to e.g., a certain slice, or with respect to a certain slice by adopting the "CANARY_RELEASE" and controlling the related selection conditions attribute.

### 6.X.2	Procedures

The procedures are not impacted but the NF profile information is impacted.

### 6.X.3	Impacts on Services, Entities and Interfaces

The impacts are:

NRF:

-	support the EnergySaving /Not EnergySaving State in the NF Profile.

-	enhancement of NRF logic to enable the indication of EnergySaving State when the "UNDISCOVERABLE" NF Status cause an NF to be shut down because of energy related reasons.

- 	enhancements of the selection conditions attribute related to the "CANARY_RELEASE" of the NF Status.

OAM:

-	support the enhancements to handle the changes introduced in the "UNDISCOVERABLE" NF Status.

- 	support the enhancements of the selection conditions attribute related to the "CANARY_RELEASE" of the NF Status.

* * * * End of Changes * * * *