3GPP TSG-SA2 Meeting #170	S2-2507404

# 25 – 29 August 2025, Goteborg, Sweden

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | Draft | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure of sending Rate Signal to UE's application using IETF's SCONE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Meta, Google, Interdigital Inc, ??? |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | 20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Many applications are capable of adjusting their bit rate based on network conditions and/or network policies. Awareness of the rate limit imposed by network can greatly improve the QoE for the user as application can self-adjust/regulated accordingly. IETF has developed a simple (protocol based) solution to allow the supported network node (e,g, UPF) to insert rate limit signal, via user-plane packet to the client’s application in the UE. Ref: Standard Communication with Network Elements (SCONE) Protocol - https://datatracker.ietf.org/doc/draft-ietf-scone-protocol/. This CR introduces this IETF solution to the 5GS to improve interoperability within the ecosystems. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding informative Annex to provide an example of using IETF SCONE protocol to send video rate limit information to the client application (e.g, video streaming) from UPF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interoperability with a common based approach within the ecosystems may be fragmented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex YY (new), 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CRxxx ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change – all new texts

Annex YY (informative):
DL video bit rate optimization with SCONE

# YY.0	General

This Annex provides an example of the UPF can use the Standard Communication with Network Elements (SCONE) Protocol [xxx] to send video rate limit information to the client application in the UE (e.g, for video streaming). This allows the application client to perform video bandwidth management with the application server based on the rate limit signal received from UPF.

# YY.1	Signalling DL rate signal with SCONE

Figure YY.1-1 depicts a call flow on using SCONE [xxx] to provide rate limit information to the applications. In this example, the operator has a network policy that DL bandwidth for video application should be limited (e.g, to 2Mbps). This rate limit is signalled by the UPF.

Figure YY.1-1: UPF signalling rate limit using SCONE

1.	The application client has a user-plane connection to the server, via a PDU session establishment procedure (see clause 4.3.2).

2.	The application client indicates that it can support SCONE by sending the SCONE supported indicator to the AS [xxx] during QUIC handshake procedure.

3. 	The application server coalesces a SCONE packet with ordinary QUIC packets that are sent toward the application client.

4.	The UPF detects SCONE packet and updates the rate limit signal.

NOTE:	It is up to network policy and implementation on how to set the value of the rate limit signal in a SCONE packet.

5.	The application client, based on internal algorithm(s) and processing, may use the rate limit information from the SCONE packet for bandwidth management with the application server.

Next change

# 2	References

…

[xxx]	draft-ietf-scone-protocol-02: "Standard Communication with Network Elements (SCONE) Protocol”.

Editor's note:	The above document cannot be formally referenced until it is published as an RFC.

End of Change